# Cyanobacterial Crowding-Out Effects on Metabolite Partitioning: Modeling MIB Release Dynamics and Implications for Lake Taihu

Yufan Ai†,1,4, Yongnian Wu†,2, Ming Su†,1,4,✉, Yuying Gui1,3, Yifan Du1, Jiao Fang1, Tengxin Cao1,4, and Min Yang1,4,✉

† These authors contributed equally to this work.

1 Key Laboratory of Environmental Aquatic Chemistry, State Key Laboratory of Regional Environment and Sustainability, Research Center for Eco-Environmental Sciences, Chinese Academy of Sciences. No. 18 Shuangqing Road, Haidian, Beijing, 100085, China
2 Bureau of Hydrology Information Center of Taihu Basin Authority, Shanghai 200434, China
3 College of Environmental Science and Engineering, Ocean University of China
4 University of Chinese Academy of Sciences. No. 19A Yuquan Road, Shijingshan, Beijing, 100049, China

✉ Correspondence: Ming Su <mingsu@rcees.ac.cn>, Min Yang <yangmin@rcees.ac.cn>

# Abstract

2-Methylisoborneol (MIB), a potent cyanobacterial metabolite, impairs drinking water quality through taste-and-odor issues at trace concentrations. Despite its significant impact, the intracellular dynamics and environmental release mechanisms of MIB remain poorly characterized. We developed a mechanistic model of growth-phase dependent MIB release through controlled experiments with two producer strains. The model reveals that the extracellular MIB proportion (f=eMIB/tMIB) follows a consistent pattern: decreasing to a minimum at mid-log phase before rising and stabilizing (f: 0.4 to 0.6) during stationary phase, suggesting crowding-induced cell lysis drives release dynamics. Application to Lake Taihu successfully reconstructed two odor events (2022-2023), elucidating both the spatiotemporal development of MIB producers and identifying critical risk thresholds at ~15°C and >30°C under moderate light (0.1-0.4 mol m-2 d-1) - patterns undetectable by conventional monitoring. Our findings demonstrate that physiological transitions, rather than just biomass accumulation, control odorant release. This framework may extend to other algal metabolites (e.g., geosmin, cyanotoxins), offering broader predictive capability. By linking cellular processes to water quality risks, our approach enables proactive management of cyanobacterial contaminants, informing both early warning systems and operational guidance for oxidant-type optimization to prevent large-scale release of hazardous compounds.

# Introduction

The issue of taste and odor (T&O) in drinking water sources is a major water quality concern, with earthy-musty odors caused by 2-methylisoborneol (MIB) being one of the most prevalent problems1–3. MIB, an odorous terpenoid derivative, poses a persistent challenge in drinking water due to its extremely low odor threshold concentration (OTC) of 10 ng L-1 and its resistance to conventional water treatment processes4–10. This compound is produced by various microorganisms, including actinomycetes11, cyanobacteria12, fungi13, and myxobacteria14. However, in drinking water sources, filamentous cyanobacteria—such as Pseudanabaena, Planktothrix, Phormidium, Oscillatoria, Lyngbya, and Planktothricoides—are the primary producers15–21.

Following biosynthesis, MIB is primarily distributed inside the cells as intracellular MIB (iMIB) or cell-bound MIB22,23. It is then transported to the extracellular environment via cell membrane transporters, forming extracellular MIB (eMIB). Additionally, MIB release can occur during cell wall rupture or lysis upon cell death24,25.

The distribution of MIB between intracellular and extracellular compartments has significant implications for drinking water treatment. If most MIB is already released into the water, advanced treatment processes such as ozone-activated carbon adsorption are required for its removal. Conversely, if MIB remains predominantly intracellular, conventional coagulation and sedimentation processes can effectively remove algal cells, or oxidants that minimize cell damage can be employed to prevent MIB release. Current monitoring practices in water treatment plants typically measure total MIB (tMIB) without distinguishing between intracellular and extracellular fractions, limiting their utility in guiding treatment process selection. Existing studies on MIB distribution are limited and often assume a constant ratio, which fails to provide actionable insights for water treatment optimization.

Our observations from culture experiments suggest that the intracellular and extracellular distribution of MIB is not constant and may be influenced by its biosynthesis and release mechanisms25–28. The biosynthetic pathway of MIB in cyanobacteria involves the methylation of geranyl diphosphate (GPP) followed by cyclization of methyl-GPP, catalyzed by 2-methylisoborneol synthase (2-MIBS) within the Sg Enc protein shell29–32. However, due to the lack of observable pores in the Sg Enc shell and the challenges in directly tracking MIB migration, the mechanisms governing its release and distribution remain poorly understood.

MIB production in cyanobacteria is highly variable and influenced by environmental stressors27,33. While total MIB concentrations typically peak during the exponential growth phase and decline during the senescence phase, some studies report a negative correlation between MIB production and cell growth rates. For instance, Pseudanabaena sp. accumulates higher intracellular MIB and releases more extracellular MIB under suboptimal temperature and light conditions25,34,35. One hypothesis suggests that cyanobacteria may increase extracellular MIB release under stress to inhibit competing algal species, thereby maintaining ecological dominance35. Alternatively, extracellular MIB may simply result from cell damage or death under adverse conditions25,36.

In high-cell-density environments, crowding effects can restrict cell proliferation, migration, and metabolism, impacting growth and death dynamics37–40. Crowding-induced growth limitation arises from resource competition, spatial constraints, and cell cycle arrest41,42, while cell death may result from programmed cell death, metabolic stress, mechanical damage, or cell-to-cell interactions43–45. These effects indirectly influence the release of intracellular metabolites such as MIB.

The Logistic growth model effectively describes cyanobacterial population dynamics under crowding effects, incorporating intrinsic growth rate (r) and carrying capacity (K) to capture growth deceleration and inflection points46–48. This model is particularly useful for analyzing shifts in metabolic activity and the timing of metabolite release, making it well-suited for studying MIB dynamics49,50.

Based on these observations, we hypothesize that the ratio of eMIB to tMIB (f) is not constant but varies predictably over time in response to cell crowding. To test this, we developed a mechanical model describing the temporal variation of f using culture experiments with typical MIB-producing strains. The model was validated to explain MIB release patterns during cell growth and applied to assess growth-stage distributions of odor-producing cyanobacteria in Taihu Lake, predicting odor risk during MIB outbreaks. This study provides new insights into MIB release dynamics, enhances understanding of cyanobacterial odor production, and offers practical guidance for water treatment process selection and bloom management.

# Materials and Methods

## Study Area and Sampling

Lake Taihu (30°55’40”–31°32’58” N, 119°52’32”–120°36’10” E), located in the southeastern Yangtze River Delta, is China’s third-largest shallow freshwater lake, with a surface area of 2,578 km2. The lake’s watershed supports a densely populated region of over 40 million people and serves as a critical drinking water source for approximately 10 million residents. It is also extensively used for irrigation and aquaculture, making water quality a vital concern for local ecosystems and livelihoods.

Lake Taihu is renowned for its recurrent Microcystis blooms, which have drawn significant scientific and policy attention. In response, China has implemented substantial efforts to reduce non-point source pollution, particularly nutrient loading, leading to measurable improvements in water quality in recent years. However, as nutrient concentrations have declined, shifts in algal community composition have occurred: Microcystis blooms have diminished, while filamentous cyanobacteria have proliferated, resulting in elevated 2-methylisoborneol (MIB) levels.

From March 2022 to February 2023, monthly water samples were collected from 31 sampling sites distributed across the lake (Fig. S1). Given the lake’s average depth of ~2 m and well-mixed conditions, surface water samples (2 L) were collected from 0.5 m below the water surface using a Kemmerer water sampler.

## Physicochemical Analysis and Phytoplankton Enumeration

In situ measurements of physicochemical parameters—including water temperature, pH, dissolved oxygen (DO), turbidity, and conductivity—were conducted using a multiparameter probe (EXO2, USA). Water transparency was quantified via Secchi depth (SD) measurements using a 20 cm diameter Secchi disk. Nutrient concentrations (nitrate, nitrite, ammonia, total nitrogen [TN], and total phosphorus [TP]) were analyzed following Chinese national standard methods TODO: [Ref].

For odorant analysis, subsamples (100 mL) were collected in amber glass bottles with airtight seals and preserved with 10 mg L⁻¹ HgCl2 to inhibit biodegradation51. Total MIB production potential was determined by measuring its concentration within 72 hours using solid-phase microextraction (SPME) coupled with gas chromatography-mass spectrometry (GC-MS; Agilent 6890/5975, USA), following established protocols16,52.

For phytoplankton enumeration, subsamples (1000 mL) were fixed with 5% Lugol’s iodine solution53, settled for 48 hours, and pre-concentrated 20-fold before analysis. Cyanobacterial species were identified based on morphological criteria54, with taxonomic revisions following55. Cell counts were performed using an upright microscope (Olympus BX53, Japan) following the Utermöhl method56.

## Model Development

The logistic growth model was applied to describe changes in cell density of cultured strains (Eq. 1):

Nt=K1+K−N0N0e−rt  1

θ=K−N0N0  2

Here, K represents the environmental carrying capacity (maximum achievable cell density), N0 is the initial cell density (t = 0), r is the intrinsic growth rate (maximum growth rate under ideal conditions)48,50, and θ denotes the initial growth potential.

In aquatic systems, MIB exists in two forms: intracellular (iMIB) and extracellular (eMIB), with their sum constituting total MIB (tMIB). The ratio f (eMIB/tMIB) serves as an odor index, reflecting the proportion of dissolved MIB over total MIB.

To model temporal variations in eMIB and tMIB produced by Pseudanabaena cinerea FACHB 1277 and Check: Planktothricoides ???? (obtained from the Freshwater Algae Culture Collection at the Institute of Hydrobiology), we developed stage-specific equations for f. For cell densities below 80% of K, f was derived from the logistic growth equation to capture early- to mid-growth dynamics (Eq. 3). For densities approaching K (>80%), f was modeled using residual growth capacity to describe the decline phase (Eq. 4):

f=1−α1TNtK′=1−α1Tθrerte2rt+2θert+θ2  3

f=12+K−NtKα2+β  4

Here, α1 and α2 are variation coefficients, T is the cell doubling time, and β is a correction factor for MIB accumulation/volatilization. Model parameters (N0, K, r) were estimated via logistic curve fitting, while α1, α2, and β were determined via linear regression. Adjusting r allowed simulation of growth and MIB production under varying environmental conditions.

## MIB Risk Assessment

For a given f, the cyanobacterial growth time (t) was derived from Eq. 3 and Eq. 4, and the growth stage indicator (St) was calculated as:

St=t2T0  5

where T0 (time at which Nt = K/2) marks the logistic curve’s inflection point. By combining St with field-measured tMIB concentrations, we generated a frequency distribution histogram (Fig. S9) to quantify the probability of tMIB occurrence at different growth stages, enabling MIB risk evaluation.

## Statistical Analysis and Visualization

All analyses were conducted in R 4.4.257. Data preprocessing and summarization used dplyr58, while regression analyses (linear/generalized linear models) employed the stats package. Generalized additive modeling (mgcv;59) and quantile regression (quantreg;60) were applied where appropriate. Figures were generated using ggplot261, with contour plots produced via the graphics package.

# Results

## Spatiotemporal Distribution Characteristics of MIB in Lake Taihu

Between March 2022 and February 2023, two severe MIB outbreaks were observed in Lake Taihu (Fig. 1A). The outbreaks exhibited distinct seasonal patterns, with elevated MIB concentrations occurring primarily in spring (40.0 ± 77.5 ng L-1) and summer (107 ± 230 ng L-1). The highest peaks were recorded in April (66.5 ± 101 ng L-1) and July (232 ± 361 ng L-1), demonstrating statistically significant seasonality (p < 0.001). Notably, MIB concentrations across the lake were relatively low in June, between the two outbreak periods.

Spatially, MIB distribution showed clear heterogeneity across the lake. The northern and southeastern regions consistently showed the highest concentrations, with the April peak centered in the northern area and the July peak affecting both northern and southeastern areas. Cluster analysis of MIB concentrations from all 31 sampling sites revealed four distinct spatial clusters that corresponded to different lake regions (Fig. S1), with the most severe outbreaks concentrated in the northern and southeastern clusters.

Analysis of the f value variation across clusters revealed an important pattern: the lowest values during the decline phase of f were typically followed by MIB outbreaks (Fig. 1C). Concurrent 16S RNA gene sequencing identified Pseudanabaena and Planktothricoides as the dominant MIB-producing genera, though their abundances varied significantly. While Microcystis dominated the cyanobacterial community for most of the study period (Fig. 1D), Pseudanabaena reached its peak relative abundance (31.0%) in September, coinciding with MIB production. In contrast, Planktothricoides remained at much lower levels (1.4%).

Fig. 1: Annual patterns of MIB and odor-producing cyanobacteria in Lake Taihu: (A) Spatiotemporal distribution of total MIB (tMIB) concentrations; (B) Spatial clustering of tMIB concentrations across the lake; (C) Temporal variation in extracellular MIB proportion (eMIB)/tMIB) by lake region; (D) Seasonal shifts in dominant cyanobacterial genera composition.

## Modeling the Relationship Between Cultured Cyanobacteria Growth and Extracellular MIB Ratio (f)

The observed f ratio in Lake Taihu water samples exhibited a characteristic temporal pattern (Fig. 1C). To investigate this phenomenon, we conducted controlled experiments with pure cultures of two odor-producing cyanobacteria (Pseudanabaena and Planktothricoides) under varying light conditions (Fig. 2C).

During growth and odor production, both species showed a consistent f ratio pattern: an initial decrease reaching its minimum at approximately half of the maximum cell density, followed by a gradual increase that stabilized near 0.5. Notably, while eMIB and tMIB concentrations increased slowly during the initial phase, they rose rapidly during the subsequent phase of f ratio increase (Fig. 2A, 2B).

We developed a logistic growth-based model to characterize these f ratio dynamics (Eq. 3, Eq. 4). The model effectively captured the growth patterns of both Pseudanabaena and Planktothricoides (Fig. 2C, 2D). Parameter estimation revealed significant interspecies differences: α1 was substantially higher for Pseudanabaena than for Planktothricoides (p < 0.01), while β showed the opposite trend (p < 0.01) (Fig. 2E). The model demonstrated good overall performance, with relatively small residuals across most growth conditions (Fig. 2F).

TODO: 增加Fig. 3的描述

Fig. 2: Model development and performance for extracellular MIB ratio (f): (A) Logistic growth curve illustrating key growth phases; (B) Conceptual framework of the f ratio model; (C-D) Logistic growth fitting for Pseudanabaena (C) and Planktothricoides (D), with raw data from refs18 and33 respectively; (E) Inter-species comparison of model parameters (α1, α1, β; (F) Model residual analysis for both species. Note: Logistic fitting was performed in this study.

Fig. 3: Modelled ratio (line) and observed ratio (circle) of extracellular MIB to total MIB (f) for cultured Pseudanabaena and Planktothricoides

## Spatiotemporal Variation of Odor-Producing Cyanobacteria Growth Stages in Lake Taihu

The cultivation-based model was successfully applied to field data from sampling locations throughout Lake Taihu to characterize growth parameters and determine St values for odor-producing cyanobacteria. Analysis revealed two distinct seasonal growth patterns across the lake. A primary growth phase developed from May through November as water temperatures increased, followed by a secondary, smaller-scale growth phase (St) occurring between December and April after winter production (Fig. 4A).

These alternating growth phases directly corresponded with the two annual peaks in MIB concentrations observed in the lake. Examination of algal growth stages across different cluster regions demonstrated strong synchronization between cyanobacterial growth dynamics and MIB outbreak events (Fig. 4B). The St values proved particularly informative, as values approaching 0.5 reliably indicated when populations reached the inflection point of logarithmic growth - a critical threshold associated with elevated risk of substantial MIB release.

Fig. 4: Growth stage distribution of odor-producing cyanobacteria in the Lake Taihu (A) and different subregions (B)

To enable broader application of the model across Lake Taihu, we compiled data on water temperature, solar radiation, and odor-producing cyanobacteria growth patterns under various conditions from multiple sources. By analyzing these monthly variations, we established relationships between environmental factors (temperature and light intensity) and the intrinsic growth rate (r) of cyanobacteria. Our analysis revealed clear seasonal differences in the distribution of cyanobacterial growth stages (Fig. 5A). From spring (March 2022) through winter (February 2023), the growth stages followed a consistent pattern of initial increase followed by gradual decline across the entire lake. Spatially, the northern and southeastern regions consistently showed more advanced growth stages compared to other areas throughout all seasons.

The seasonal progression of growth stages exhibited distinct spatial patterns. In early spring, odor-producing cyanobacteria first began growing in the northwest and southeast regions. By summer, active growth had spread to most areas of the lake. During autumn, the northern, southern and southeastern regions reached later growth stages, while in winter, cyanobacterial growth became minimal across nearly the entire lake, with only a small portion of the northern region showing any activity. This spatiotemporal progression of growth stages aligned closely with the observed patterns of MIB production throughout the study period.

Fig. 5: Growth stage distribution of odor-producing cyanobacteria across the entire Lake Taihu; Odor risk assessment across the entire Lake Taihu (probability of textMIB > 10 ng L​−1).

The relationship between odor-producing cyanobacteria growth stage (St) and total MIB concentration (tMIB) was further analyzed to assess annual odor risk distribution in Lake Taihu (Fig. 5B, Fig. S8). The analysis revealed similar spatial patterns for the probability of MIB exceeding both 10 ng L-1 and 50 ng L-1 thresholds, with the higher threshold’s probability typically being half that of the lower threshold.

Seasonal risk patterns showed elevated MIB concentrations occurring during two main periods: spring (March-May) and late summer through autumn (July-October). June represented a transitional period between these two risk episodes, while from November to February, most of the lake showed minimal risk except for localized areas in the northern and southeastern regions.

Spatially, spring risks were primarily concentrated in the northern lake region. During late summer and autumn, high-risk areas expanded to encompass nearly the entire lake. Throughout all seasons, the northern and southeastern regions consistently emerged as high-probability hotspots for elevated MIB concentrations.

## Environmental Influences on Cyanobacteria Growth and Odor Risk

Research has established that Pseudanabaena exhibits distinct intrinsic growth rates (r) under varying light and temperature conditions. These growth rate variations significantly influence the temporal dynamics of the growth stage indicator (St) by modifying the timing of the logistic growth inflection point (T0). Across the environmental ranges typical for Lake Taihu (15-35°C and 0.1-0.4 mol m-2 d-1), T0 demonstrates consistent negative correlations with three key factors: initial cell density, light availability, and temperature (Fig. 6A).

The ecological implications of these relationships are noteworthy. When environmental carrying capacity remains stable but initial cell density increases, the growth stage duration of odor-producing cyanobacteria shortens substantially, resulting in decreased T0 values. This accelerated growth timeline leads to earlier onset of crowding effects, subsequent cell lysis, and increased release of odor compounds - all contributing to elevated odor risk. Parallel effects occur under favorable light and temperature conditions, where higher light intensity and warmer temperatures similarly reduce T0 through enhanced cellular growth rates.

Application of these relationships within Lake Taihu’s odor risk prediction framework reveals two distinct risk peaks under moderate light conditions. The first occurs around 15°C, with a second peak emerging above 30°C (Fig. 6B). These critical temperature thresholds align precisely with the observed bimodal pattern of MIB-related odor events in the lake, explaining the characteristic spring and summer outbreak periods. The model output confirms that temperature-mediated alterations in cyanobacterial growth dynamics represent the primary driver of seasonal MIB risk variations throughout Lake Taihu.

Fig. 6: Environmental influences on cyanobacterial growth dynamics and odor risk in Lake Taihu: (A) Variation in logistic growth inflection points under different light and temperature conditions; (B) Resulting probability of total MIB concentrations exceeding 10 ng L-1 (tMIB > 10 ng L-1).

# Discussion

## Modeling MIB Partitioning Dynamics in Relation to Cyanobacterial Growth Phases

The production and distribution of 2-methylisoborneol (MIB) by odor-producing cyanobacterial cells are influenced by both their growth stage and environmental stressors. Previous studies have rarely focused on the intracellular and extracellular distribution of MIB during the growth of cyanobacteria in both natural aquatic environments and laboratory cultures. Some even suggested that the variation in MIB partitioning lacks a discernible pattern62,63. In this study, we introduce the ratio f (dMIB/tMIB) as an indicator to represent the distribution pattern of MIB. Our findings show that f is not a constant, but follows a specific dynamic trend, which may be attributed to cell crowding effects. After synthesis, MIB is primarily bound to cytoplasmic and soluble protein fractions within cyanobacterial cells64. The major mechanism of MIB release is cell lysis, occurring either through programmed cell death or mechanical rupture of the cell wall25,27,28,36. In this study, we established a model describing the relationship between f and growth time t, and further proposed a metric—growth stage (St)—to evaluate the distribution of cyanobacterial growth stages across the lake (Fig. 7). The St index ranges from 0 to 1, representing the progression of cell growth from the lag phase to the stationary phase, with St = 0.5 corresponding to the inflection point of the logistic growth curve. A higher St indicates a larger proportion of cells in the later stages of growth, implying a greater potential for MIB release. As cell growth progresses toward the stationary phase, environmental cell density increases, leading to intensified crowding effects. These conditions exacerbate cell damage and apoptosis, causing a larger proportion of MIB to be released into the surrounding water65.

Fig. 7: Schematic representation of MIB allocation dynamics influenced by cell crowding, both intracellularly and extracellularly

In this study, we developed a model linking f to cell growth time t, thereby associating the cyanobacterial growth stage with odor production patterns. This model enables the estimation of the growth stage distribution of odor-producing cyanobacteria, offering insights into current odor issues and supporting prediction of future taste and odor (T&O) risks. From a management perspective, f serves dual functions: (1) as a T&O index reflecting the proportion of dissolved MIB (eMIB) to total MIB (tMIB) in the water, and (2) as an ecological indicator of the relative proportions of cells at different growth stages. Model results (Fig. 2A, B) demonstrate that f follows a nonlinear pattern—initially decreasing, then increasing and eventually stabilizing—as the cell population transitions from the lag phase to the stationary phase. During the exponential phase, f reaches its minimum when cell growth rate peaks at the inflection point of the logistic growth curve. After this point, crowding-induced stress increases, leading to significant MIB release due to cell damage and apoptosis. Toward the end of the exponential phase and during the stationary phase, cell density approaches the environmental carrying capacity and enters a state of dynamic equilibrium. At this stage, f is primarily influenced by total cell density and the accumulation and degradation dynamics of eMIB in the environment. This trend is consistent with observations reported in previous studies25,27. By applying the f-model, the growth stage distribution of odor-producing cyanobacteria can be inferred based on measured MIB partitioning. This framework enhances our understanding of T&O events in freshwater lakes and reservoirs and provides valuable guidance for implementing more targeted management strategies.

## Field-Based Analysis of MIB Dynamics in Relation to Cyanobacterial Growth in Lake Taihu

Based on the intracellular and extracellular MIB distribution patterns caused by the cell crowding effect, the growth and odor-producing processes of MIB-producing cyanobacteria in Lake Taihu can be further analyzed. Field monitoring of MIB in Lake Taihu revealed two major MIB outbreaks: one in April (in the northern region) and another in July, with high concentrations predominantly observed in summer and early autumn. Spatial clustering analysis showed that the northern and southeastern regions were identified as MIB outbreak hotspots. Simultaneously, the lake-wide distribution of St indicated that during the transition from winter to spring, cyanobacteria in certain lake areas began to grow slowly. This early-phase growth persisted until early summer, after which a second, more rapid growth period occurred, continuing until late autumn. Similar dual-peak MIB patterns have been reported in other studies, although with slightly different timing. For instance, in 2019 and 2021, smaller MIB peaks were observed in May, while the major peaks occurred in August in Lake Taihu. Notably, the May MIB outbreaks were predominantly located in the southeastern region66,67. Temperature and light availability are the primary environmental drivers of cyanobacterial growth. During the summer, both factors typically reach optimal levels for the growth of Pseudanabaena, a key MIB-producing filamentous cyanobacterium67–70. From April to May, when the surface water temperature exceeds 20°C, MIB-producing cyanobacteria are already able to grow at a considerable rate71–73. Meanwhile, the lake lacks other dominant cyanobacterial species to compete with them, resulting in a minor peak in MIB concentration during spring. Between May and July, Microcystis undergoes rapid proliferation and becomes the dominant cyanobacterial genus. This dominance reduces light penetration to the subsurface layers, where filamentous MIB-producing cyanobacteria typically reside, thereby inhibiting their growth70,74. Previous studies also confirm that Microcystis blooms begin around May and subsequently dominate the phytoplankton community75. Research on microcystin production in Lake Taihu supports this timeline, with significant increases in microcystin concentrations observed only after May66.

Further identification of high-risk regions for T&O events indicates that the northern and southeastern areas are the most vulnerable. In the southeastern risk zone, which includes four sampling sites, MIB levels slightly increased in April, declined briefly, and then peaked again in July, remaining elevated through September. In contrast, the northern region exhibited a similar trend, but with a higher April–May peak, and sustained MIB levels >10 ng L-1 between the two major peaks (Fig. 1B). The different odor production dynamics in these two risk zones are attributable to variations in cyanobacterial growth stages. In the southeast, cyanobacteria were in the late growth stage in March, but transitioned to an early growth stage by April, suggesting a mass cell death event between March and April. As a result, cell density did not increase significantly, and MIB production potential was limited. In contrast, cyanobacteria in the northern region remained in the exponential phase from March to April, with continuous growth and higher MIB production potential. At the lake-wide scale, the spatial progression of odor risk zones from north to south from spring through winter was observed—a pattern not previously reported in studies of MIB-related odor problems in Lake Taihu.

Different odor-producing cyanobacteria exhibit distinct growth and odor-production characteristics. Modeling efforts tailored to specific taxa revealed significant differences in fitted parameters (Fig. 2E), likely due to variations in intrinsic growth rates and MIB production efficiencies among species. Studies have shown that Pseudanabaena has a maximum cellular MIB yield of 0.15 ± 0.04 pg cell⁻¹ under a light intensity of 85 μmol photons m⁻² s⁻¹18, while Planktothricoides exhibits a higher yield of 0.22 ± 0.02 pg cell⁻¹ under similar conditions33. In addition,the inflection point time (T0) for these two species also vary significantly across studies (Fig. S10), further contributing to their differential contributions to odor events in aquatic systems.

## Environmental significance

By developing a model that links the growth stages of odor-producing cyanobacteria with MIB release and its intracellular–extracellular distribution, this study offers important insights into the ecological dynamics of odor events in freshwater systems. First, it enhances our understanding of the growth behavior of odor-producing cyanobacteria in natural waters. Second, the model provides a tool for forecasting the temporal trends of MIB concentration, thereby improving early-warning capabilities. Third, by quantifying the ratio of intracellular to extracellular MIB, the results can inform water treatment strategies—specifically, whether to target intact cells through coagulation and sedimentation or to apply cell-disruption methods when extracellular MIB dominates. This integrated approach supports more effective and condition-specific odor mitigation in drinking water treatment9,76–78.

This study investigates the distribution pattern of MIB-producing cyanobacteria and constructs a model for f based on cell crowding effects and the Logistic growth curve to explain the dynamics of eMIB and pMIB during different growth phases. Furthermore, applying the model to Lake Taihu enables the estimation of annual cyanobacterial growth phase distributions based on MIB distribution patterns and facilitates the prediction of high-risk odor zones, providing guidance for odor management in lakes and reservoirs. However, this study primarily considers MIB release through cell damage and apoptosis, which accounts for the majority of MIB release, while the potential active release of MIB remains unaddressed and requires further investigation.

# Conclusion

The intracellular and extracellular distribution dynamics of 2-methylisoborneol (MIB), a musty-smelling secondary metabolite produced by cyanobacteria, remain poorly understood, limiting our ability to predict odor events in source waters. This study developed a growth-stage-based model to characterize MIB release patterns and applied it to Lake Taihu, where conventional approaches have struggled to explain complex odor phenomena. Results revealed that odor-producing cyanobacteria in the lake underwent two distinct growth cycles annually, corresponding to dual MIB concentration peaks—in spring and summer. The f model, derived from laboratory-cultured cyanobacteria data, accurately captured these growth trends and was successfully applied to field observations across the lake. By integrating environmental variables such as water temperature and light intensity, the model enabled lake-wide prediction of cyanobacterial growth stages and corresponding MIB risk. Spatially, the northern and southeastern regions of Lake Taihu consistently showed higher growth potential and odor risk. Temporally, high MIB risk was concentrated in early spring and mid-summer, aligning with inflection points in the cyanobacterial growth phase. Further analysis indicated that environmental factors—especially temperature and light dose—significantly influenced the timing of growth phase transitions, thereby affecting odor risk. Notably, the model identified two risk peaks under moderate light conditions, reflecting the dual seasonal odor patterns observed in the lake. These findings demonstrate the applicability of the f model as a predictive tool for early MIB warnings and provide valuable insights for water quality monitoring and lake management.

# FIGURES

- Fig. 1: Caption of Fig. 1

- Fig. 2: Caption of Fig. 2

# ASSOCIATED CONTENT

## Supporting Information

The Supporting Information is available free of charge at https://.

Description of Figure S1 (Fig. S1); Description of Table S1 (Table S1); (PDF)

# AUTHOR INFORMATION

## Corresponding Authors

- Ming Su: 1. Key Laboratory of Environmental Aquatic Chemistry, State Key Laboratory of Regional Environment and Sustainability, Research Center for Eco-Environmental Sciences, Chinese Academy of Sciences, Beijing 100085, China; 2. University of Chinese Academy of Sciences, Beijing 100049, China; ORCID: https://orcid.org/0000-0001-9821-1268; Email: mingsu@rcees.ac.cn

- Min Yang: National Engineering Research Center of Industrial Wastewater Detoxication and Resource Recovery, Research Center for Eco-Environmental Sciences, Chinese Academy of Sciences, Beijing 100085, China; ORCID: https://orcid.org/0009-0005-4902-7620; Email: yangmin@rcees.ac.cn

## Authors

- First Last: 1. Key Laboratory of Environmental Aquatic Chemistry, State Key Laboratory of Regional Environment and Sustainability, Research Center for Eco-Environmental Sciences, Chinese Academy of Sciences; 2. School of Ecology and Environment, Inner Mongolia University, Hohhot 010021, China; ORCID: https://orcid.org/0009-0005-4902-7620

## Author contributions

## Notes

The authors declare no competing financial interest.

# References

(1) 	WHO, W. H. O. Guidelines for Drinking-Water Quality; World Health Organization: Geneva, Switzerland, 2011.

(2) 	Devi, A.; Chiu, Y.-T.; Hsueh, H.-T.; Lin, T.-F. Quantitative PCR Based Detection System for Cyanobacterial Geosmin/2-Methylisoborneol (2-MIB) Events in Drinking Water Sources: Current Status and Challenges. Water Research 2021, 188, 116478. https://doi.org/10.1016/j.watres.2020.116478.

(3) 	Suurnäkki, S.; Gomez-saez, G. V.; Rantala-ylinen, A.; Jokela, J.; Fewer, D. P.; Sivonen, K. Identification of Geosmin and 2-Methylisoborneol in Cyanobacteria and Molecular Detection Methods for the Producers of These Compounds. Water Research 2015, 68 (0), 56–66. https://doi.org/10.1016/j.watres.2014.09.037.

(4) 	Izaguirre, G.; Taylor, W. D. A Guide to Geosmin- and MIB-Producing Cyanobacteria in the United States. Water Science and Technology 2004, 49 (9), 19–24. https://doi.org/10.2166/wst.2004.0524.

(5) 	Wang, C.; An, W.; Zhao, Y.; Dietrich, A. M.; Gallagher, D. L.; Zhang, Y.; Qian, Y.; Yu, J.; Yang, M. Risk Assessment for 2-MIB Odor Complaint in Drinking Water Based on Zero-Inflated Log-Normal Model. ACS ES&T Water 2023, 4 (1), 146–154. https://doi.org/10.1021/acsestwater.3c00477.

(6) 	Suffet, I. h. (mel); Khiari, D.; Bruchet, A. The Drinking Water Taste and Odor Wheel for the Millennium: Beyond Geosmin and 2-Methylisoborneol. Water Science and Technology 1999, 40 (6), 1–13. https://doi.org/10.1016/s0273-1223(99)00531-4.

(7) 	Gillogly, T. E. T.; Snoeyink, V. L.; Newcombe, G.; Elarde, J. R. A Simplified Method to Determine the Powdered Activated Carbon Dose Required to Remove Methylisoborneol. Water Science and Technology 1999, 40 (6), 59–64. https://doi.org/10.1016/S0273-1223(99)00538-7.

(8) 	Wang, Q.; Zietzschmann, F.; Yu, J.; Hofman, R.; An, W.; Yang, M.; Rietveld, L. C. Projecting Competition Between 2-Methylisoborneol and Natural Organic Matter in Adsorption onto Activated Carbon from Ozonated Source Waters. Water Research 2020, 173, 115574. https://doi.org/10.1016/j.watres.2020.115574.

(9) 	Li, L.; Yang, S.; Yu, S.; Zhang, Y. Variation and Removal of 2-MIB in Full-Scale Treatment Plants with Source Water from Lake Tai, China. Water Research 2019, 162, 180–189. https://doi.org/10.1016/j.watres.2019.06.066.

(10) 	AWWA, A. W. W. A. Algae: Source to Treatment; American Water Works Association, 2010.

(11) 	Gerber, N. N. A Volatile Metabolite of Actinomycetes, 2-Methylisoborneol. Antibiotics 1969, 22 (10), 508–509.

(12) 	Tabachek, J. L.; Yurkowski, M. Isolation and Identification of Blue-Green Algae Producing Muddy Odor Metabolites, Geosmin, and 2-Methylisoborneol, in Saline Lakes in Manitoba. Journal of the Fisheries Research Board of Canada 1976, 33 (1), 25–35. https://doi.org/10.1139/f76-004.

(13) 	Yamada, Y.; Kuzuyama, T.; Komatsu, M.; Shin-ya, K.; Omura, S.; Cane, D. E.; Ikeda, H. Terpene Synthases Are Widely Distributed in Bacteria. Proceedings of the National Academy of Sciences 2014, 112 (3), 857–862. https://doi.org/10.1073/pnas.1422108112.

(14) 	Dickschat, Jeroen s.; Nawrath, T.; Thiel, V.; Kunze, B.; Müller, R.; Schulz, S. Biosynthesis of the Off-Flavor 2-Methylisoborneol by the Myxobacterium Nannocystis Exedens. Angewandte Chemie International Edition 2007, 46 (43), 8287–8290. https://doi.org/10.1002/anie.200702496.

(15) 	Su, M.; Suruzzaman, MD.; Zhu, Y.; Lu, J.; Yu, J.; Zhang, Y.; Yang, M. Ecological Niche and in-Situ Control of MIB Producers in Source Water. Journal of Environmental Sciences 2021, 110, 119–128. https://doi.org/10.1016/j.jes.2021.03.026.

(16) 	Su, M.; Yu, J.; Zhang, J.; Chen, H.; An, W.; Vogt, R. D.; Andersen, T.; Jia, D.; Wang, J.; Yang, M. MIB-Producing Cyanobacteria (Planktothrix Sp.) in a Drinking Water Reservoir: Distribution and Odor Producing Potential. Water Research 2015, 68, 444–453. https://doi.org/10.1016/j.watres.2014.09.038.

(17) 	Lu, J.; Su, M.; Su, Y.; Fang, J.; Burch, M.; Cao, T.; Wu, B.; Yu, J.; Yang, M. MIB-Derived Odor Management Based Upon Hydraulic Regulation in Small Drinking Water Reservoirs: Principle and Application. Water Research 2023, 244, 120485. https://doi.org/10.1016/j.watres.2023.120485.

(18) 	Cao, T.; Fang, J.; Jia, Z.; Zhu, Y.; Su, M.; Zhang, Q.; Song, Y.; Yu, J.; Yang, M. Early Warning of MIB Episode Based on Gene Abundance and Expression in Drinking Water Reservoirs. Water Research 2023, 231, 119667. https://doi.org/https://doi.org/10.1016/j.watres.2023.119667.

(19) 	Izaguirre, G.; Jungblut, A.; Neilan, B. A. Benthic Cyanobacteria (Oscillatoriaceae) That Produce Microcystin-LR, Isolated from Four Reservoirs in Southern California. Water Research 2007, 41 (2), 492–498. https://doi.org/10.1016/j.watres.2006.10.012.

(20) 	Van Der Ploeg, M.; Dennis, M.; De Regt, M. Biology of Oscillatoria Chalybea, a 2-Methylisoborneol Producing Blue-Green Alga of Mississippi Catfish Ponds. Water Science and Technology 1995, 31 (11), 173–180. https://doi.org/10.1016/0273-1223(95)00473-Z.

(21) 	Zhang, T.; Li, L.; Song, L.; Chen, W. Effects of Temperature and Light on the Growth and Geosmin Production of Lyngbya Kuetzingii (Cyanophyta). Journal of Applied Phycology 2009, 21, 279–285. https://doi.org/10.1007/s10811-008-9363-z.

(22) 	Bentley, R.; Meganathan, R. Geosmin and Methylisoborneol Biosynthesis in Streptomycetes. Evidence for an Isoprenoid Pathway and Its Absence in Non-Differentiating Isolates. FEBS Letters 1981, 125 (2), 220.

(23) 	Wu, J.; Jüttner, F. Differential Partitioning of Geosmin and 2-Methylisoborneol Between Cellular Constituents in Oscillatoria Tenuis. Archives of Microbiology 1988, 150, 580–583. https://doi.org/10.1007/bf00408253.

(24) 	Rashash, D. m. C.; Dietrich, A. m.; Hoehn, R. c.; Parker, B. c. The Influence of Growth Conditions on Odor-Compound Production by Two Chrysophytes and Two Cyanobacteria. Water Science and Technology 1995, 31 (11), 165–172. https://doi.org/10.1016/0273-1223(95)00472-y.

(25) 	Zhang, T.; Zheng, L.; Li, L.; Song, L. 2-Methylisoborneol Production Characteristics of Pseudanabaena sp. FACHB 1277 Isolated from Xionghe Reservoir, China. Journal of Applied Phycology 2016, 1–10. https://doi.org/10.1007/s10811-016-0864-x.

(26) 	Oh, H.-S.; Lee, C. S.; Srivastava, A.; Oh, H.-M.; Ahn, C.-Y. Effects of Environmental Factors on Cyanobacterial Production of Odorous Compounds: Geosmin and 2-Methylisoborneol. Journal of Microbiology and Biotechnology 2017, 27 (7), 1316–1323. https://doi.org/10.4014/jmb.1702.02069.

(27) 	Wang, H.; Li, L.; Cheng, S.; Chen, L.; Zhang, H.; Zhang, X. Production and Release of 2-MIB in Pseudanabaena: Effects of Growth Phases on Cell Characteristics and 2-MIB Yield. Ecotoxicology and Environmental Safety 2024, 274, 116198. https://doi.org/10.1016/j.ecoenv.2024.116198.

(28) 	Alghanmi, H. A.; Alkam, F. M.; AL-Taee, M. M. Effect of Light and Temperature on New Cyanobacteria Producers for Geosmin and 2-Methylisoborneol. Journal of Applied Phycology 2018, 30 (1), 319–328. https://doi.org/10.1007/s10811-017-1233-0.

(29) 	Giglio, S.; Chou, W. K. W.; Ikeda, H.; Cane, D. E.; Monis, P. T. Biosynthesis of 2-Methylisoborneol in Cyanobacteria. Environmental Science & Technology 2011, 45 (3), 992–998. https://doi.org/10.1021/es102992p.

(30) 	Wang, Z.; Xu, Y.; Shao, J.; Wang, J.; Li, R. Genes Associated with 2-Methylisoborneol Biosynthesis in Cyanobacteria: Isolation, Characterization, and Expression in Response to Light. Plos One 2011, 6 (4), E18665. https://doi.org/10.1371/journal.pone.0018665.

(31) 	Komatsu, M.; Tsuda, M.; omura, S.; Oikawa, H.; Ikeda, H. Identification and Functional Analysis of Genes Controlling Biosynthesis of 2-Methylisoborneol. Proceedings of the National Academy of Sciences 2008, 105 (21), 7422–7427. https://doi.org/10.1073/pnas.0802312105.

(32) 	Andreas, M. P.; Giessen, T. W. The Biosynthesis of the Odorant 2-Methylisoborneol Is Compartmentalized Inside a Protein Shell. Nature Communications 2024, 15 (1). https://doi.org/10.1038/s41467-024-54175-4.

(33) 	Lu, J.; Su, M.; Su, Y.; Wu, B.; Cao, T.; Fang, J.; Yu, J.; Zhang, H.; Yang, M. Driving Forces for the Growth of MIB-Producing Planktothricoides Raciborskii in a Low-Latitude Reservoir. Water Research 2022, 118670. https://doi.org/10.1016/j.watres.2022.118670.

(34) 	Jia, Z.; Su, M.; Liu, T.; Guo, Q.; Wang, Q.; Burch, M.; Yu, J.; Yang, M. Light as a Possible Regulator of MIB-Producing Planktothrix in Source Water Reservoir, Mechanism and in-Situ Verification. Harmful Algae 2019, 88, 101658. https://doi.org/10.1016/j.hal.2019.101658.

(35) 	Wang, Z.; Li, R. Effects of Light and Temperature on the Odor Production of 2-Methylisoborneol-Producing Pseudanabaena Sp. and Geosmin-Producing Anabaena Ucrainica (Cyanobacteria). Biochemical Systematics and Ecology 2015, 58, 219–226. https://doi.org/10.1016/j.bse.2014.12.013.

(36) 	Zimba, P. V.; Dionigi, C. P.; Millie, D. F. Evaluating the Relationship Between Photopigment Synthesis and 2-Methylisoborneol Accumulation in Cyanobacteria. Journal of Phycology 1999, 35 (6), 1422–1429. https://doi.org/10.1046/j.1529-8817.1999.3561422.x.

(37) 	Eisenhoffer, G. T.; Rosenblatt, J. Bringing Balance by Force: Live Cell Extrusion Controls Epithelial Cell Numbers. Trends in Cell Biology 2013, 23 (4), 185–192. https://doi.org/10.1016/j.tcb.2012.11.006.

(38) 	Bermudez, A. G.; Latham, Z.; Hu, J.; Lin, N. Cell Crowding-Induced Geometric Constraint Regulates Chromatin Organizations. Biophysical Journal 2024, 123 (3), 407a. https://doi.org/10.1016/j.bpj.2023.11.2493.

(39) 	Yang, S.; Golkaram, M.; Oh, S.; Oh, Y.; Cho, Y.; Yoe, J.; Ju, S.; Lalli, M. A.; Park, S.-Y.; Lee, Y.; Jang, J. ETV4 Is a Mechanical Transducer Linking Cell Crowding Dynamics to Lineage Specification. Nature Cell Biology 2024, 26 (6), 903–916. https://doi.org/10.1038/s41556-024-01415-w.

(40) 	Lawson-Keister, E.; Manning, M. L. Jamming and Arrest of Cell Motion in Biological Tissues. Current Opinion in Cell Biology 2021, 72, 146–155. https://doi.org/10.1016/j.ceb.2021.07.011.

(41) 	Drewinko, B.; Yang, L. Y.; Barlogie, B.; Trujillo, J. M. Cultured Human Tumour Cells May Be Arrested in All Stages of the Cycle During Stationary Phase: Demonstration of Quiescent Cells in G1, s and G2 Phase. Cell Proliferation 1984, 17 (5), 453–463. https://doi.org/10.1111/j.1365-2184.1984.tb00604.x.

(42) 	Eisenhoffer, G. T.; Loftus, P. D.; Yoshigi, M.; Otsuna, H.; Chien, C.-B.; Morcos, P. A.; Rosenblatt, J. Crowding Induces Live Cell Extrusion to Maintain Homeostatic Cell Numbers in Epithelia. Nature 2012, 484 (7395), 546–549. https://doi.org/10.1038/nature10999.

(43) 	Bar, J.; Cohen-Noyman, E.; Geiger, B.; Oren, M. Attenuation of the P53 Response to DNA Damage by High Cell Density. Oncogene 2004, 23 (12), 2128–2137. https://doi.org/10.1038/sj.onc.1207325.

(44) 	Nishio, M.; Maehama, T.; Goto, H.; Nakatani, K.; Kato, W.; Omori, H.; Miyachi, Y.; Togashi, H.; Shimono, Y.; Suzuki, A. Hippo Vs. Crab: Tissue‐specific Functions of the Mammalian Hippo Pathway. Genes to Cells 2017, 22 (1), 6–31. https://doi.org/10.1111/gtc.12461.

(45) 	Sheridan, J. W.; Simmons, R. J. Studies on a Human Melanoma Cell Line: Effect of Cell Crowding and Nutrient Depletion on the Biophysical and Kinetic Characteristics of the Cells. Journal of Cellular Physiology 1981, 107 (1), 85–100. https://doi.org/10.1002/jcp.1041070111.

(46) 	Tjørve, K. M. C.; Tjørve, E. The Use of Gompertz Models in Growth Analyses, and New Gompertz-Model Approach: An Addition to the Unified-Richards Family. PLOS ONE 2017, 12 (6), e0178691. https://doi.org/10.1371/journal.pone.0178691.

(47) 	Lv, J.; Wang, K.; Jiao, J. Stability of Stochastic Richards Growth Model. Applied Mathematical Modelling 2015, 39 (16), 4821–4827. https://doi.org/10.1016/j.apm.2015.04.016.

(48) 	Peleg, M.; Cole, M. B. Reinterpretation of Microbial Survival Curves. Critical Reviews in Food Science and Nutrition 1998, 38 (5), 353–380. https://doi.org/10.1080/10408699891274246.

(49) 	Wang, Y.; Zhou, J. X.; Pedrini, E.; Rubin, I.; Khalil, M.; Taramelli, R.; Qian, H.; Huang, S. Cell Population Growth Kinetics in the Presence of Stochastic Heterogeneity of Cell Phenotype. Journal of Theoretical Biology 2023, 575, 111645. https://doi.org/10.1016/j.jtbi.2023.111645.

(50) 	Karim, M. A. U.; Aithal, V.; Bhowmick, A. R. Random Variation in Model Parameters: A Comprehensive Review of Stochastic Logistic Growth Equation. Ecological Modelling 2023, 484, 110475. https://doi.org/10.1016/j.ecolmodel.2023.110475.

(51) 	Li, Z.; Yu, J.; Yang, M.; Zhang, J.; Burch, M. D.; Han, W. Cyanobacterial Population and Harmful Metabolites Dynamics During a Bloom in Yanghe Reservoir, North China. Harmful Algae 2010, 9 (5), 481–488. https://doi.org/10.1016/j.hal.2010.03.003.

(52) 	Liang, C.; Wang, D.; Yang, M.; Sun, W.; Zhang, S. Removal of Eathy-Must Odorants in Drinking Water by Powdered Activated Carbon. Journal of Environmental Science and Health 2005, 40 (4), 767–778. https://doi.org/10.1081/ese-200048262.

(53) 	Sherr, E. B.; Sherr, B. F. Preservation and Storage of Samples for Enumeration of Heterotrophic Protists. Handbook of Methods in Aquatic Microbial Ecology. Lewis Publishers, Boca Raton 1993, 207–212.

(54) 	Komárek, J.; Anagnostidis, K. Cyanoprokaryota 1; Deu, 1998; Vol. 19(1).

(55) 	Komarek, J.; Kastovsky, J.; Mares, J.; Johansen, J. R. Taxonomic Classification of Cyanoprokaryotes (Cyanobacterial Genera) 2014, Using a Polyphasic Approach. Preslia 2014, 86, 295–335.

(56) 	Utermöhl, H. Zur Vervollkommnung Der Quantitativen Phytoplankton-Methodik. Mitteilungen Internationale Vereinigung Für Theoretische Und Angewandte Limnologie 1958, 9, 1–38.

(57) 	R Core Team. R: A Language and Environment for Statistical Computing; R Foundation for Statistical Computing: Vienna, Austria, 2021.

(58) 	Timbers, T.; Campbell, T.; Lee, M. R and the Tidyverse. In Data science; Chapman; Hall/CRC, 2022; pp 1–26. https://doi.org/10.1201/9781003080978-1.

(59) 	Wood, S. N. Generalized Additive Models: An Introduction with R, 2nd ed.; Chapman; Hall/CRC, 2017.

(60) 	Handbook of Quantile Regression, 1st ed.; Koenker, R., Chernozhukov, V., He, X., Peng, L., Eds.; Chapman; Hall/CRC, 2017. https://doi.org/10.1201/9781315120256.

(61) 	Wickham, H. Introduction. In ggplot2; Springer New York, 2009; pp 1–7. https://doi.org/10.1007/978-0-387-98141-3_1.

(62) 	Liu, T.; Yu, J.; Su, M.; Jia, Z.; Wang, C.; Zhang, Y.; Dou, C.; Burch, M.; Yang, M. Production and Fate of Fishy Odorants Produced by Two Freshwater Chrysophyte Species Under Different Temperature and Light Conditions. Water Research 2019, 157, 529–534. https://doi.org/10.1016/j.watres.2019.04.004.

(63) 	Perkins, R. G.; Slavin, E. I.; Andrade, T. M. C.; Blenkinsopp, C.; Pearson, P.; Froggatt, T.; Godwin, G.; Parslow, J.; Hurley, S.; Luckwell, R.; Wain, D. J. Managing Taste and Odour Metabolite Production in Drinking Water Reservoirs: The Importance of Ammonium as a Key Nutrient Trigger. Journal of Environmental Management 2019, 244, 276–284. https://doi.org/10.1016/j.jenvman.2019.04.123.

(64) 	Bafford, R. A.; Seagull, R. W.; Chung, S.; Millie, D. F. INTRACELLULAR LOCALIZATION OF THE TASTE/ODOR METABOLITE 2‐METHYLISOBORNEOL IN OSCILLATORIA LIMOSA (CYANOPHYTA)1. Journal of Phycology 1993, 29 (1), 91–95. https://doi.org/10.1111/j.1529-8817.1993.tb00285.x.

(65) 	Pavel, M.; Renna, M.; Park, S. J.; Menzies, F. M.; Ricketts, T.; Füllgrabe, J.; Ashkenazi, A.; Frake, R. A.; Lombarte, A. C.; Bento, C. F.; Franze, K.; Rubinsztein, D. C. Contact Inhibition Controls Cell Survival and Proliferation via YAP/TAZ-Autophagy Axis. Nature Communications 2018, 9 (1). https://doi.org/10.1038/s41467-018-05388-x.

(66) 	Li, H.; Gu, X.; Chen, H.; Mao, Z.; Shen, R.; Zeng, Q.; Ge, Y. Co-Occurrence of Multiple Cyanotoxins and Taste-and-Odor Compounds in the Large Eutrophic Lake Taihu, China: Dynamics, Driving Factors, and Challenges for Risk Assessment. Environmental Pollution 2022, 294, 118594. https://doi.org/10.1016/j.envpol.2021.118594.

(67) 	Shi, X.; Huang, Q.; Shen, X.; Wu, J.; Nan, J.; Li, J.; Lu, H.; Yang, C. Distribution, Driving Forces, and Risk Assessment of 2-MIB and Its Producer in a Drinking Water Source-Oriented Shallow Lake. Environmental Science and Pollution Research 2023, 30 (27), 71194–71208. https://doi.org/10.1007/s11356-023-27506-z.

(68) 	Qi, C.; Fang, J.; Wang, G.; Huang, H.; Wang, Z.; Si, Z.; Zhang, L. Characterization of Odorants in Contrasting Ecotypes of Lake Taihu: Algae-Dominated Versus Macrophyte-Dominated Zones. Environmental Science and Pollution Research 2020, 27 (34), 42221–42229. https://doi.org/10.1007/s11356-020-07896-0.

(69) 	Rong, C.; Liu, D.; Li, Y.; Yang, K.; Han, X.; Yu, J.; Pan, B.; Zhang, J.; Yang, M. Source Water Odor in One Reservoir in Hot and Humid Areas of Southern China: Occurrence, Diagnosis and Possible Mitigation Measures. Environmental Sciences Europe 2018, 30 (1). https://doi.org/10.1186/s12302-018-0175-8.

(70) 	Su, M.; Zhu, Y.; Andersen, T.; Wang, X.; Yu, Z.; Lu, J.; Song, Y.; Cao, T.; Yu, J.; Zhang, Y.; Yang, M. Light-Dominated Selection Shaping Filamentous Cyanobacterial Assemblages Drives Odor Problem in a Drinking Water Reservoir. npj Clean Water 2022, 5 (1), 37. https://doi.org/10.1038/s41545-022-00181-2.

(71) 	Nan, J.; Yang, C.; Yu, H.; Shi, X.; Huang, Q.; Huang, J.; Li, J. Enclosure Culture Ban and Its Effects on Water Quality and Phytoplankton Community in a Subtropical Shallow Lake. Aquaculture Research 2021, 53 (1), 221–231. https://doi.org/10.1111/are.15568.

(72) 	Wang, M.; Yoshimura, C.; Allam, A.; Kimura, F.; Honma, T. Causality Analysis and Prediction of 2-Methylisoborneol Production in a Reservoir Using Empirical Dynamic Modeling. Water Research 2019, 163, 114864. https://doi.org/10.1016/j.watres.2019.114864.

(73) 	Yu, C.; Shi, C.; Ji, M.; Xu, X.; Zhang, Z.; Ma, J.; Wang, G. Taste and Odor Compounds Associated with Aquatic Plants in Taihu Lake: Distribution and Producing Potential. Environmental Science and Pollution Research 2019, 26 (33), 34510–34520. https://doi.org/10.1007/s11356-019-06188-6.

(74) 	Su, M.; Andersen, T.; Burch, M.; Jia, Z.; An, W.; Yu, J.; Yang, M. Succession and Interaction of Surface and Subsurface Cyanobacterial Blooms in Oligotrophic/Mesotrophic Reservoirs: A Case Study in Miyun Reservoir. Science of the Total Environment 2019, 649, 1553–1562. https://doi.org/J.scitotenv.2018.08.307.

(75) 	Xuwei, D.; Min, Q.; ren, R.; Jiarui, L.; Xiaoxue, S.; Ping, X.; Jun, C. The Relationships Between Odors and Environmental Factors at Bloom and Non-Bloom Area in Lake Taihu, China. Chemosphere 2019, 218, 569–576. https://doi.org/10.1016/j.chemosphere.2018.11.121.

(76) 	Li, H.; Li, L.; Yin, Q.; Yu, S.; Gao, N.; Wang, X.; Chen, J. Different Drinking Water Production Schemes to Treat Authentic Algae-Laden Source Water: Removal of 2-Methylisoborneol in Different Forms. Chemical Engineering Journal 2022, 440, 135962. https://doi.org/10.1016/j.cej.2022.135962.

(77) 	Zamyadi, A.; Henderson, R.; Stuetz, R.; Hofmann, R.; Ho, L.; Newcombe, G. Fate of Geosmin and 2-Methylisoborneol in Full-Scale Water Treatment Plants. Water Research 2015, 83, 171–183. https://doi.org/10.1016/j.watres.2015.06.038.

(78) 	Drikas, M.; Dixon, M.; Morran, J. Removal of MIB and Geosmin Using Granular Activated Carbon with and Without MIEX Pre-Treatment. Water Research 2009, 43 (20), 5151–5159. https://doi.org/10.1016/j.watres.2009.08.016.