# Controlling filamentous cyanobacterial blooms requires adaptive, weather-informed strategy

Jiao Fanga,b, Ming Su ID,a,e,*, Yande Lic, Yuying Guia,d, Yufan Aia,e, Ogalo Josepha,e, Tengxin Caoa,e, Shilong Heb, and Min Yanga,e,*

a Key Laboratory of Environmental Aquatic Chemistry, State Key Laboratory of Regional Environment and Sustainability, Research Center for Eco-Environmental Sciences, Chinese Academy of Sciences, Beijing 100085, China.
b School of Environment and Spatial Informatics, China University of Mining and Technology, Xuzhou 221116, China.
c Management Station of Shuangxikou Reservoir, Reservoir Management Service Center of Yuyao, Ningbo 315423, China.
d College of Environmental Science and Engineering, Ocean University of China, Qingdao 266100, China.
e University of Chinese Academy of Sciences, Beijing 100049, China.

* Corresponding to: Ming Su (mingsu@rcees.ac.cn), Min Yang (yangmin@rcees.ac.cn)

# Graphical Abstract

A scalable, adaptive framework that bridges forecasting with intervention, offering a new paradigm for ecological water quality management that supports sustainable drinking water security.

# Abstract

The global expansion of filamentous cyanobacteria threatens water security due to their production of toxins and taste-and-odor compounds. As subsurface dwellers, filamentous cyanobacteria are resistant to conventional nutrient and flocculation controls, exposing a management gap. We developed an adaptive, forecast-guided framework that integrates predictive modeling with precision sediment resuspension (SR), in which SR-associated light attenuation likely contributes substantially to bloom suppression. A 2023-2024 survey of 40 reservoirs in eastern China showed filamentous dominance of over 80% biomass in half the systems. An XGBoost model (R2 = 0.57) identified September-October as the highest-risk period, with over 80% of reservoirs affected. SR efficacy is light-dependent: it suppresses growth under low irradiance but can promote it under high light if shading shifts irradiance into the optimal range for filamentous taxa. We optimized SR through modulated sediment flux (0.1-5.2 g L-1) to dynamically attenuate light in response to real-time forecasts. Field validation confirmed forecast-guided SR effectively limited Pseudanabaena via light control. This ecology-based management provides a scalable framework for sustainable water security under changing climates.

# Keywords

Filamentous cyanobacteria; Sediment resuspension; Light regulation; Machine learning; Water quality management

# Introduction

Global climate change destabilizes freshwater ecosystems and promotes cyanobacterial blooms (Huisman et al., 2018; Paerl and Huisman, 2008). Concurrently, global efforts to reduce nutrient loads are designed to suppress scum-forming genera like Microcystis, but may also alter cyanobacterial succession dynamics and promote increasing dominance or co-occurrence of filamentous cyanobacteria under certain environmental conditions (Komárek and Johansen, 2015; Posch et al., 2012; Su et al., 2019). Such succession patterns may be associated with nitrogen availability and and involve filamentous cyanobacteria with differing nitrogen acquisition strategies, including Pseudanabaena (Acinas et al., 2008) and nitrogen-fixing genera such as Aphanizomenon and Dolichospermum (Beversdorf et al., 2013). This shift complicates management, since many filamentous taxa are important producers of taste-and-odor compounds (e.g., 2-methylisoborneol (2-MIB), geosmin) (Cao et al., 2023; Gaget et al., 2017; Ibelings et al., 2016; Izaguirre and Taylor, 1998; Lu et al., 2022; Mantzouki et al., 2015; Xiao et al., 2024)) and cyanotoxins (Hitzfeld et al., 2000; Lei et al., 2014; Lu et al., 2024; Mohamed et al., 2023; Sinha et al., 2012; Wörmer et al., 2010). And they can impair water quality even at low biomass due to the low odor thresholds of compounds such as 2-MIB (~10 ng L-1) (AWWA, 2010; Watson, 2003)

In contrast to colonial taxa like Microcystis, which thrives across a broader, higher-light range (e.g., 100-500 µmol m-2 s-1 (Hesse et al., 2001; Paerl et al., 1985; Raps et al., 1983; Wiedner et al., 2003)), filamentous cyanobacteria exhibit a unimodal light-response curve with optimal growth at moderate intensities (e.g., 36-85 µmol m-2 s-1 for Pseudanabaena and Planktothricoides) (Cao et al., 2023; Jia et al., 2019; Komárek and Johansen, 2015; Lu et al., 2022; Su et al., 2022; Zohary et al., 2010). Thus they tend to dwell in subsurfaces such as littoral and benthic layers (Gaget et al., 2017; Halstvedt et al., 2007; Su et al., 2019) and access sediment-derived nutrients (Gao et al., 2025; Tatters et al., 2025; Yang et al., 2020), becoming more prevalent as surface-scum blooms weaken due to intensified nutrient reduction efforts. Conventional bloom-control strategies, including nutrient management (Fastner et al., 2015), hydrodynamic adjustments (Lu et al., 2023), and chemical (Anantapantula et al., 2025; Kibuye et al., 2021a; Tsai, 2016; Xu et al., 2024), physical (Bormans et al., 2015; Lee et al., 2014), or biological approaches (Kibuye et al., 2021b), are less effective against these subsurface-dwelling filamentous cyanobacteria (Flores and Herrero, 2009; Tamulonis et al., 2011; Zhi et al., 2023), which often exhibit greater resistance and more treatment-related constraints (Table S1). These strategies were designed for the control of surface-scum, buoyant and colonial taxa like Microcystis, exposing a gap in contemporary water quality management approaches.

Given their subsurface-dwelling niche characteristics, regulating the underwater light climate has been recognized as a strategy for controlling filamentous cyanobacteria (Su et al., 2021; Zohary et al., 2010). For example, the suppression of MIB-producing Planktothrix in deep reservoirs has been achieved through water-level manipulation which minimizes shallow areas and reduces underwater light availability (Su et al., 2017, 2015). We developed a sediment resuspension (SR)-based technique that mechanically disperses sediments at the water surface. This method controls cyanobacterial blooms by reducing underwater light availability and promoting particle flocculation, thereby facilitating the removal of algae and phosphorus. This approach can be applied in a wider range of reservoirs than water level manipulation (Fang et al., 2024) and is effective against surface-scum-forming Microcystis due to their predominantly linear light-response pattern under common weather conditions. However, the method is less effective against filamentous cyanobacteria due to their linear trichome morphology, weaker aggregation capacity, and lower optimal light requirements under subsurface environments (Flores and Herrero, 2009; Jia et al., 2019; Lu et al., 2022; Tamulonis et al., 2011; Wang et al., 2011; Xu et al., 2019; Zhi et al., 2023). Reducing underwater light intensity through SR or other measures may favor these subsurface‑dwelling filamentous taxa, especially under high solar radiation conditions that would otherwise inhibit their growth. Based on these ecological characteristics, we hypothesize that filamentous cyanobacteria occupy a distinct ecological niche, with heightened sensitivity to underwater light availability and optimal growth under moderate light intensities. Consequently, effective bloom management in filamentous‑dominated water bodies must account for real‑time weather conditions; failure to do so may lead to suboptimal or even counterproductive outcomes. We further hypothesize that integrating real‑time weather forecasting with adaptive control strategies provides a robust framework for managing such blooms—a prediction we test here using SR as an experimental tool.

We present an adaptive framework integrating weather forecasting, predictive modeling, and precision sediment resuspension to mitigate harmful filamentous cyanobacterial blooms. The approach, developed from a one-year survey of 40 reservoirs in eastern China, employs a machine-learning model to assess bloom risk and uses field-based light manipulation experiments targeting Pseudanabaena, a dominant filamentous cyanobacterial genus frequently associated with bloom events (Xiao et al., 2024) during investigation, to define critical light thresholds. The resulting adaptive strategies dynamically combine real-time meteorological data with species-specific niche characteristics for targeted bloom control. This framework provides a practical solution for managing blooms that cause taste-and-odor issues and cyanotoxin production, advancing sustainable water quality management.

# Materials and methods

## Sample collection in 40 reservoirs and preparation

Water samples (1 L) from the surface layer (0.5 m) were collected using a Kemmerer water sampler from 40 drinking water reservoirs in Zhejiang Province, China, covering 4 cities with a latitude spanning from 28°00’ N to 31°50’ N (Fig. 1). The reservoirs primarily consisted of important water source systems across eastern China, covering a range of nutrient environments (Fig. S2). Physicochemical parameters (pH, redox potential (ORP), depth, turbidity, water temperature, and chlorophyll-a) were recorded using a multi-water quality parameter instrument (YSI 6600V2, US) during each sampling session. The mixed layer depth (zmix) was identified as the depth at which the temperature deviated by 0.2-0.5 °C from the surface temperature (Kara et al., 2000; Read et al., 2011). For phytoplankton analysis, 500 mL subsamples were preserved with 5% Lugol’s iodine (v/v), settled for 48 h, and preconcentrated 20× prior to microscopic enumeration (Sherr and Sherr, 1993). Phytoplankton identification followed refs (Bellinger and Sigee, 2015; Komarek et al., 2014; Komárek and Anagnostidis, 1998), using an upright microscope (Olympus BX53, Japan) following the protocol established by ref. (Martinez et al., 1975). The filamentous cyanobacteria abundance was calculated based on the length of each filament and the mean cell length of each strain. The number of cells in colony species such as Microcystis sp. was estimated based on colony volume and mean cell number per volume. The mean cell morphological characteristics including cell length, cell volume etc. were determined according to >50 filaments/colonies of each strain using a in-house developed cell counting tool (CCT v1.4, https://drwater.net, in Chinese) (Fang et al., 2024; Lu et al., 2022).

## In-situ suppression of Pseudanabaena under varying light attenuation levels

Field experiments targeting Pseudanabaena suppression were conducted in S Reservoir (29°93’N, 121°33’E, Fig. 3a) during a bloom event from May to June 2024, with initial Pseudanabaena abundance reaching (9.81 ± 0.43) × 108 cells L-1. Five custom cylindrical simulators (height: 2.0 m, diameter: 0.5 m), designated L1-L5, were fixed on a floating platform. Each simulator was filled with raw bloom water from the reservoir, maintaining surface level alignment with the reservoir. A 0.2 µm filter membrane at the base prevented introduction of external algae while permitting natural nutrients exchange. Variable irradiance conditions were created using light-blocking films with five transmittance areas: 0.004 m2 (L1), 0.026 m2 (L2), 0.068 m2 (L3), 0.102 m2 (L4) and 0.196 m2 (L5). HOBO Pendant 64K loggers (UA-002-64) positioned at different depths (0.05 m, 0.5 m, 1.0 m, 2.0 m) recorded water temperature and light intensity at 15-minute intervals. The initial median light intensity (0.05 m, 9:00 and 16:00) in each simulator on Day 1 of SR deployment was 5.9 µmol m2 s-1 (L1), 89.0 µmol m2 s-1 (L2), 248.3 µmol m2 s-1 (L3), 571.1 µmol m2 s-1 (L4) and 670.4 µmol m2 s-1 (L5), respectively (Fig. S3). Since the experiment was conducted in-situ, light availability in the setup was weather-dependent. Daily water samples (300 mL) were collected from three depths (0.5 m, 1.0 m and 2.0 m) between 08:00 and 09:00 h. Subsamples (200 mL) were preserved with 5% Lugol’s iodine for cell enumeration, left to settle for 48 hours, and pre-concentrated 10× before being stored in the dark for later counting. Parallel subsamples (100 mL) were filtered through 0.45 µm PES membrane for nutrient concentrations analysis (total nitrogen, total phosphorus, total ammonia nitrogen) using a Continuous Flow Analyzer (SAN ++, Skalar Analytical B.V).

## In-situ assessment of underwater light attenuation under varying sediment loads

Five cuboid simulators (4.0 m × 0.5 m × 0.5 m) were deployed on a floating platform with open hydraulic exchange (Fig. 4a). Native sediment samples collected by Ekman dredge were partitioned into five portions with different weights: 0.3 kg, 6.3 kg, 12.4 kg, 18.5 kg and 25.0 kg. Due to a moisture content of approximately 67%, the dry weights (sediment particles) in each portion were approximately 0.1 kg, 2.0 kg, 4.0 kg, 6.0 kg, and 8.0 kg, respectively. These were homogenized (30 min orbital mixing) and slowly injected into simulators (7:00-8:00) to achieve initial sediment concentration of 0.1-8.0 g L-1. Five HOBO Pendant 64K loggers (UA-002-64) recording temperature and light intensity were positioned at different depths (0.5 m, 1.0 m, 2.0 m, 3.0 m and 4.0 m) with 15-min resolution.

## The field application of sediment resuspension in S Reservoir

Severe Pseudanabaena and Raphidiopsis blooms were found in S Reservoir, an anonymized drinking-water reservoir in eastern China (29°55’50.2’‘N, 121°19’53.03’’E) in June 2023. Short-term SR operations (two devices, day 1-5, 10 h per day) were conducted around risk area (29°55’38.39”N, 121°20’2.86”E, 12 ha). Surface sediment was dredged by shipborne SR equipment and sprayed to water surface (Fang et al., 2024). Each position sediment was re-suspended approximately 5 times per day (n = 5). The water depth of the area was around 28.0 m. After 5 days, only one device remained operating in the reservoir due to decreased light intensity and reduced bloom density (Fig. S12). The sediment concentration (csed) and control responses in this operation were analysed according to our previous study (Fang et al., 2024).

Water samples (1 L) from the surface layer (0.5m) were collected every 2 days using a Kemmer water sampler from each site. The cell counting and preparation method was described in section 2.1. Fifteen temperature and light data loggers (HOBO Pendant 64K, UA-002-64) were positioned within the water column at different depths (uniformly distributed throughout the entire depth). Water temperature and underwater light intensities were recorded every 15 minutes.

## Data analysis

### Data calculation

The extinction coefficient (k, m-1) was determined according to a logarithmic-linear model between underwater light intensity (I, µmol m-2 s-1) and water depth (z, m) at different depths, following the Beer-Lambert Law (Cao et al., 2023) (Eq. 1).

Iz=I0e−kz  1

where I0 represents light intensity at the surface and Iz represents light intensity at depth z.

Planktonic cells can move vertically in the water column, resulting in non-homogeneous light availability over time. We use a simplified model that assumes cells are evenly distributed in surface mixed layer (zmix, m). The instantaneous light available to an algal cell (Ic, µmol m-2 s-1) was calculated by Eq. 2.

Ic=I01−e−kzmixkzmix  2

According to Eq. 2, the daily underwater light available (Idc, mol m-2 d-1) was calculated by Eq. 3.

Idc=∑Ici+Ici+15×900  3

where Ici represents Ic analysed every 15 minutes.

Regarding the control experiment, the cell control rate (CR) was calculated based on the cell density decrease (Nt2/Nt1) over time (t2−t1, d), which was determined by the slope of log10-linear model between Nt and t:

CR=log10Nt2−log10Nt1t2−t1  4

The impact of SR on the extinction coefficient dynamics was analysed. The extinction coefficient (k, m-1) followed a logarithmic decay model with time (t, min) within each treatment (Eq. 5):

ϕ=ϕ0−ρlog10t  5

Here, ϕ=k/k0 is the relative extinction coefficient, with ϕ0 representing its initial value at t = 1 min after SR and k0 representing the background extinction coefficient due to water properties and pre‐existing bloom self‐shading. The parameter ρ quantifies the rate of decline in ϕ over time. Both ϕ0 and ρ were subsequently described using second‐order polynomial models (Eq. 6, Eq. 7):

ϕ0=α+βcsed+γcsed2  6

ρ=δ+εcsed+ζcsed2  7

Based on these equations (Eq. 6, Eq. 7), the extinction coefficient (k, Eq. 8) is expressed as a function of time (t) relative to SR, the concentration of resuspended sediment (csed), and the background extinction coefficient (k0):

k=k0ϕ0−ρlog10t  8

### Workflow for determining optimal sediment resuspension (SR) operating conditions

SR intensity is defined by suspended-sediment concentration (csed) and resuspension frequency (n, cycles d-1), with lower frequency considered more cost-effective (Fig. S13). First, we calculated the critical control rate (CR*) based on the modeled risk (RM, expressed as filamentous cyanobacteria cell density) and our target risk level (RL, expressed as filamentous cyanobacteria cell density) to be achieved within N days (Eq. 9).

CR*=log10RM−log10RL/N  9

Second, the required underwater-light reduction (expressed as critical light dose Idc*) was determined from CR* via an Idc-CR model derived from in-situ experiments (Fig. 3e). Third, minimum SR intensities (Fig. 5a; Fig. S13) were calculated using an SR-underwater-light model based on the nonlinear relationship between sediment concentration (csed) and light conditions, quantified by the light-extinction coefficient (k) and light dose (Idc) (Fig. 4d, Fig. S26-Fig. S65). SR operation may not be required under certain weather conditions and current risk levels; such cases are labelled “Not Necessary”. Additionally, SR may be deemed “Not Applicable” when the required frequency exceeds practical or cost-effective limits-here, more than five cycles per day was considered infeasible.

### Statistical analysis and model evaluation

All analyses and visualizations were conducted in R 4.0 using the vegan and tidyverse packages (Dixon, 2003; R Core Team, 2021; Wickham et al., 2019). Box plots show medians, interquartile ranges (IQR), and 1.5 × IQR whiskers, with outliers indicated as points; other variables are presented as mean ± standard error. Statistical significance was defined at p < 0.05. Bloom risk was predicted using an XGBoost regression model (500 trees, trained with a 5/6-1/6 train-test split). Model robustness was assessed using 10-fold cross-validation, and performance evaluated by R2, RMSE, MAE, and bias calculated on log10-transformed cyanobacterial abundance. Sensitivity analysis was further conducted by analysing optimal SR conditions across a broad range of predicted bloom intensities and meteorological conditions (Fig. 5c). To account for biological response lags, light exposure was represented by cumulative or averaged cellular light dose over a defined time window (e.g., the days preceding a forecast target), rather than instantaneous irradiance.

# Results

## Widespread filamentous cyanobacterial dominance in eastern Chinese drinking water reservoirs

Our investigation of 40 reservoirs in eastern China (28°00’-31°50’N; Fig. 1a) revealed that over 45% (18 reservoirs) exhibited total cyanobacterial densities exceeding 5 × 107 cells L-1 (Fig. S1) during summer and autumn-surpassing the moderate bloom threshold defined by China’s National Environmental Protection Standard (HJ 1098-2020) and aligning with WHO guideline ranges (Ingrid and Jamie, 1999; Pilotto et al., 1997). Only one reservoir (ZGZ) maintained densities below 1 × 106 cells L-1 (Fig. S1). Filamentous cyanobacteria (e.g., Pseudanabaena, Raphidiopsis, Lyngbya) dominated (>50% relative cell abundance) in 72.5% of reservoirs (29/40), with 52.5% (21/40) exceeding 80% relative cell abundance. In contrast, Microcystis prevailed in 22.5% (9/40) of reservoirs (Fig. 1d). Mean densities reached 0.84 × 108 cells L-1 for filamentous and 0.28 × 108 cells L-1 for colonial cyanobacteria (Fig. 1c), with maxima of 4.70 × 108 cells L-1 (filamentous) and 3.58 × 108 cells L-1 (colonial) (Fig. 1c). Over 10 filamentous genera were identified (Fig. 1b&e), led by Pseudanabaena (25.34%), Raphidiopsis (21.56%), and Lyngbya (15.22%) (Fig. 1f). Pseudanabaena exceeded 1 × 108 cells L-1-a level comparable to WHO alert thresholds (Ingrid and Jamie, 1999)-in three reservoirs, where it comprised >90% of total cyanobacterial abundance. This genus was detected in 30 reservoirs, 13 of which surpassed 2 × 107 cells L-1 (Fig. 1e).

Fig. 1: Spatial distribution of filamentous cyanobacteria in 40 drinking water reservoirs in eastern China. a, Taxonomic composition and morphological partitioning (filamentous/colonial/other) across reservoirs. b, Spatial distribution of filamentous genera. c, Cell density ranges for total cyanobacteria and subcategories during 2023-2024 survey. d, Bloom intensity stratification for the cyanobacteria. The coral orange, light orange, azure blue and light grey indicates filamentous cyanobacteria dominance (>80% relative abundance), filamentous cyanobacteria dominance (>50% relative abundance), colony-forming cyanobacteria dominance (>50% relative abundance) and other cyanobacteria dominance, respectively. e, Spatial occurrence frequency of major filamentous cyanobacterial genera across 40 reservoirs, expressed as the number of reservoirs where each genus was detected. Bar shading indicates bloom risk tiers based on cell density, with four categories indicating progressively increasing bloom risk, from < 2 × 107 cells L-1 to > 1 × 108 cells L-1. Pseudanabaena and Raphidiopsis exhibit bimodal dominance, collectively occupying > 15% of high-risk reservoirs (Levels 3–4) while maintaining baseline populations in 57% of low-risk systems (Levels 1), reflecting niche differentiation along trophic gradients. f, Relative contribution of filamentous genera to total cell density, showing the proportional composition across all sampled reservoirs.

## XGBoost-based forecasting of filamentous cyanobacterial bloom risks

An XGBoost-based model was developed to assess the risk of filamentous cyanobacterial blooms in 40 drinking water reservoirs across various months. The model incorporated key variables, including mixing layer depth (zmix, Fig. S5), cellular light available (Idc, Fig. S6), water temperature (T, Fig. S7), surface irradiance, sampling month, and reservoir latitude. Model performance was evaluated using a held-out test set. It demonstrated predictive performance within the range of 106 to 109 cells L-1, achieving an R2 of 0.5704 for the testing dataset (p < 0.0001). The model yielded an RMSE of 0.24, an MAE of 0.17, and a bias of 0.02 in log10-transformed abundance (Fig. 2a). Using this model, the bloom risk was evaluated for the months between March and October (Fig. 2b) while the remaining months were excluded from the model’s scope due to lower temperatures and minimal bloom risks.

The lowest risk of 0.34 × 108 cells L-1 occurred in March (Fig. S8a), coinciding with the lowest water temperature (14.34 ºC; Fig. S8c) and light intensity (0.74 mol m-2 d-1; Fig. S8b). August exhibited a relatively low risk (0.25 × 108 cells L-1), despite having the highest water temperature (30.92 ºC) and light intensity (2.15 mol m-2 d-1). Higher bloom risks were observed in months with moderate water temperature and light intensity, including October (0.78 × 108 cells L-1), September (0.77 × 108 cells L-1), May (0.25 × 108 cells L-1), and June (0.42 × 108 cells L-1).

Bloom risk for filamentous cyanobacteria was evaluated according to the WHO guidelines and applied for control targets by SR: Risk Level 1 (RL1) at 1 × 108 cells L-1 and Risk Level 2 (RL2) at 2 × 107 cells L-1 (Fig. S9) (Ingrid and Jamie, 1999). The proportion of reservoirs exceeding RL1 was 40% (16 reservoirs) in October and 15% (6 reservoirs) in September. For RL2, exceedance occurred from March to October, peaking in October (90%) and September (82.5%). Notably, 62.5% of reservoirs still exceeded RL2 even in March, a month of typically low bloom risk.

Fig. 2: Machine learning-driven risk assessment framework for filamentous cyanobacterial blooms. a, XGBoost model performance validation against reservoir-month observations (n = 303). b, Spatiotemporal prediction of bloom intensities across 40 reservoirs from March to October.

## Underwater light thresholds for the suppression of filamentous cyanobacteria based on in-situ experiment

Culture studies indicate that filamentous cyanobacteria have lower optimal light requirements than surface-scum-forming Microcystis (Cao et al., 2023; Jia et al., 2019; Lu et al., 2022; Su et al., 2022; Wang et al., 2011). Given its widespread occurrence across the surveyed reservoirs, Pseudanabaena was selected as a model taxon to evaluate light-regulation strategies (Fig. 1b, c). We conducted a light-gradient experiment in S Reservoir under five natural irradiance levels (Ic, L1-L5: 6.3 µmol m-2 s-1 - 603.3 µmol m-2 s-1; Fig. 3b, c). Light intensities of 466.3 µmol m-2 s-1 or lower (L1-L4) suppressed Pseudanabaena (p<0.01), reducing abundance by 0.06 d-1 - 0.13 d-1 (Fig. 3d). By day 12, cell densities in L1-L4 fell to (0.10 ± 0.00) × 108 cells L-1 - (1.27 ± 0.00) × 108 cells L-1, while L5 (603.4 µmol m-2 s-1) maintained 47.5% survival ((4.66 ± 0.00) × 108 cells L-1) despite identical initial densities ((9.81 ± 0.43) × 108 cells L-1; Fig. 3d). A linear model (CR=−0.14Idc+0.44, R2=0.985, p=0.0007; Fig. 3e) confirmed that reducing underwater light intensity (Idc) limits Pseudanabaena proliferation, with each decrease of 1 mol m-2 d-1 in light intensity increasing the control rate (CR) by 0.14 d-1.

Fig. 3: Light-response dynamics of Pseudanabaena suppression through precision light modulation. a, Five water columns (2 m depth) with gradient shading film (Light-transmitting area: 0.004 m2 - 0.196 m2) establishing five irradiance regimes (initial light intensity Ic: 5.9 µmol m-2 s-1 - 670.4 µmol m-2 s-1, Supplementary Fig. 2) in S Reservoir. b, Light intensities across treatments. Each light intensity (Ic) referred to the mean light intensity (0.05 m) from 9:00 to 16:00. c, Quantifies of 12-day light regimes (n = 12). d, Pseudanabaena abundance reduction across treatments. e, The relationship between Pseudanabaena control rates (CR) and underwater light availability (Idc).

## Dynamic modeling of light attenuation through sediment resuspension

We evaluated light attenuation through SR in a field experiment in S Reservoir, applying five sediment concentrations (csed: 0.1, 2.0, 4.0, 6.0, and 8.0 g L-1; Fig. 4a, b). The ϕ0 and ρ were described using second-order polynomial models (Fig. 4c) as shown below:

ϕ0=0.781+2.03csed−0.196csed2  10

ρ=−0.0713+0.601csed−0.0567csed2  11

k=ϕ0−ρlog10tk0  12

Based on these equations (Eq. 10, Eq. 11), the extinction coefficient (k, Eq. 12) is expressed. To illustrate the model’s behavior, we simulated sediment concentrations ranging from 0.1 g L-1 to 8 g L-1 and time intervals from 1 minute to 1000 minutes (Fig. 4d). The model revealed a nonlinear response of k to csed, with k plateauing at an intermediate sediment concentration of 5.2 g L-1 (95% CI: 4.1-6.2 g L-1) under the tested sediment properties.

Fig. 4: Sediment resuspension-driven light extinction coefficient dynamics. a, Five water columns (4 m depth) with gradient sediment concentration (csed: 0.1 g L-1 - 8.0 g L-1) establishing five dynamic extinction coefficient regimes in S Reservoir. b, Time-resolved extinction coefficient (ø = k / k0) decay under varying sediment loads (0.1-8.0 g L-1). c, Initial extinction-scaling factor ø (ø0) versus sediment concentration (csed). d, ø decay rate (ρ) dependence on particulate loading. d, Validated extinction model spanning 0.1 g L-1 to 8 g L-1 concentrations and 1 - 1000 timescales.

## Weather forecast-based operational sediment resuspension to control filamentous cyanobacteria

To suppress filamentous cyanobacteria to a target risk level under variable weather, we developed a decision cascade integrating predictive modeling with SR-based light regulation (Fig. S13).

We applied the framework (Fig. S13) to the 40 reservoirs based on their modeled monthly risk levels from March through October, determining the SR intensity (ISR) required to suppress filamentous cyanobacteria to RL2 (2 × 107 cells L-1) within N=10 days (Fig. 5a). In March, 32.5% (13/40) of reservoirs, and in August, 42.5% (17/40), required no intervention because cyanobacterial densities remained below the target threshold (Fig. 4), aligning with lower bloom risks during these months. Moderate SR intensities (0.1-0.5 g L-1) were sufficient for 20-60% of reservoirs across the study period, peaking at 60% in March, April, and September (Fig. 5a, Fig. S11), highlighting broad applicability under typical conditions. Three reservoirs-LY (June), LHT (July), and XK (October)-required elevated SR intensities (3.0-4.0 g L-1). The most demanding scenarios (4.0-5.2 g L-1) targeted reservoirs with extreme risks, including CT (July, 1.95 × 108 cells L-1), HM (March, April, June, August; 1.94 × 108 cells L-1 - 3.00 × 108 cells L-1), and SXK (June, 2.41 × 108 cells L-1). SR efficacy was limited in reservoirs such as HM (May, July, September, October), SXK (May), and GXZ/YML (October), where achieving the target suppression required operational durations exceeding 10 days-a constraint attributed to light-limited sediment-retention efficiency.

The field validation of the framework’s predictive accuracy (predicted: 2.41 × 108 cells L-1, observed: 2.23 × 108 cells L-1; Fig. 2) and necessary weather‑adaptive SR operations was conducted in a single reservoir (S Reservoir) during one bloom event, without replicated treatments or a control (Fig. 5b). This was due to operational constraints imposed by the reservoir authority, which prioritizes drinking water safety and maintains fixed SR schedules. Nevertheless, the field-implemented SR (csed=1.5 g L-1, n=5 cycles d-1) achieved a control efficiency of 0.10 d-1 and reduced biomass to 0.89 × 108 cells L-1 within 5 days. This was followed by further decline associated with weather conditions, which is consistent with the mechanistic predictions from our mesocosm (Fig. 3) and literature‑based analyses (Fig. S25). Future work should include replicated, multi‑site validations to strengthen the statistical generalizability of the framework.

For broader scenarios, we evaluated SR applicability across varying initial algal densities (0.8-2.8 × 108 cells L-1, Fig. S18-Fig. S23), mixing depths (6-28 m, Fig. S16), and daily irradiance (I0: 0-20 mol m-2 d-1) (Fig. 5c), which also provides a sensitivity assessment of how uncertainty in bloom intensity influences SR recommendations. At a mixing depth of 6.0 m and relatively low initial densities, SR consistently reduced abundance below RL2 (2 × 107 cells L-1) within 10 days across all I0 conditions. Under low-light regimes (I0 below 10.5 mol m-2 d-1, 7.9 mol m-2 d-1 and 6.1 mol m-2 d-1 for densities of 0.8 × 108 cells L-1, 1.2 × 108 cells L-1 and 1.6 × 108 cells L-1), cyanobacteria declined naturally without intervention. In contrast, at higher initial densities, suppression failed when I0 exceeded critical thresholds (16.6 mol m-2 d-1, 12.5 mol m-2 d-1 and 9 mol m-2 d-1 for 2.0 × 108 cells L-1, 2.4 × 108 cells L-1 and 2.8 × 108 cells L-1), even with 10-day SR. Higher I0 generally required greater SR intensity (csed×n)-for example, at 2.8 × 108 cells L-1, 8.6 ≤ I0 ≤ 8.9 mol m-2 d-1 demanded 4.0 - 4.8 g L-1 (n = 5), whereas 0.1 - 4.7 g L-1 (n = 1) sufficed at I0 = 6.0 mol m-2 d-1. These thresholds scale with typical seasonal irradiance, which ranges from 2.4 - 10.9 mol m-2 d-1 (winter) to 10.6 - 18.3 mol m-2 d-1 (summer).

Fig. 5: Operational optimization of sediment resuspension (SR) for filamentous cyanobacteria control. a, Optimal SR operation parameters (csed: 0.1 g L-1 - 5.2 g L-1 × resuspension frequency: n) achieving Risk Level 2 targets (2 × 107 cells L-1) within 10 days across 40 reservoirs. b, Case study in S Reservoir showing efficiency of filamentous cyanobacteria control via SR (1.5 g L-1 × 5 cycles d-1). c, Optimal SR flux (csed and n) for control of filamentous cyanobacterial blooms under various meteorological conditions (daily surface light intensity, I0, 0 - 20 mol m-2 d-1) and bloom intensities (80 - 280 × 106 cells L-1) at mixed layer depth of 6.0 m.To represent realistic seasonal weather variability, the 20th - 80th percentile range of daily surface irradiance (I0) was used for each season. Lower irradiance values (approaching the 20th percentile) generally represent cloudy or rainy conditions, intermediate values represent typical overcast conditions, and higher values (approaching the 80th percentile) represent clear-sky conditions. Each rectangular bar represents the SR management strategies under different bloom intensity conditions across varying I0. The green, blue and orange segment represent SR is not necessary, applicable (i.e., SR can suppress cyanobacterial growth effectively) and not applicable (i.e., beyond the controllable ranges of SR), respectively, under under different light conditions and bloom conditions. Within the effective light range for SR application, higher surface irradiance (I0) corresponds to a greater light attenuation requirement, thereby necessitating stronger sediment resuspension intensity (0.1 – 5.2 g L-1 × 1 – 5 cycles d-1).

# Discussion

The succession of phytoplankton communities under global climate change scenarios has been extensively discussed, though the direction of this succession remains inconclusive (Benedetti et al., 2021; Dai et al., 2023; Henson et al., 2021; Winder and Sommer, 2012). While many studies suggest that global warming favors the proliferation of warm-adapted, buoyant cyanobacteria such as Microcystis (Carey et al., 2012), the situation in China appears to differ. Our survey of 40 reservoirs, along with observations from major lakes such as Lake Taihu and Lake Chaohu, indicates that submerged filamentous cyanobacteria have become the dominant cyanobacterial group in many Chinese water bodies in recent years (Ai et al., 2025; Huo et al., 2021; Su et al., 2022; Zhang et al., 2020).

The mechanisms driving this shift remain incompletely understood, but we suggest that China’s intensive, nationwide nutrient-reduction efforts may be a key contributing factor. Nutrient reduction may favor the proliferation of filamentous cyanobacteria (Su et al., 2019) by creating ecological conditions that select for taxa with enhanced tolerance to low-light environments, access to sediment-derived nutrients, and, in some genera, nitrogen-fixing capability (Halstvedt et al., 2007; Komárek and Johansen, 2015), while simultaneously reducing the competitive advantage of surface-scum-forming Microcystis. The decline of dense surface blooms may also increase light penetration into deeper water layers and littoral sediments, potentially facilitating the growth of subsurface and benthic filamentous cyanobacterial populations (Catherine et al., 2013; Scott and Marcarelli, 2012). Such benthic populations may further act as persistent source pools of cyanobacterial biomass, contributing to subsequent bloom development. In addition to light adaptation, differences in nitrogen acquisition strategies may also contribute to cyanobacterial succession. Unlike Microcystis (Kim et al., 2020), some filamentous genera such as Aphanizomenon and Dolichospermum can fix atmospheric nitrogen (Feuring et al., 2026; Fiore et al., 2005; Z. Liu et al., 2024; Scoglio et al., 2024) and may therefore gain a competitive advantage under nitrogen-limited conditions (Y. Liu et al., 2024). Although nitrogen limitation was not evident in our field surveys (Fig. S2) or experiments (Fig. S4), variation in nitrogen-fixing capacity among filamentous cyanobacteria may partially influence community succession. Future studies could further evaluate the potential of nif-related indicators for adaptive bloom management (Cao et al., 2023; Dixon and Kahn, 2004; Pi et al., 2025; Tromas et al., 2017).

While water temperature is a key driver of growth (Karlberg and Wulff, 2012; Maurer et al., 2024; Thomas and Litchman, 2015), it is difficult to manipulate in the field. The submerged niche makes light regulation a practical control strategy-either by increasing the light-transmission distance (water-level manipulation) or adjusting the light-extinction coefficient (turbidity regulation) according to the Lambert-Beer law (Su et al., 2021, 2017). Understanding their light response is essential for effective implementation. This narrower, irradiance-defined growth window suggests that filamentous cyanobacterial dynamics are sensitive to short-term light variability (Fig. S17).

Sediment resuspension (SR) has emerged as a light-regulation-based strategy for cyanobacterial control. It is suitable for drinking-water reservoirs as it avoids chemical use and is economically feasible compared to alternatives such as salvage and sediment dredging (Yu et al., 2017). Although SR effectively suppresses Microcystis blooms through flocculation and settling (Fang et al., 2024), its efficacy against filamentous cyanobacteria is complicated by their low light requirements and greater resistance to flocculation (Lucas Pardo et al., 2015; Song et al., 2023; Zhi et al., 2023). Our findings reveal that SR-associated light attenuation may be an important factor influencing control efficacy and this effect is dynamically modulated by ambient weather conditions. Under high solar radiation, SR-induced shading risks alleviating photo-inhibition, thereby promoting growth (Fig. S17, Fig. S24). In contrast, under reduced ambient light (e.g., overcast or post-rain conditions), SR-associated shading may drive underwater light below survival thresholds. These findings suggest that SR implementation could benefit from a forecast-driven adaptive framework, targeting intervention windows when ambient light conditions are more likely to facilitate bloom suppression.

By integrating weather forecasting, our strategy dynamically adjusts SR flux to optimize underwater light climate and maximize control of filamentous cyanobacteria. We present a scalable, adaptive framework that bridges forecasting with intervention, offering a paradigm for ecological water quality management that supports sustainable drinking water security (Fig. 6). This approach offers operational advantages: it aligns control with forecasted environmental windows (as demonstrated in S Reservoir, where further SR was halted after five days in response to weather changes), increases ecological precision by tailoring light attenuation to species-specific light-response traits, and enables proactive, data-driven management that adapts to both short-term weather variability and long-term climate trends-shifting cyanobacterial control from reactive mitigation to forecast-guided prevention.

Fig. 6: Predictive and operational framework for harmful cyanobacterial control: Mitigation of filamentous cyanobacterial blooms through light regulation by dynamic SR flux under various meteorological conditions.

SR promotes sediment oxidation, enhancing aerobic processes and suppressing the release of reduced solutes, thereby immobilizing phosphorus, iron, and manganese in the sediment (Broman et al., 2017; Chen et al., 2021; Fang et al., 2024; Fu et al., 2024). However, this process also entails risks, most notably a transient manganese pulse when background soluble Mn exceeds ~100 μg L‐1 (Su et al., 2025). Potential impacts on aquatic biota—including fish and microbial communities—appear limited under moderate SR regimes, with field observations indicating no detectable ecological disruption. Although fish responses were not directly assessed, moderate SR‐induced nutrient exchange may indirectly support higher trophic levels and contribute to top‐down control of algal biomass (Havens, 1991; Yahel et al., 2008).

Our study provides a practical pathway for implementing this adaptive framework while highlighting areas for future refinement. The XGBoost model demonstrated moderate predictive performance (R2 = 0.57), consistent with regional environmental models operating under limited spatiotemporal resolution (Abbas et al., 2023; Pawley et al., 2024; Read et al., 2014). Its fidelity is constrained by monthly sampling frequency, cross-reservoir heterogeneity, and the absence of short-term drivers like sub-daily light variability (Wahlin and Grimvall, 2008). Importantly, sensitivity analyses Fig. 5c across a broad range of bloom intensities and meteorological conditions demonstrated that uncertainty in predicted bloom intensity influences the magnitude of recommended SR interventions, while the overall light-regulation framework remains robust. Higher operational accuracy will require reservoir-specific models trained on high-frequency, long-term datasets, (e.g., 2–3 years per reservoir) together with periodic retraining to accommodate changing environmental conditions and future climate scenarios.

Although light thresholds were derived from Pseudanabaena, whose filamentous and photophysiological traits are representative of many nuisance taxa, extending threshold determination to a broader range of cyanobacterial genera would enhance ecological generality and predictive robustness. For instance, taxa such as Aphanizomenon may become dominant under cooler conditions, taxa such as Dolichospermum may may maintain competitive advantages under nitrogen-limited environments, whereas other genera may prevail under different seasonal light and temperature regimes (Feuring et al., 2026; Z. Liu et al., 2024; Maurer et al., 2024; Paerl and Otten, 2015). In addition, the optimal irradiance for cyanotoxin production may differ from that for cyanobacterial growth, potentially leading to asynchronous responses between bloom suppression and toxin or taste-and-odor control under light-regulation strategies (Cao et al., 2023). Consequently, future adaptive management frameworks should incorporate taxa-specific ecological characteristics, integrate cyanotoxin monitoring, and explore the potential value of nif-related indicators for anticipating shifts toward nitrogen-fixing filamentous cyanobacteria (Cao et al., 2023; Dixon and Kahn, 2004; Pi et al., 2025), while light-regulation thresholds may require calibration according to the dominant cyanobacterial community in different reservoirs and seasons.

In operational deployments, SR protocols should be calibrated using site-specific optical and sediment properties. Background light attenuation (k0) should be constrained through in-situ measurements, while sediment composition and settling dynamics must inform the optimization of SR intensity and frequency (Hipsey et al., 2019; Kirk, 2010). The framework is scalable and transferable beyond drinking-water reservoirs. It enables risk-informed, spatially targeted interventions in larger or more heterogeneous systems, minimizing basin-wide disturbance while improving operational efficiency and economic sustainability.

# Conclusions

This study demonstrates that filamentous cyanobacteria have become the dominant phytoplankton group in drinking water reservoirs in eastern China. The XGBoost-based predictive model identified September-October as the highest-risk period for filamentous cyanobacterial blooms. Field experiments established critical light thresholds for Pseudanabaena suppression and light attenuation model by SR under ambient weather conditions. Accordingly, we developed an adaptive, weather-informed framework that integrates machine learning forecasting with precision sediment resuspension for bloom control. This adaptive approach providing a scalable solution for sustainable water quality management under changing climates.

# Author contributions

- Jiao Fang: Writing - review & editing, Writing - original draft, Visualization, Methodology, Investigation, Formal analysis, Data curation

- Ming Su: Writing - review & editing, Writing - original draft, Visualization, Validation, Supervision, Formal analysis, Data curation, Conceptualization, Funding acquisition, Resources, Project administration

- Yande Li: Resources, Investigation

- Yuying Gui: Methodology, Formal analysis

- Yufan Ai: Methodology, Formal analysis

- Ogalo Joseph: Writing - review & editing, Methodology, Formal analysis

- Tengxin Cao: Methodology, Formal analysis

- Shilong He: Supervision, Resources, Methodology

- Min Yang: Validation, Supervision, Resources, Project administration, Conceptualization

# Declaration of competing interest

The authors declare that they have no known competing financial interests or personal relationships that could have appeared to influence the work reported in this paper.

# Acknowledgement

This work was financially supported by the National Natural Science Foundation of China (Grant No. W2412156 and 51878649), Shaoxing Yuanshui Group Co., Ltd. (Grant No. TPSK2025008), the China South to North Water Diversion Middle Route Corporation Limited (Grant No. NSBDZX/SH/KY‑2025‑002). and the National Key R&D Program of China (2022YFC3203603).

# Data availability

Data is available at https://git.drwater.net/codes/fang2026controlling.git.

# References

Abbas, A., Park, M., Baek, S.-S., Cho, K.H., 2023. Deep learning-based algorithms for long-term prediction of chlorophyll-a in catchment streams. Journal of Hydrology 626, 130240. https://doi.org/10.1016/j.jhydrol.2023.130240

Acinas, S.G., Haverkamp, T.H.A., Huisman, J., Stal, L.J., 2008. Phenotypic and genetic diversification of Pseudanabaena spp. (cyanobacteria). The ISME Journal 3, 31–46. https://doi.org/10.1038/ismej.2008.78

Ai, Y., Wu, Y., Su, M., Gui, Y., Du, Y., Fang, J., Cao, T., Yang, M., 2025. Cyanobacterial crowding-out effects on metabolite partitioning: Modeling 2-methylisoborneol (MIB) release dynamics and implications. Journal of Hazardous Materials 140118. https://doi.org/10.1016/j.jhazmat.2025.140118

Anantapantula, S., Wittenzeller, S., Gladfelter, M.F., Tenison, S.E., Zinnert, H., Belfiore, A.P., Wilson, A.E., 2025. Copper sulfate treatment harms zooplankton and ultimately promotes algal blooms: A field mesocosm experiment. Harmful Algae 142, 102800. https://doi.org/10.1016/j.hal.2025.102800

AWWA, A.W.W.A., 2010. Algae: Source to treatment. American Water Works Association.

Bellinger, E., Sigee, D., 2015. Freshwater algae: Identification, enumeration and use as bioindicators: Second edition. https://doi.org/10.1002/9781118917152

Benedetti, F., Vogt, M., Elizondo, U.H., Righetti, D., Zimmermann, N.E., Gruber, N., 2021. Major restructuring of marine plankton assemblages under global warming. Nature Communications 12. https://doi.org/10.1038/s41467-021-25385-x

Beversdorf, L.J., Miller, T.R., McMahon, K.D., 2013. The role of nitrogen fixation in cyanobacterial bloom toxicity in a temperate, eutrophic lake. PLoS ONE 8, e56103. https://doi.org/10.1371/journal.pone.0056103

Bormans, M., Maršálek, B., Jančula, D., 2015. Controlling internal phosphorus loading in lakes by physical methods to reduce cyanobacterial blooms: A review. Aquatic Ecology 50, 407–422. https://doi.org/10.1007/s10452-015-9564-x

Broman, E., Sjöstedt, J., Pinhassi, J., Dopson, M., 2017. Shifts in coastal sediment oxygenation cause pronounced changes in microbial community composition and associated metabolism. Microbiome 5. https://doi.org/10.1186/s40168-017-0311-5

Cao, T., Fang, J., Jia, Z., Zhu, Y., Su, M., Zhang, Q., Song, Y., Yu, J., Yang, M., 2023. Early warning of MIB episode based on gene abundance and expression in drinking water reservoirs. Water Research 231, 119667. https://doi.org/https://doi.org/10.1016/j.watres.2023.119667

Carey, C.C., Ibelings, B.W., Hoffmann, E.P., Hamilton, D.P., Brookes, J.D., 2012. Eco-physiological adaptations that favour freshwater cyanobacteria in a changing climate. Water Research 46, 1394–1407. https://doi.org/10.1016/j.watres.2011.12.016

Catherine, Q., Susanna, W., Isidora, E.-S., Mark, H., Aurélie, V., Jean-François, H., 2013. A review of current knowledge on toxic benthic freshwater cyanobacteria – ecology, toxin production and risk management. Water Research 47, 5464–5479. https://doi.org/10.1016/j.watres.2013.06.042

Chen, H., Wang, Z., Liu, H., Nie, Y., Zhu, Y., Jia, Q., Ding, G., Ye, J., 2021. Variable sediment methane production in response to different source-associated sewer sediment types and hydrological patterns: Role of the sediment microbiome. Water Research 190, 116670. https://doi.org/10.1016/j.watres.2020.116670

Dai, Y., Yang, S., Zhao, D., Hu, C., Xu, W., Anderson, D.M., Li, Y., Song, X.-P., Boyce, D.G., Gibson, L., Zheng, C., Feng, L., 2023. Coastal phytoplankton blooms expand and intensify in the 21st century. Nature 615, 280–284. https://doi.org/10.1038/s41586-023-05760-y

Dixon, P., 2003. VEGAN, a package of r functions for community ecology. Journal of Vegetation Science 14, 927–930. https://doi.org/10.1111/j.1654-1103.2003.tb02228.x

Dixon, R., Kahn, D., 2004. Genetic regulation of biological nitrogen fixation. Nature Reviews Microbiology 2, 621–631. https://doi.org/10.1038/nrmicro954

Fang, J., Li, Y., Su, M., Cao, T., Sun, X., Ai, Y., Qin, J., Yu, J., Yang, M., 2024. Mitigating harmful cyanobacterial blooms in drinking water reservoirs through in-situ sediment resuspension. Water Research 267, 122509. https://doi.org/10.1016/j.watres.2024.122509

Fastner, J., Abella, S., Litt, A., Morabito, G., Vörös, L., Pálffy, K., Straile, D., Kümmerlin, R., Matthews, D., Phillips, M.G., Chorus, I., 2015. Combating cyanobacterial proliferation by avoiding or treating inflows with high p load experiences from eight case studies. Aquatic Ecology 50, 367–383. https://doi.org/10.1007/s10452-015-9558-8

Feuring, A., Lawrence, C.D., Salcedo, J., Whitehouse, M.J., Vogts, A., Zoccarato, L., Klawonn, I., 2026. Fungal parasites infecting N2-fixing cyanobacteria reshape carbon and N2 fixation and trophic transfer. Nature Communications 17. https://doi.org/10.1038/s41467-025-67818-x

Fiore, M. de F., Neilan, B.A., Copp, J.N., Rodrigues, J.L.M., Tsai, S.M., Lee, H., Trevors, J.T., 2005. Characterization of nitrogen-fixing cyanobacteria in the brazilian amazon floodplain. Water Research 39, 5017–5026. https://doi.org/10.1016/j.watres.2005.10.002

Flores, E., Herrero, A., 2009. Compartmentalized function through cell differentiation in filamentous cyanobacteria. Nature Reviews Microbiology 8, 39–50. https://doi.org/10.1038/nrmicro2242

Fu, L., Liu, Y., Wang, M., Lian, C., Cao, L., Wang, W., Sun, Y., Wang, N., Li, C., 2024. The diversification and potential function of microbiome in sediment-water interface of methane seeps in south china sea. Frontiers in Microbiology 15. https://doi.org/10.3389/fmicb.2024.1287147

Gaget, V., Humpage, A.R., Huang, Q., Monis, P., Brookes, J.D., 2017. Benthic cyanobacteria: A source of cylindrospermopsin and microcystin in Australian drinking water reservoirs. Water Research 124, 454–464. https://doi.org/10.1016/j.watres.2017.07.073

Gao, G., Bai, D., Li, T., Li, J., Jia, Y., Li, J., Wang, Z., Cao, X., Song, L., 2025. Understanding filamentous cyanobacteria and their adaptive niches in lake honghu, a shallow eutrophic lake. Journal of Environmental Sciences 152, 219–234. https://doi.org/10.1016/j.jes.2024.05.010

Halstvedt, C.B., Rohrlack, T., Andersen, T., Skulberg, O., Edvardsen, B., 2007. Seasonal dynamics and depth distribution of Planktothrix spp. In Lake Steinsfjorden (Norway) related to environmental factors. Journal of Plankton Research 29, 471–482. https://doi.org/10.1093/plankt/fbm036

Havens, K.E., 1991. Fish-induced sediment resuspension: Effects on phytoplankton biomass and community structure in a shallow hypereutrophic lake. Journal of Plankton Research 13, 1163–1176. https://doi.org/10.1093/plankt/13.6.1163

Henson, S.A., Cael, B.B., Allen, S.R., Dutkiewicz, S., 2021. Future phytoplankton diversity in a changing climate. Nature Communications 12. https://doi.org/10.1038/s41467-021-25699-w

Hesse, K., Dittmann, E., BÃrner, T., 2001. Consequences of impaired microcystin production for light-dependent growth and pigmentation of Microcystis aeruginosa PCC 7806. FEMS Microbiology Ecology 37, 39–43. https://doi.org/10.1111/j.1574-6941.2001.tb00851.x

Hipsey, M.R., Bruce, L.C., Boon, C., Busch, B., Carey, C.C., Hamilton, D.P., Hanson, P.C., Read, J.S., Sousa, E. de, Weber, M., Winslow, L.A., 2019. A general lake model (GLM 3.0) for linking with high-frequency sensor data from the global lake ecological observatory network (GLEON). Geoscientific Model Development 12, 473–523. https://doi.org/10.5194/gmd-12-473-2019

Hitzfeld, B.C., Höger, S.J., Dietrich, D.R., 2000. Cyanobacterial toxins: Removal during drinking water treatment, and human risk assessment. Environmental Health Perspectives 108, 113–122. https://doi.org/10.1289/ehp.00108s1113

Huisman, J., Codd, G.A., Paerl, H.W., Ibelings, B.W., Verspagen, J.M.H., Visser, P.M., 2018. Cyanobacterial blooms. Nature Reviews Microbiology 16, 471–483. https://doi.org/10.1038/s41579-018-0040-1

Huo, D., Gan, N., Geng, R., Cao, Q., Song, L., Yu, G., Li, R., 2021. Cyanobacterial blooms in china: Diversity, distribution, and cyanotoxins. Harmful Algae 109, 102106. https://doi.org/10.1016/j.hal.2021.102106

Ibelings, B.W., Fastner, J., Bormans, M., Visser, P.M., 2016. Cyanobacterial blooms. Ecology, prevention, mitigation and control: Editorial to a CYANOCOST special issue. Aquatic Ecology 50, 327–331. https://doi.org/10.1007/s10452-016-9595-y

Ingrid, C., Jamie, B., 1999. Toxic cyanobacteria in water: A guide to their public health consequences, monitoring, and management. E & FN Spon, London New York.

Izaguirre, G., Taylor, W.D., 1998. A Pseudanabaena species from Castaic Lake, California, that produces 2-methylisoborneol. Water Research 32, 1673–1677. https://doi.org/10.1016/s0043-1354(97)00379-5

Jia, Z., Su, M., Liu, T., Guo, Q., Wang, Q., Burch, M., Yu, J., Yang, M., 2019. Light as a possible regulator of MIB-producing Planktothrix in source water reservoir, mechanism and in-situ verification. Harmful Algae 88, 101658. https://doi.org/10.1016/j.hal.2019.101658

Kara, A.B., Rochford, P.A., Hurlburt, H.E., 2000. An optimal definition for ocean mixed layer depth. Journal of Geophysical Research: Oceans 105, 16803–16821. https://doi.org/10.1029/2000jc900072

Karlberg, M., Wulff, A., 2012. Impact of temperature and species interaction on filamentous cyanobacteria may be more important than salinity and increased pCO2 levels. Marine Biology 160, 2063–2072. https://doi.org/10.1007/s00227-012-2078-3

Kibuye, F.A., Zamyadi, A., Wert, E.C., 2021a. A critical review on operation and performance of source water control strategies for cyanobacterial blooms: Part i-chemical control methods. Harmful Algae 109, 102099. https://doi.org/10.1016/j.hal.2021.102099

Kibuye, F.A., Zamyadi, A., Wert, E.C., 2021b. A critical review on operation and performance of source water control strategies for cyanobacterial blooms: Part II-mechanical and biological control methods. Harmful Algae 109, 102119. https://doi.org/10.1016/j.hal.2021.102119

Kim, K., Mun, H., Shin, H., Park, S., Yu, C., Lee, J., Yoon, Y., Chung, H., Yun, H., Lee, K., Jeong, G., Oh, J., Lee, I., Lee, H., Kang, T., Ryu, H.S., Park, J., Shin, Y., Rhew, D., 2020. Nitrogen stimulates Microcystis-dominated blooms more than phosphorus in river conditions that favor non-nitrogen-fixing genera. Environmental Science &amp; Technology 54, 7185–7193. https://doi.org/10.1021/acs.est.9b07528

Kirk, J.T.O., 2010. Light and photosynthesis in aquatic ecosystems. Cambridge University Press. https://doi.org/10.1017/cbo9781139168212

Komarek, J., Kastovsky, J., Mares, J., Johansen, J.R., 2014. Taxonomic classification of cyanoprokaryotes (cyanobacterial genera) 2014, using a polyphasic approach. Preslia 86, 295–335.

Komárek, J., Anagnostidis, K., 1998. Cyanoprokaryota 1, Susswasserflora Von Mitteleuropa. Deu.

Komárek, J., Johansen, J.R., 2015. Chapter 4 - filamentous cyanobacteria, in: Freshwater Algae of North America (Second Edition). Academic Press, Boston, pp. 135–235. https://doi.org/10.1016/b978-0-12-385876-4.00004-9

Lee, K., Chantrasakdakul, P., Kim, D., Kong, M., Park, K.Y., 2014. Ultrasound pretreatment of filamentous algal biomass for enhanced biogas production. Waste Management 34, 1035–1040. https://doi.org/10.1016/j.wasman.2013.10.012

Lei, L., Peng, L., Huang, X., Han, B.-P., 2014. Occurrence and dominance of Cylindrospermopsis raciborskii and dissolved cylindrospermopsin in urban reservoirs used for drinking water supply, south china. Environmental Monitoring and Assessment 186, 3079–3090. https://doi.org/10.1007/s10661-013-3602-8

Liu, Y., Zhu, G., Fan, Y., Guo, C., 2024. Successional conditions of Dolichospermum and Microcystis in taihu lake, china. Journal of Oceanology and Limnology 42, 1777–1788. https://doi.org/10.1007/s00343-024-4063-3

Liu, Z., Xie, F., Wang, L., Yu, L., Shi, X., Yang, Z., Zhang, M., 2024. Response of heterocyst differentiation of dolichospermum to different forms of nitrogen deficiency. Harmful Algae 139, 102742. https://doi.org/10.1016/j.hal.2024.102742

Lu, J., Su, M., Su, Y., Fang, J., Burch, M., Cao, T., Wu, B., Yu, J., Yang, M., 2023. MIB-derived odor management based upon hydraulic regulation in small drinking water reservoirs: Principle and application. Water Research 244, 120485. https://doi.org/10.1016/j.watres.2023.120485

Lu, J., Su, M., Su, Y., Wu, B., Cao, T., Fang, J., Yu, J., Zhang, H., Yang, M., 2022. Driving forces for the growth of MIB-producing Planktothricoides raciborskii in a low-latitude reservoir. Water Research 118670. https://doi.org/10.1016/j.watres.2022.118670

Lu, Z., Cai, Q., Lai, S., Chen, N., Huang, L., Liu, Y., Lei, L., Gan, S., Zhang, L., Paerl, H.W., Wang, F., 2024. Coupling of cylindrospermopsin and pho-harboring verrucomicrobia supports the formation of Raphidiopsis blooms in low-phosphorus waters. Water Research 250, 121010. https://doi.org/10.1016/j.watres.2023.121010

Lucas Pardo, M.A. de, Sarpe, D., Winterwerp, J.C., 2015. Effect of algae on flocculation of suspended bed sediments in a large shallow lake. Consequences for ecology and sediment transport processes. Ocean Dynamics 65, 889–903. https://doi.org/10.1007/s10236-015-0841-y

Mantzouki, E., Visser, P.M., Bormans, M., Ibelings, B.W., 2015. Understanding the key ecological traits of cyanobacteria as a basis for their management and control in changing lakes. Aquatic Ecology 50, 333–350. https://doi.org/10.1007/s10452-015-9526-3

Martinez, M., Chakroff, R., Pantastico, J., 1975. Note: Direct phytoplankton counting techniques, using the haemacytometer. Philippine agriculturist.

Maurer, J.A., Xia, R., Kim, A.M., Oblie, N., Hefferan, S., Xie, H., Slitt, A., Jenkins, B.D., Bertin, M.J., 2024. Temporal dynamics of cyanobacterial bloom community composition and toxin production from urban lakes. ACS ES&amp;T Water 4, 3423–3432. https://doi.org/10.1021/acsestwater.4c00266

Mohamed, Z.A., Elnour, R.O., Alamri, S., Hashem, M., 2023. Paramecium jenningsi effectively grazes on toxic Raphidiopsis raciborskii and degrades cylindrospermopsin: Implications for control harmful cyanobacterial blooms. Ecohydrology & Hydrobiology 23, 614–622. https://doi.org/10.1016/j.ecohyd.2023.04.004

Paerl, H.W., Bland, P.T., Bowles, N.D., Haibach, M.E., 1985. Adaptation to high-intensity, low-wavelength light among surface blooms of the cyanobacterium Microcystis aeruginosa. Applied and Environmental Microbiology 49, 1046–1052. https://doi.org/10.1128/aem.49.5.1046-1052.1985

Paerl, H.W., Huisman, J., 2008. Blooms like it hot. Science 320, 57–58. https://doi.org/10.1126/science.1155398

Paerl, H.W., Otten, T.G., 2015. Duelling “CyanoHABs”: Unravelling the environmental drivers controlling dominance and succession among diazotrophic and non‐N2 ‐fixing harmful cyanobacteria. Environmental Microbiology 18, 316–324. https://doi.org/10.1111/1462-2920.13035

Pawley, S.M., Atkinson, L., Utting, D.J., Hartman, G.M.D., Atkinson, N., 2024. Evaluating spatially enabled machine learning approaches to depth to bedrock mapping, alberta, canada. PLOS ONE 19, e0296881. https://doi.org/10.1371/journal.pone.0296881

Pi, H.-W., Yu, C.-P., Chen, Y.-F., Wu, B.-S., Peng, L.-Y., Li, W.-H., 2025. Nitrogen-fixing microbes gain genes in diverse types of living environments. Proceedings of the National Academy of Sciences 122. https://doi.org/10.1073/pnas.2523106122

Pilotto, L.S., Douglas, R.M., Burch, M.D., Cameron, S., Beers, M., Rouch, G.J., Robinson, P., Kirk, M., Cowie, C.T., Hardiman, S., Moore, C., Attewell, R.G., 1997. Health effects of exposure to cyanobacteria (blue-green algae) during recreational water-related activities. Australian and New Zealand Journal of Public Health 21, 562–566. https://doi.org/10.1111/j.1467-842x.1997.tb01755.x

Posch, T., Köster, O., Salcher, M.M., Pernthaler, J., 2012. Harmful filamentous cyanobacteria favoured by reduced water turnover with lake warming. Nature Climate Change 2, 809. https://doi.org/10.1038/nclimate1581

R Core Team, 2021. R: A language and environment for statistical computing. R Foundation for Statistical Computing, Vienna, Austria.

Raps, S., Wyman, K., Siegelman, H.W., Falkowski, P.G., 1983. Adaptation of the cyanobacterium Microcystis aeruginosa to light intensity. Plant Physiology 72, 829–832. https://doi.org/10/fnbmfx

Read, J.S., Hamilton, D.P., Jones, I.D., Muraoka, K., Winslow, L.A., Kroiss, R., Wu, C.H., Gaiser, E., 2011. Derivation of lake mixing and stratification indices from high-resolution lake buoy data. Environmental Modelling &amp; Software 26, 1325–1336. https://doi.org/10.1016/j.envsoft.2011.05.006

Read, J.S., Winslow, L.A., Hansen, G.J.A., Van Den Hoek, J., Hanson, P.C., Bruce, L.C., Markfort, C.D., 2014. Simulating 2368 temperate lakes reveals weak coherence in stratification phenology. Ecological Modelling 291, 142–150. https://doi.org/10.1016/j.ecolmodel.2014.07.029

Scoglio, G.D., Jackson, H.O., Purton, S., 2024. The commercial potential of aphanizomenon flos-aquae, a nitrogen-fixing edible cyanobacterium. Journal of Applied Phycology 36, 1593–1617. https://doi.org/10.1007/s10811-024-03214-0

Scott, J.T., Marcarelli, A.M., 2012. Cyanobacteria in freshwater benthic environments, in: Ecology of Cyanobacteria Ii. Springer Netherlands, pp. 271–289. https://doi.org/10.1007/978-94-007-3855-3_9

Sherr, E.B., Sherr, B.F., 1993. Preservation and storage of samples for enumeration of heterotrophic protists. Handbook of Methods in Aquatic Microbial Ecology. Lewis Publishers, Boca Raton 207–212.

Sinha, R., Pearson, L.A., Davis, T.W., Burford, M.A., Orr, P.T., Neilan, B.A., 2012. Increased incidence of Cylindrospermopsis raciborskii in temperate zones – is climate change responsible? Water Research 46, 1408–1419. https://doi.org/10.1016/j.watres.2011.12.019

Song, Y., Zhang, B., Si, M., Chen, Z., Geng, J., Liang, F., Xi, M., Liu, X., Wang, R., 2023. Roles of extracellular polymeric substances on microcystis aeruginosa exposed to different sizes of polystyrene microplastics. Chemosphere 312, 137225. https://doi.org/10.1016/j.chemosphere.2022.137225

Su, M., Andersen, T., Burch, M., Jia, Z., An, W., Yu, J., Yang, M., 2019. Succession and interaction of surface and subsurface cyanobacterial blooms in oligotrophic/mesotrophic reservoirs: A case study in Miyun Reservoir. Science of the Total Environment 649, 1553–1562. https://doi.org/J.scitotenv.2018.08.307

Su, M., Jia, D., Yu, J., Vogt, R.D., Wang, J., An, W., Yang, M., 2017. Reducing production of taste and odor by deep-living cyanobacteria in drinking water reservoirs by regulation of water level. Science of the Total Environment 574, 1477–1483. https://doi.org/10.1016/j.scitotenv.2016.08.134

Su, M., Li, W., Fang, J., Cao, T., Ai, Y., Lü, C., Zhao, J., Yang, Z., Yang, M., 2025. Effects of oxygenation resuspension on DOM composition and its role in reducing dissolved manganese in drinking water reservoirs. Environmental Science &amp; Technology. https://doi.org/10.1021/acs.est.5c00235

Su, M., Suruzzaman, MD., Zhu, Y., Lu, J., Yu, J., Zhang, Y., Yang, M., 2021. Ecological niche and in-situ control of MIB producers in source water. Journal of Environmental Sciences 110, 119–128. https://doi.org/10.1016/j.jes.2021.03.026

Su, M., Yu, J., Zhang, J., Chen, H., An, W., Vogt, R.D., Andersen, T., Jia, D., Wang, J., Yang, M., 2015. MIB-producing cyanobacteria (Planktothrix sp.) in a drinking water reservoir: Distribution and odor producing potential. Water Research 68, 444–453. https://doi.org/10.1016/j.watres.2014.09.038

Su, M., Zhu, Y., Andersen, T., Wang, X., Yu, Z., Lu, J., Song, Y., Cao, T., Yu, J., Zhang, Y., Yang, M., 2022. Light-dominated selection shaping filamentous cyanobacterial assemblages drives odor problem in a drinking water reservoir. npj Clean Water 5, 37. https://doi.org/10.1038/s41545-022-00181-2

Tamulonis, C., Postma, M., Kaandorp, J., 2011. Modeling filamentous cyanobacteria reveals the advantages of long and fast trichomes for optimizing light exposure. PLoS ONE 6, e22084. https://doi.org/10.1371/journal.pone.0022084

Tatters, A.O., Clevenger, C., Strangman, W.K., Oehrle, S., Kudela, R.M., Aukamp, J., Wan, Y., 2025. Benthic cyanobacterial accumulations and associated cyanotoxins in coastal urban stormwater pond networks. Harmful Algae 144, 102833. https://doi.org/10.1016/j.hal.2025.102833

Thomas, M.K., Litchman, E., 2015. Effects of temperature and nitrogen availability on the growth of invasive and native cyanobacteria. Hydrobiologia 763, 357–369. https://doi.org/10.1007/s10750-015-2390-2

Tromas, N., Fortin, N., Bedrani, L., Terrat, Y., Cardoso, P., Bird, D., Greer, C.W., Shapiro, B.J., 2017. Characterising and predicting cyanobacterial blooms in an 8-year amplicon sequencing time course. The ISME Journal 11, 1746–1763. https://doi.org/10.1038/ismej.2017.58

Tsai, K.-P., 2016. Management of target algae by using copper-based algaecides: Effects of algal cell density and sensitivity to copper. Water, Air, & Soil Pollution 227. https://doi.org/10.1007/s11270-016-2926-8

Wahlin, K., Grimvall, A., 2008. Uncertainty in water quality data and its implications for trend detection: Lessons from swedish environmental data. Environmental Science &amp; Policy 11, 115–124. https://doi.org/10.1016/j.envsci.2007.12.001

Wang, Z., Xu, Y., Shao, J., Wang, J., Li, R., 2011. Genes associated with 2-methylisoborneol biosynthesis in cyanobacteria: Isolation, characterization, and expression in response to light. Plos One 6, E18665. https://doi.org/10.1371/journal.pone.0018665

Watson, S.B., 2003. Cyanobacterial and eukaryotic algal odour compounds: Signals or by-products? A review of their biological activity. Phycologia 42, 332–350. https://doi.org/10.2216/i0031-8884-42-4-332.1

Wickham, H., Averick, M., Bryan, J., Chang, W., McGowan, L., François, R., Grolemund, G., Hayes, A., Henry, L., Hester, J., Kuhn, M., Pedersen, T., Miller, E., Bache, S., Müller, K., Ooms, J., Robinson, D., Seidel, D., Spinu, V., Takahashi, K., Vaughan, D., Wilke, C., Woo, K., Yutani, H., 2019. Welcome to the tidyverse. Journal of Open Source Software 4, 1686. https://doi.org/10.21105/joss.01686

Wiedner, C., Visser, P.M., Fastner, J., Metcalf, J.S., Codd, G.A., Mur, L.R., 2003. Effects of light on the microcystin content of Microcystis strain PCC 7806. Applied and Environmental Microbiology 69, 1475–1481. https://doi.org/10.1128/aem.69.3.1475-1481.2003

Winder, M., Sommer, U., 2012. Phytoplankton response to a changing climate. Hydrobiologia 698, 5–16. https://doi.org/10.1007/s10750-012-1149-2

Wörmer, L., Huerta-Fontela, M., Cirés, S., Carrasco, D., Quesada, A., 2010. Natural photodegradation of the cyanobacterial toxins microcystin and cylindrospermopsin. Environmental Science & Technology 44, 3002–3007. https://doi.org/10.1021/es9036012

Xiao, M., Wei, Y., Zheng, D., Shi, L., Huang, C., Wang, Y., Zhu, G., 2024. Outbreak of 2-methylisoborneol in a drinking water reservoir attributed to the cyanobacterium pseudanabaena sp. Journal of Oceanology and Limnology 42, 1789–1804. https://doi.org/10.1007/s00343-024-4034-8

Xu, H., Brookes, J., Hobson, P., Pei, H., 2019. Impact of copper sulphate, potassium permanganate, and hydrogen peroxide on Pseudanabaena galeata cell integrity, release and degradation of 2-methylisoborneol. Water Research 157, 64–73. https://doi.org/10.1016/j.watres.2019.03.082

Xu, H., Yang, A., Ma, X., Wang, W., Pang, Y., Pei, H., 2024. Molecular mechanisms underlying sodium percarbonate treatment suppress the recovery and growth of Pseudanabaena sp. In early spring. Journal of Hazardous Materials 476, 135145. https://doi.org/10.1016/j.jhazmat.2024.135145

Yahel, G., Yahel, R., Katz, T., Lazar, B., Herut, B., Tunnicliffe, V., 2008. Fish activity: A major mechanism for sediment resuspension and organic matter remineralization in coastal marine sediments. Marine Ecology Progress Series 372, 195–209. https://doi.org/10.3354/meps07688

Yang, Y., Zheng, X., Tang, Q., Gu, J., Lei, L., Han, B.-P., 2020. Species diversity and seasonal dynamics of filamentous cyanobacteria in urban reservoirs for drinking water supply in tropical china. Ecotoxicology 29, 780–789. https://doi.org/10.1007/s10646-020-02189-1

Yu, Z., Song, X., Cao, X., Liu, Y., 2017. Mitigation of harmful algal blooms using modified clays: Theory, mechanisms, and applications. Harmful Algae 69, 48–64. https://doi.org/10.1016/j.hal.2017.09.004

Zhang, M., Yang, Z., Yu, Y., Shi, X., 2020. Interannual and seasonal shift between microcystis and dolichospermum: A 7-year investigation in lake chaohu, china. Water 12, 1978. https://doi.org/10.3390/w12071978

Zhi, M., Zhao, Y., Zeng, X., Maddela, N.R., Xiao, Y., Chen, Y., Prasad, R., Zhou, Z., 2023. Filamentous cyanobacteria and hydrophobic protein in extracellular polymeric substances facilitate algae—bacteria aggregation during partial nitrification. International Journal of Biological Macromolecules 251, 126379. https://doi.org/10.1016/j.ijbiomac.2023.126379

Zohary, T., Padisák, J., Naselli-flores, L., 2010. Phytoplankton in the physical environment: Beyond nutrients, at the end, there is some light. Hydrobiologia 639, 261–269. https://doi.org/10.1007/s10750-009-0032-2