# Integrating littoral zone odor dynamics into early warning and intervention systems for drinking water reservoirs.

Yuying Guib,a,#, Tengxin Caob,c,#, Ming Su ID,b,c,*, Yufan Aib,c, Yingjie Lib,c, and Min Yangb,c

# These authors contributed equally to this work.

a College of Environmental Science and Engineering, Ocean University of China, Qingdao 266100, China.
b Key Laboratory of Environmental Aquatic Chemistry, State Key Laboratory of Regional Environment and Sustainability, Research Center for Eco-Environmental Sciences, Chinese Academy of Sciences, Beijing 100085, China.
c University of Chinese Academy of Sciences, Beijing 100049, China.

* Corresponding to: Ming Su (mingsu@rcees.ac.cn)

# Abstract

This is the abstract.

# Introduction

The increasing frequency and intensity of harmful cyanobacterial blooms (CyanoHABs) in surface waters, driven by climate change and intensified anthropogenic pressures, pose a critical and escalating global threat to water security (Feng et al., 2021; Heisler et al., 2008; Huisman et al., 2018; O’neil et al., 2012; Paerl et al., 2001; Paerl and Paul, 2012; Woolway et al., 2020). While substantial research has focused on eutrophication-driven bloom mechanisms and risk assessment in shallow lakes (Kosten et al., 2011; Martinez i Quer et al., 2024; Paerl et al., 2011; Reinl et al., 2022; Rousso et al., 2020; Xu et al., 2009; Zhang et al., 2023), studies on reservoir systems—which constitute a primary source of global drinking water—remain disproportionately scarce. This significant knowledge gap carries critical implications, as CyanoHABs directly threaten drinking water safety through the production of toxins (Carmichael et al., 2001; LEFLAIVE and TEN‐HAGE, 2007; Merel et al., 2013; Plaas and Paerl, 2020) and taste-and-odor (T&O) compounds (Burlingame et al., 2017; Lin et al., 2018; Su et al., 2015; Yang et al., 2008), impacting both human health and water utility. For example, nearly half of China’s source-water reservoirs experience recurrent algal-derived T&O episodes (Sun et al., 2014). Unlike chemical contaminants, cyanobacterial populations can undergo exponential growth under suitable conditions (Asato, 2003; Mehnert et al., 2010), rapidly releasing metabolites (Ai et al., 2025; LEFLAIVE and TEN‐HAGE, 2007; Singh et al., 2005) that often exceed the response capacity of conventional water treatment facilities (Cerón-Vivas et al., 2022; Mustapha et al., 2021; Srinivasan and Sorial, 2011), leading to widespread drinking water quality incidents (Li et al., 2010; Yang et al., 2008).

Effectively managing CyanoHABs in reservoirs presents distinct and formidable challenges. Compared to natural lakes, reservoirs display greater spatial and hydraulic complexity, resulting in less stable and predictable ecological regimes (Dordoni et al., 2023; Thornton, 1984). Conventional reservoir monitoring strategies, typically confined to sampling near dam intakes, fail to capture the spatially heterogeneous distribution of algal biomass and metabolites. Furthermore, the short flow time from intake to drinking water treatment plant—often on the scale of minutes to hours—provides an inadequate operational window for timely intervention. In contrast, littoral zones in reservoirs are characterized by markedly different environmental conditions, such as enhanced nutrient availability and light penetration (Cefalì et al., 2016; Zohary and Gasith, 2014), which may promote early cyanobacterial proliferation. Despite their potential significance as critical sentinel sites, these areas remain critically under-monitored. Consequently, the mechanistic linkages between littoral zone CyanoHAB dynamics and downstream water quality at intakes are poorly understood and quantified.

Geosmin (GSM) and 2-methylisoborneol (MIB) are among the most concerning terpenoid T&O compounds produced by cyanobacteria (Jüttner and Watson, 2007; Watson, 2003). Their low molecular weight and chemical nature allow them to penetrate conventional water treatment barriers and persist into tap water (Lin et al., 2018; Zamyadi et al., 2015), directly impairing drinking water quality and consumer acceptance. In China, MIB-related episodes have been widely reported and studied, with corresponding mitigation strategies developed (Su et al., 2021, 2015). In contrast, GSM incidents have historically been more prevalent in the Southern Hemisphere (Bowmer et al., 1992; Uwins et al., 2007) but are increasingly reported across Northern Hemisphere water bodies (Qiu et al., 2023; Wu et al., 2022). Notably, GSM-producing cyanobacterial taxa are phylogenetically more diverse than those associated with MIB (AWWA, 2010), complicating source-tracking and management efforts. To address these emerging challenges, we conducted a multi-year, spatially resolved monitoring study in Yuqiao Reservoir—a major drinking water source for Tianjin, China, with a documented history of GSM episodes. Integrating water quality analysis, molecular sequencing of algal communities, and advanced statistical modeling, we demonstrate significant spatial decoupling of cyanobacterial assemblages and T&O compounds between littoral zones and the dam area. We further identify key odor-producing taxa and establish that littoral zone cyanobacterial dynamics exert a decisive influence on T&O levels at the intake. These findings position the littoral zone as a critical sentinel for early warning and a strategic target for proactive intervention. This study advances a scientifically informed monitoring framework, shifting the paradigm from reactive response to preventive management of CyanoHAB-derived T&O risks in reservoir-based water supplies.

# Materials and methods

## Study area and sampling sites

Yuqiao Reservoir is located in the east of the city of Jizhou district in Tianjin, with a total capacity of 1.559 billion cubic metres, making it one of the key large-scale reservoirs in the country and an important urban water source for Tianjin. The reservoir has been experiencing persistent odour problems in recent years. Depending on the geographical location, a total of eight sampling points were selected in this reservoir to investigate the spatial distribution of geosmin concentrations, namely, east of the reservoir, southeast of the reservoir, south of the reservoir, centre of the reservoir, west of the reservoir, water discharge hole, north of the reservoir, and Fengshan. Among them, the water depth in the discharge hole could reach 12 metres, where stratified sampling was conducted. The rest of the sites were sampled at depths of 2 to 4 metres and surface water samples were taken. Sampling is carried out during the annual odour period, two to three times per week, with nutrient measurements and algal counts. All samples were stored at 4°C for 24 hours prior to use. Geomin concentrations were determined using solid phase microextraction (SPME) coupled with gas chromatography-mass spectrometry (GC-MS). Physicochemical properties such as water temperature, ph, dissolved oxygen (DO) and turbidity were measured in situ using a multiprobe instrument (EXO2, Xylem, USA). Ammonia (NH 4-N), total nitrogen (TN), and total phosphorus (TP) were measured according to national water quality standards. Samples for algal counts were preserved in 5% luger iodine, concentrated to 10 mL after 48 h of sedimentation, and cell counts were carried out under a microscope (OLYMPUS BX51, Olympus Optical, Tokyo,Japan) using cell counting plates (S52, 1 mL, Sedgewick-Rafter).

## Quantification of geoA gene

Quantitative PCR was employed to determine the absolute abundance of geoA genes,which encode the geosmin syntheses.The amplification of mic genes was carried out using the 173AF(5’-TGTGAGTACCCAAGAGG-3’) and 173AR(5’-CTGCCAATCCTGAAGTCCTTT-3’) primers.The quantitative PCR assays were performed on a LightCycler 96 instrument(Roche, USA) with a 25μL reaction mixture comprising 12.5μLTBGreen®Premix ExTaq™(Takara, Japan), 0.8μL of each primer (173AF and 173AR), 5.9μL deionized water, and 5μL template DNA.

The reaction conditions were as follows: initial pre-incubation at 95°C for 10 min, followed by 35 cycles at 95°C for 20 s, 50°C for 20 s, and 72°C for 20 s. A subsequent DNA melting step ranging from 65°C to 97°C was included.

Absolute abundance was determined based on a standard curve using the formula: Cq = -3.6261lg(cgeoA)+37.70(R2=0.999).All samples were quantified in triplicate, and a negative control was implemented to mitigate the risk of nonspecific amplification.

## epicPCR of geoA-16S rRNA Gene Fusion Fragments

epicPCR combines established cell isolation, encapsulation and paired amplification methods. The method is outlined as follows: an initial aqueous oil-in-oil sample emulsion produces approximately 500 million droplets, each with a volume of approximately 1 nanolitre, containing single cells. These cells are loaded and dispersed such that, on average, less than one out of every 100 droplets contains cells. Each of these droplets also contains acrylamide monomers, which encapsulate the cells when a catalyst is added, forming polyacrylamide beads. The polyacrylamide beads provide protection for the bacterial chromosomes and plasmids, preventing them from spreading when used for fusion PCR. Polyacrylamide beads are typically 5 - 30μm in diameter, with most beads being approximately 10μm in diameter, and fusion PCR is performed on hydrogel-captured genomes in a secondary emulsion to ensure that each epicPCR is compartmentalised and performed as a single reaction. （在 2023 年水库 geosmin 爆发期间采集水样进行 GC-MS 检测，结果表明 11 月的样品中采集自 YQ05 的水样、12 月的样品中采集自 YQ03 的水样有着高于同月份其他点位的 tgsm，浓度分别达到了 28.43ng/L 和 29.84ng/L.由于产嗅蓝藻通常在水体中的相对丰度较低，所以用 1.2um 滤膜富集这两个点位的水样以提高产嗅蓝藻的比例。用 Fast DNA试剂盒提取富集在滤膜上的藻类 DNA 进行了 Quantitative PCR 扩增，结果表明这两个点位的 geoA gene 浓度均达到了 10^5copies/L.对这两个点位的水样制备细胞悬浮液，进行 epicPCR 扩增，对巢氏 PCR 扩增产物进行 1% 琼脂糖凝胶电泳，结果如图 x-e 所示。）

## Preparation of Polyacrylamide Gel Beads Containing Exposed Genomes:

Sample Collection and Filtration: Water samples were filtered using 1.2μm Isopore™Membrane Filters, and microbial cells within the filters were washed using normal saline(0.85%). 制备环境水样的细胞凝珠：取上述水样、丙烯酰胺溶液和过硫酸铵溶液混匀后加入 STT 乳化油；接着再加入四甲基乙二胺，混匀，静置状态下，进行丙烯酰胺聚合反应，制成包裹水体中藻类细胞的聚丙烯酰胺-细胞凝珠混合液；对聚丙烯酰胺-细胞凝珠混合液依次进行水饱和乙醚清洗、超纯水洗涤后向聚丙烯酰胺-细胞凝珠加入 1×TK 缓冲液，形成聚丙烯酰胺-细胞凝珠 TK 悬液，然后采用细胞过滤器进行过滤，制得纯化聚丙烯酰胺-细胞凝珠 TK 悬液；向纯化聚丙烯酰胺-细胞凝珠 TK 悬液中添加溶菌酶，溶解藻类细胞细胞壁后加入蛋白酶 K 和 TritonX-100，酶解去除与核酸结合的组蛋白，使藻类细胞 DNA 游离，以及破坏藻类细胞细胞膜，使得藻类细胞基因组暴露在聚丙烯酰胺凝珠壳内，获得含有暴露基因组的聚丙烯酰胺-细胞凝珠。 Preparation of environmental water samples of cell beads: take the above water samples, acrylamide solution and ammonium persulfate solution mixed well, then add STT emulsified oil; then add tetramethylethylenediamine, mixing, under the state of static, acrylamide polymerisation reaction, made of polyacrylamide-cell beads wrapped in the algae cells in the water body mixture; polyacrylamide-cell beads mixture in order to carry out water-saturated ethyl ether wash, ultrapure water After washing, 1×TK buffer was added to the polyacrylamide-cell beads to form a polyacrylamide-cell beads TK suspension, which was then filtered using a cell filter to produce a purified polyacrylamide-cell beads TK suspension; lysozyme was added to the purified polyacrylamide-cell beads TK suspension to lyse algal cell cell walls and then proteinase K and TritonX-100 were added to enzymatically digest it. Removing histones bound to nucleic acids, freeing algal cell DNA, as well as destroying the cell membrane of the algal cell, so that the genome of the algal cell is exposed in the shell of the polyacrylamide beads, and obtaining polyacrylamide-cellular gel beads containing the exposed genome.

# Results

## epic PCR

Functional genes allow the identification of the ecological behaviour of organisms, the gene coding for Geosmin synthase (geoA) can be used to determine whether an organism is capable of producing geosmin or not, whilst highly variable regions of the 16S rRNA gene allow the identification of cyanobacterial species. Thus combining these two parts of the genetic information can identify cyanobacteria that produce geosmin. For this purpose, we screened five gene fusion primers, including fusion PCR, block PCR and nested PCR. primers geo_cya728F (geoA forward primer), 1492R (16S rRNA reverse primer) and 173AR-515F (geoA reverse primer with 16S rRNA forward primer) were used to combine the geoA gene ( 224bp) and 16S rRNA gene (977bp) DNA fragments were fusion PCR ligated to produce the fused fragment (1201bp). This fragment contained both taxonomic and functional information of the cyanobacteria. Subsequently blocking PCR was performed to prevent the influence of the unamplified fragment of the fusion PCR on the nested PCR. Finally, primers 173AF (geoA forward primer) and 806R (16S rRNA reverse primer) were used to shorten the fusion product (465bp) to ensure the amplification efficiency and to reduce the impact of mismatches.

Only if the geoA gene and the 16S rRNA gene in the above process come from the genome of the same organism, can it be proved that its host organism is a positive target. In order to achieve the above PCR process in isolated pure organisms, certain pre-treatment measures were taken, where the environmental samples were thoroughly diluted and mixed with polyacrylamide, and each fine polyacrylamide gel bead contained only one cell or filament. proteinase K was also added to promote efficient cleavage of the cyanobacterial cell wall due to the presence of a cell wall in cyanobacterial cells.

Once single-cell/organism gels are generated within each gel and the PCR process is completed, they can be mixed and subjected to high-throughput sequencing to identify geosmin-producing olfactory algal species based on the 16S rRNA gene sequences trimmed from the epicPCR amplification results.

## National investigation

近年来，全国 geosmin 嗅味问题频繁被报道，geosmin 问题可以发生在河流、水库、湖泊等各种水源水体中。随着近年水体富营养化治理的投入，全国水体的富营养化问题得到了一定程度的缓解，但是有关嗅味问题的报道却不减反增。2023 年我们对中国近 30 个湖泊和水库进行了调研，结果表明从全国尺度来看，geosmin 浓度整体呈现东南高西北低的特点，检出 geosmin 浓度超过国家标准(10ng/L)的湖泊和水库主要有长桥海水库、洱海、巢湖、青草沙水库、太湖、于桥水库等;尽管咸水中曾经报道过 geosmin 问题，但是在我们的调研中，西藏的四大咸水湖中均未检出 geosmin，青海的多个咸水湖泊中也仅有小于 2.5ng/L 的微量检出。

我们的调查发现，不同湖泊和水库的 geosmin 嗅味爆发期不同，且 geosmin 事件多为阶段性发生。调查中 geosmin 浓度超标的湖泊和水库都是富营养或者曾经是富营养的水体，嗅味爆发期主要是每年的春夏高温季节，他们主要在中国的东部和南部靠近海洋的地区，这些地区处于中国大河流域的下游，气候湿热，适合产嗅蓝藻的生长。

例外地，于桥水库位于中国北方的天津市，于桥水库的 geosmin 爆发期为每年的 6 月到次年的 1 月，不同于大部分湖泊和水库的爆发期为春夏季节，于桥水库的嗅味爆发期长且爆发浓度高，并且在寒冷的冬天仍然能够保持超过 10ng/L 的 geosmin 浓度。这里 1 月的平均气温低于 0℃，平均水温仅为 4℃左右，他人的研究表明，这样的水温不适宜产嗅藻的生长与产嗅；也有研究表明结冰封冻后的水库中也有藻类可以缓慢繁殖，这些藻类释放的嗅味物质在封冻条件下难以挥发，导致较高的嗅味浓度。

## Geosmin problem in Yuqiao Reservoir

于桥水库的 YQ01 至 YQ08 八个点位采集原水样本检测 geosmin。根据历史数据显示Fig. 1，自 2019 年以来，于桥水库的 geosmin 均为季节性发生，通常持续长达 8 个月，发生时间为每年的 6 月到次年的 1 月。于桥水库的 geosmin 每年会经历两次峰值，第一次峰值在 6 到 8 月，从 2019 年至 2023 年平均值分别为 16.9，8.5，10.3，12.8，9.2ng/L，90% 分位数值为 20.5，12.3，16.2，15.8，18.8ng/L；第二次峰值在 10 到 12 月，从 2019 至 2023 年平均值分别为 22.0,15.1,17.6,15.6,21.2ng/L，90% 分位数值为 24.4，21.2,25.1,22.9,29.8ng/L，第一次峰值均低于第二次峰值。从年际分布来看，在 2020 年至 2023 年的 geosmin 发生期间，geosmin 平均浓度分别为 8.33,10.04,11.47,13.25 ng/L ,有逐年增长的趋势(p = 0.001)，嗅味发生期间 geosmin 浓度最高可达 36.4ng/L。在非 geosmin 发生期间，即每年的 2 到 5 月，geosmin 浓度通常在检测限附近（1.9ng/L）。

Fig. 1: Seasonal dynamic of geosmin concentration in Yuqiao Reservoir

## Distribution of geosmin producer

调查期间，共记录到蓝藻 137 属，蓝藻群落表现出明显的时间和空间差异，蓝藻门为全年的优势门，蓝藻门丰度在每年 5 到 10 月有明显峰值，可达 10^8cells/L。于桥水库已知的潜在 geomsin 产嗅藻表现出与 geosmin 浓度高度相似的时间分布特点（p<0.0001）。对巢氏 PCR 扩增产物进行高通量测序，结果表明在这两个点位的 geosmin 产嗅藻均为 planktothrix。

于桥水库的阿氏浮丝藻每年会爆发两次峰值。第一次峰值发生于 5 月末到 6 月初，阿氏浮丝藻从水库上游的 YQ07、YQ06 点位开始迅速生长，生长速率可达 106cells/(L·d)，阿氏浮丝藻密度峰值达到1.1×107cells/L，并维持该密度水平至8月末开始显著降低，降低速率约为-5×105cells/(L·d)，9月中旬开始，阿氏浮丝藻迎来第二次爆发，并于10月中旬达到峰值，第二次爆发的生长速率和密度峰值高于第一次爆发，分别可达2.5×106cells/(L·d)与2×107cells/L，第二次峰值后密度迅速降低，降低速率可达-2.5×106cells/(L·d)。而中游的YQ03、YQ04、YQ05点位的阿氏浮丝藻第一次爆发晚于上游点位一周左右，爆发后密度起伏变化较大，无明显的变化规律，生长速率在-106cells/(L·d)与106cells/(L·d)之间，但总体保持在高密度水平，最高可达1.1×107cells/L，第二次爆发从9月中旬开始，生长速率和密度峰值也明显高于第一次爆发，分别可达3.7×106cells/(L·d)和2×107cells/L，第二次峰值后密度下降速率也更快，降低速率可达-5×106cells/(L·d)。水库下游的YQ01点位的阿氏浮丝藻密度变化规律比中上游点位更为明显，第一次爆发晚于上游点位两周左右，并且在爆发后呈持续上升的趋势(p=0.0339)，三周后到达峰值，生长速率维持在106cells/(L·d)左右，第一次爆发的密度峰值高于中上游点位，可达1.9×107cells/L，随后持续下降，降低速率可达-1.5×106cells/(L·d)，第二次爆发从10月初开始，同样晚于上游两周，爆发后仍然呈持续上升趋势，第二次爆发的藻密度峰值与中上游相同，即2×107cells/L，生长速率约为1.5×106cells/(L·d)，并于11月初回到爆发前的低密度水平(105cells/L)，降低速率可达-2×106cells/(L·d)，此后藻密度则一直保持在爆发前的低浓度水平(105cells/L)。

Fig. 2: heatmap of algae in Yuqiao reservoir and geosmin and plaktothrix by site

## Distribution of geosmin in Yuqiao Reservoir

于桥水库全年会经历两次 geosmin 爆发事件。第一次爆发为每年的 6 到 8 月，geosmin 从上游较浅的点位开始爆发，在 YQ01 和 YQ05 以外的点位，geosmin 爆发之初的 6 月即达到第一次浓度峰，浓度峰值均在 20ng/L 左右，并在爆发后保持持续下降(p<0.001)，于 8 月降至最低，多数点位在 8 月未检出 geosmin。而 YQ01 和 YQ05 点位 geosmin 浓度变化表现出明显的不同，第一次爆发时，这两个点位的 geosmin 浓度从 4 到 8 月呈持续上升的趋势(p=0.002)，在 4、5 月份，其他点位 geosmin 爆发时，这两个点位的 geosmin 仅为接近检测限的 1.9ng/L，直至 8 月分别上升至 7.71 与 9.33ng/L,geosmin 增长速率分别为 1.11ng/(L·month)与 1.28ng/(L·month)。 第二次 geosmin 爆发事件为每年的 10 到 12 月。此时，水库中上游点位的 geosmin 浓度变化规律接近，均在 8 到 10 月持续上升(p<0.001)，从 8 月的未检出 geosmin 上升至 10 月份全年最高峰值 30ng/L 左右，此后 10 到 12 月则呈持续下降趋势(p=0.017)。不同于其他点位，第二次 geosmin 事件中，水库下游的 YQ01 点位 geosmin 浓度从 8 到 12 月一直呈持续上升的趋势(p=0.007)，geosmin 浓度从 8 月的 7.1ng/L 上升至 12 月的 23.4ng/L，geosmin 增长速率分别为 4.63，8.18，0.87，3.55ng/(L·month)，在其他点位 geosmin 达到浓度峰的 10 月，YQ01 的 geosmin 增长速率也达到最快，但该点位的 geosmin 达峰时间为 12 月，晚于其他点位，并且峰值也低于其他点位.

Fig. 3: Spatial distribution of geosmin in Yuqiao Reservoir in 2023-2024

## Time Lag Analysis between geosmin and Planktothrix in Yuqiao Reservoir。

通过 YQ01 与 YQ05 点位近四年的 geosmin 与 planktothrix 相关性分析，评价 planktothrix 和 geosmin 之间的滞后时间关系。在 YQ05 点位，geosmin 与阿氏浮丝藻在无滞后时相关性最强，此时 R2为 0.50Fig. 4A，随着滞后周数的增加，YQ05 点位的 geosmin 与阿氏浮丝藻相关性越来越弱。而在 YQ01 点位，geomin 与阿氏浮丝藻从滞后 0 周到滞后 5 周的相关性逐渐增强，并在滞后 5 周时，两者相关性达到最高，R^2 为 0.41Fig. 4A，随着滞后周数继续增加，两者相关性逐渐减弱，在 YQ01 点位两者相关性最强的一组数据中Fig. 4B，可以看到仍然有较多藻密度较高而 geosmin 浓度较低的数据点. 在中游的 YQ05 点位，阿氏浮丝藻和 geosmin 于每年的 6 月初开始爆发Fig. 4C，阿氏浮丝藻在 12 月开始减少，而 geosmin 仍然能够保持高浓度到 2 月左右降低至爆发前的低浓度水平。在下游的 YQ01 点位Fig. 4D，阿氏浮丝藻则早于 geosmin 5 周左右爆发，冬季藻密度降低后，geosmin 浓度仍能保持一到两个月才开始缓慢降低。

Fig. 4: Time lag between geosmin and Planktothrix in Yuqiao Reservoir

## Migration patterns of geosmin and its olfactory producing algae in Yuqiao Reservoir

Total geosmin 的箭头朝向接近右下方，说明 Total geosmin 主要受到 PC1 和 PC2 的负面共同影响。氨氮的箭头与 Total geosmin 几乎平行，说明从全库水平来看两者相关性非常强。TP、COD 和 Chl.a 与 Total geosmin 的夹角也较小，表明它们之间也存在较强的正相关性，充足的营养盐条件是水体富营养化加剧的标志，促进了产嗅藻的生长和产嗅，geosmin 的生物合成与 Chl.a 的生物合成有一定的相关性。透明度和 DO 与 Total geosmin 完全相反，表明水体透明度越低，DO 越低，geosmin 浓度越高，这是由于藻类爆发引起水体浑浊程度增加，同时耗氧速率增加。 从岸边较浅的点位来看，于桥水库的 geosmin 与氨氮相关性极强，R2为 0.35。岸边浅水区域通常受到外界输入的影响较大，比如地表径流带来的养分，使得氨氮浓度较高，同时这些区域因为水体流速缓慢，光照较强，水体混合程度更高，所以更适合藻类生长和产嗅，导致两者相关性强。水库中游的 YQ05 点位，二者的相关性有所下降，R2为 0.19，而水库下游最深的 YQ01 点位，二者相关性非常低，R2仅为 0.03 这是因为水库深处的环境相对稳定，氨氮可能因为沉降或者硝化过程在水体中减少，水库深处的氨氮更多来自内循环，如沉积物释放或分解，且下游 geosmin 的产生可能更多受限于藻类生长条件，如光照和温度等，从而降低了氨氮和 geosmin 的相关性。

结合于桥水库的地形，进一步分析于桥水库 geosmin 与其产嗅藻的分布规律。岸边地区的水深较浅，特别是水库上游的 YQ07 点位，水深仅有 2 到 3 米，此处接近入库河流，是水库水体的主要来源。每年 5 月底至 6 月初，携带者阿氏浮丝藻的水体从上游较浅的点位输入于桥水库。在浅水区及入库口，水流减速易形成沉积，有利于产嗅藻的富集。浅水区水体混合程度高，入库河流携带的营养盐可以快速混合均匀，输入水库的产嗅蓝藻可以即时适应水体环境，在输入水库的同时开始产嗅。浅水区光照充足，水温高，更容易促进蓝藻生长产嗅。上游的水体流动性强于下游，可能促进 geosmin 的挥发和稀释，所以在嗅味事件末期，geosmin 总是从上游和较浅的岸边开始降低。水库整体流速较缓，上游输入的产嗅藻需要经过两周左右的迁移最终汇于水库下游，水深最深的 YQ01 点位，该处水深可达 10 米。深水区水动力较弱，水体交换时间长，不同深度的水体环境差异较大，所以产嗅藻随水体迁移至下游后，需要一定时间的适应才能继续生长产嗅。此处靠近水库的大坝区域，停留时间长于上游点位，更有利于累积更多产嗅藻和 geosmin，所以阿氏浮丝藻和 geosmin 爆发事件时，YQ01 点位的峰值总是达到全库最高。在水库中游的 YQ05 点位水深大约为 5 米，此处在第一次嗅味事件（6 到 8 月）时为 geosmin 的汇，第二次嗅味事件（10 到 12 月）为 geosmin 的源.

Fig. 5A

Fig. 5: Mechanism of Planktothrix induced risk in Yuqiao Reservoir

# Discussion

geosmin 是饮用水中最常见的致嗅化合物之一,丝状蓝藻和放线菌均为土臭素的生产者，但已有研究表明，在湖库型水体中，geosmin 主要由丝状蓝藻产生(lee2020occurrence?)。中国超过 80% 的地表水源均存在嗅味问题(Sun et al., 2014)，饮用水处理中的 geosmin 难以用传统工艺去除，因此常在饮用水中成为嗅味问题的主要原因。揭示 geosmin 在环境中的产生、分布特征为解决水体嗅味问题提供了科学依据。中国湖库型水源地中，geosmin 问题与富营养化治理成效呈现复杂关联,尽管中国水体富营养化程度有所缓解，但 2023 年调研显示，东南地区 geosmin 超过 10 ng/L国家标准的湖库仍然较多。这一现象可能与营养盐水平变化导致的蓝藻群落演替有关：随着水体富营养化治理，水体营养盐水平整体降低，优势藻种从营养盐限制型的微囊藻向光照限制型产嗅丝状蓝藻（如鱼腥藻、束丝藻）过渡，多种 geosmin 产嗅藻能够在低氮、低磷的条件下通过形成异形胞固定无机氮，通过磷储存机制和胞外碱性磷酸酶利用有机磷，能够在各种生境中广泛存在(wan2019phosphorus?)。因此，geosmin 不仅是水质指标，更是生态系统响应环境压力的重要信号分子。

于桥水库原位监测研究中，geosmin 浓度与其产嗅藻 planktothrix 的相关性整体较低，其 R2最高值仅见于 YQ05 点位的（R²=0.50，p<0.05）Fig. 4B，该结果与多项原位监测研究中产嗅藻生物量与特征嗅味物质浓度间的相关性水平基本一致，表明在复杂水环境条件下藻类生物量与次生代谢产物浓度间可能受到多因素调控(shang2024temporal?)。而实验室纯培养条件下 geosmin 与其产嗅藻相关性极强，R2可达 0.9 以上(SU20133444?)。同时，在纯培养条件下，geosmin 生产者在实验室环境中表现出比原位生长更高的 geosmin 产量(Gerber and Lechevalier, 1965)。导致Geosmin 与其生产者的生物量相关性在两种环境中表现出显著差异的主要原因是纯培养条件下与原位水体环境中的微生物代谢活动不同。在实验室纯培养环境下，geosmin 浓度更多地依赖于特定微生物的基因表达和代谢能力，而在原位水体中，其浓度受到溶解氧、微生物群落动态等多种环境因素影响(Lukassen et al., 2022)。原位水体与纯培养条件下 geosmin 与其生产者生物量的的相关性差异主要源于环境条件和微生物群落结构的不同。因此，原位调查在揭示土臭素产嗅藻的真实生态行为及其环境驱动机制方面具有更重要的研究价值，应注重开展原位调查，以期为水体嗅味问题的精准防控提供科学依据。

## Geosmin in Yuqiao Reservoir

本研究揭示于桥水库的 geosmin 污染事呈现独特的时空分布特征，与我国东部及南部湖泊水库典型嗅味事件相比，该水库土臭素污染具有显著差异：时间分布上，污染事件发生于夏季并持续至次年一月，呈现双峰型浓度特征Fig. 1，而中国东部和南部的湖泊和水库的嗅味污染一般发生在春夏季节，而不发生在低温的冬季，且峰值持续时间较短，呈单峰浓度特征(Cao et al., 2023; shang2024temporal?)。空间分布上，下游监测点 YQ01 的 geosmin 与其产嗅藻 planktothrix 生物量峰值较中上游点位滞后 4-5 周，且浓度衰减同步延迟Fig. 3，该滞后周期可能表征 geosmin 在水体中的停留时间。5-6 月支流来水输入了 planktothrix 同步产生了 geosmin，7-8 月二者在中上游形成首个浓度峰值后，由于浅水区 8-10 月的高光强环境促进了表层藻类水华，间接通过遮光效应抑制了 planktothrix 的生长，导致土臭素浓度阶段性下降。(Oberhaus et al., 2007)。值得注意的是，上游点位浮丝藻生物量与土臭素浓度呈现显著同步变化（r=0.82, p<0.01），表明支流输入时浮丝藻已具备原位产嗅能力。至 10-12 月，环境条件改变促使浮丝藻二次增殖，形成冬季次峰，此现象突破了传统认知中低温环境抑制产嗅的理论。该研究揭示了于桥水库土臭素污染的双峰特征及其水动力驱动机制，为建立流域特异性嗅味预警模型、追踪污染源项及评估污染物迁移扩散规律提供了科学依据。

## The migration process of geosmin in Yuqiao Reservoir

本研究表明随着滞后周数的增加，YQ05 点位的 geosmin 与阿氏浮丝藻相关性越来越弱，这表明在 YQ05 点位阿氏浮丝藻与 geosmin 同时输入，并不存在明显的滞后关系。在 YQ01 点位两者相关性最强的一组数据中Fig. 4B，可以看到仍然有较多藻密度较高而 geosmin 浓度较低的数据点，这表明水库下游的产嗅藻由中上游输入，并于下游生长 5 周左右开始大量产嗅。在中游的 YQ05 点位，阿氏浮丝藻和 geosmin 于每年的 6 月初开始爆发Fig. 4C，阿氏浮丝藻在 12 月开始减少，而 geosmin 仍然能够保持高浓度到 2 月左右开始降低，这是由于冬季于桥水库经历冰封期，冰盖使 geosmin 难以挥发，保留在水体中，这与 Wang 等在画匠水库的研究相符，冬季富营养化是影响 geosmin 合成和释放的主要因素而不是温度(Wang et al., 2015)。在下游的 YQ01 点位，geosmin 增长时间滞后于阿氏浮丝藻 5 周，冬季藻密度降低后，geosmin 仍能维持峰值 4 到 5 周。在美国 Eagle Creek Reservoir 中也观测到 geosmin 与 Planktothrix 紧密相关，且水库中 geosmin 的最高检出值与产嗅藻峰值之间存在 37 天的滞后期，这一滞后期代表产嗅藻从流域迁移到水库所需的时间(Clercin and Druschel, 2019)。这些信息提供了一种预测 geosmin 事件的线索,使得水库可以在异味变得严重之前阻止产嗅藻生长。

## Identification of geosmin-producing algae

目前已有的研究表明，基因组中是否携带 geosmin synthase 是进行 geosmin 产嗅藻识别的关键依据，对功能基因的定量检测可以检出水体中 geosmin 的产生潜力，但是不能识别出 geosmin 产嗅藻的不同物种信息。基于分子生物学的系统发育基因（如 16S rRNA 基因）扩增子测序仅能用于识别藻类群落结构，不能特定地识别出 geosmin 产嗅藻。目前 epicPCR 方法可以高通量精准识别环境中功能基因宿主，并已经成功应用于特异性识别硫酸盐还原菌(Spencer2015massively?)，本研究通过改进 epicPCR 方法，筛选了特异性引物 PCR 完成了对于桥水库 geosmin 产嗅藻的识别，但是该方法还不适用于识别如放线菌等除蓝藻之外的 geosmin producers。 通过 epicPCR 方法我们识别出于桥水库 2023 年的 geosmin producer 是 planktothrix，但 geosmin producer 并不是一直不变的，也可能同时存在多种 geosmin producers，LI 等人的研究表明在 2017 到 2019 年于桥水库的 geosmin 与鱼腥藻具有极显著的正相关性，并且主要嗅味物质从 2017 年的 geosmin 转变为 2018 年后 mib(li2020analysis?)

# Conclusions

# Acknowledgements

This work was financially supported by the National Natural Science Foundation of China (52030002, 51878649), and Youth Innovation Promotion Association CAS.

# References

Ai, Y., Wu, Y., Su, M., Gui, Y., Du, Y., Fang, J., Cao, T., Yang, M., 2025. Cyanobacterial crowding-out effects on metabolite partitioning: Modeling 2-methylisoborneol (MIB) release dynamics and implications. Journal of Hazardous Materials 140118. https://doi.org/https://doi.org/10.1016/j.jhazmat.2025.140118

Asato, Y., 2003. Toward an understanding of cell growth and the cell division cycle of unicellular photoautotrophic cyanobacteria. Cellular and Molecular Life Sciences (CMLS) 60, 663–687. https://doi.org/10.1007/s00018-003-2079-y

AWWA, A.W.W.A., 2010. Algae: Source to treatment. American Water Works Association.

Bowmer, K.H., Padovan, A., Oliver, R.L., Korth, W., Ganf, G.G., 1992. Physiology of geosmin production by Anabaena Circinalis isolated from the murrumbidgee river, australia. Water Science and Technology 25, 259–267. https://doi.org/10.2166/wst.1992.0060

Burlingame, G.A., Doty, R.L., Dietrich, A.M., 2017. Humans as sensors to evaluate drinking water taste and odor: A review. Journal AWWA 109, 13–24. https://doi.org/10.5942/jawwa.2017.109.0118

Cao, T., Fang, J., Jia, Z., Zhu, Y., Su, M., Zhang, Q., Song, Y., Yu, J., Yang, M., 2023. Early warning of MIB episode based on gene abundance and expression in drinking water reservoirs. Water Research 231, 119667. https://doi.org/https://doi.org/10.1016/j.watres.2023.119667

Carmichael, W.W., Azevedo, S.M., An, J.S., Molica, R.J., Jochimsen, E.M., Lau, S., Rinehart, K.L., Shaw, G.R., Eaglesham, G.K., 2001. Human fatalities from cyanobacteria: Chemical and biological evidence for cyanotoxins. Environmental Health Perspectives 109, 663–668. https://doi.org/10.1289/ehp.01109663

Cefalì, M.E., Cebrian, E., Chappuis, E., Pinedo, S., Terradas, M., Mariani, S., Ballesteros, E., 2016. Life on the boundary: Environmental factors as drivers of habitat distribution in the littoral zone. Estuarine, Coastal and Shelf Science 172, 81–92. https://doi.org/10.1016/j.ecss.2016.01.043

Cerón-Vivas, A., Villamizar León, M.P., Cajigas, Á.A., 2022. Geosmin and 2-methylisoborneol removal in drinking water treatment. Water Practice and Technology 18, 159–167. https://doi.org/10.2166/wpt.2022.167

Clercin, N.A., Druschel, G.K., 2019. Influence of environmental factors on the production of MIB and geosmin metabolites by bacteria in a eutrophic reservoir. Water Resources Research 55, 5413–5430. https://doi.org/10.1029/2018wr023651

Dordoni, M., Zappalà, P., Barth, J.A.C., 2023. A preliminary global hydrochemical comparison of lakes and reservoirs. Frontiers in Water 5. https://doi.org/10.3389/frwa.2023.1084050

Feng, L., Dai, Y., Hou, X., Xu, Y., Liu, J., Zheng, C., 2021. Concerns about phytoplankton bloom trends in global lakes. Nature 590, E35–E47. https://doi.org/10.1038/s41586-021-03254-3

Gerber, N.N., Lechevalier, H.A., 1965. Geosmin, an earthy-smelling substance isolated from actinomycetes. Applied Microbiology 13, 935–938.

Heisler, J., Glibert, P.M., Burkholder, J.M., Anderson, D.M., Cochlan, W., Dennison, W.C., Dortch, Q., Gobler, C.J., Heil, C.A., Humphries, E., Lewitus, A., Magnien, R., Marshall, H.G., Sellner, K., Stockwell, D.A., Stoecker, D.K., Suddleson, M., 2008. Eutrophication and harmful algal blooms: A scientific consensus. Harmful Algae 8, 3–13. https://doi.org/10.1016/j.hal.2008.08.006

Huisman, J., Codd, G.A., Paerl, H.W., Ibelings, B.W., Verspagen, J.M.H., Visser, P.M., 2018. Cyanobacterial blooms. Nature Reviews Microbiology 16, 471–483. https://doi.org/10.1038/s41579-018-0040-1

Jüttner, F., Watson, S.B., 2007. Biochemical and ecological control of geosmin and 2-methylisoborneol in source waters. Applied and Environmental Microbiology 73, 4395–4406. https://doi.org/10.1128/aem.02250-06

Kosten, S., Huszar, V.L.M., Bécares, E., Costa, L.S., Donk, E. van, Hansson, L., Jeppesen, E., Kruk, C., Lacerot, G., Mazzeo, N., De Meester, L., Moss, B., Lürling, M., Nõges, T., Romo, S., Scheffer, M., 2011. Warmer climates boost cyanobacterial dominance in shallow lakes. Global Change Biology 18, 118–126. https://doi.org/10.1111/j.1365-2486.2011.02488.x

LEFLAIVE, J., TEN‐HAGE, L., 2007. Algal and cyanobacterial secondary metabolites in freshwaters: A comparison of allelopathic compounds and toxins. Freshwater Biology 52, 199–214. https://doi.org/10.1111/j.1365-2427.2006.01689.x

Li, Z., Yu, J., Yang, M., Zhang, J., Burch, M.D., Han, W., 2010. Cyanobacterial population and harmful metabolites dynamics during a bloom in yanghe reservoir, north China. Harmful Algae 9, 481–488. https://doi.org/10.1016/j.hal.2010.03.003

Lin, T.-F., Watson, S., Suffet, I.H.M., 2018. Taste and odour in source and drinking water: Causes, controls, and consequences. IWA Publishing.

Lukassen, M.B., Menanteau-Ledouble, S., Jonge, N. de, Schram, E., Nielsen, J.L., 2022. Impact of water quality parameters on geosmin levels and geosmin producers in european recirculating aquaculture systems. Journal of Applied Microbiology 132, 2475–2487. https://doi.org/10.1111/jam.15358

Martinez i Quer, A., Larsson, Y., Johansen, A., Arias, C.A., Carvalho, P.N., 2024. Cyanobacterial blooms in surface waters – nature-based solutions, cyanotoxins and their biotransformation products. Water Research 251, 121122. https://doi.org/10.1016/j.watres.2024.121122

Mehnert, G., Leunert, F., Cires, S., Johnk, K.D., Rucker, J., Nixdorf, B., Wiedner, C., 2010. Competitiveness of invasive and native cyanobacteria from temperate freshwaters under various light and temperature conditions. Journal of Plankton Research 32, 1009–1021. https://doi.org/10.1093/plankt/fbq033

Merel, S., Walker, D., Chicana, R., Snyder, S., Baurès, E., Thomas, O., 2013. State of knowledge and concerns on cyanobacterial blooms and cyanotoxins. Environment International 59, 303–327. https://doi.org/10.1016/j.envint.2013.06.013

Mustapha, S., Tijani, J.O., Ndamitso, M.M., Abdulkareem, A.S., Shuaib, D.T., Mohammed, A.K., 2021. A critical review on geosmin and 2-methylisoborneol in water: Sources, effects, detection, and removal techniques. Environmental Monitoring and Assessment 193, 204. https://doi.org/10.1007/s10661-021-08980-9

O’neil, J.m., Davis, T.w., Burford, M.a., Gobler, C.j., 2012. The rise of harmful cyanobacteria blooms: The potential roles of eutrophication and climate change. Harmful Algae 14, 313–334. https://doi.org/10.1016/j.hal.2011.10.027

Oberhaus, L., Briand, J.F., Leboulanger, C., Jacquet, S., Humbert, J.F., 2007. Comparative effects of the quality and quantity of light and temperature on the growth of Planktothrix agardhii and P. rubescens. Journal of Phycology 43, 1191–1199. https://doi.org/10.1111/j.1529-8817.2007.00414.x

Paerl, H.W., Fulton, R.S., Moisander, P.H., Dyble, J., 2001. Harmful freshwater algal blooms, with an emphasis on cyanobacteria. The Scientific World Journal 1, 76–113. https://doi.org/10.1100/tsw.2001.16

Paerl, H.W., Paul, V.J., 2012. Climate change: Links to global expansion of harmful cyanobacteria. Water Research 46, 1349–1363. https://doi.org/10.1016/j.watres.2011.08.002

Paerl, H.W., Xu, H., Mccarthy, M.J., Zhu, G., Qin, B., Li, Y., Gardner, W.S., 2011. Controlling harmful cyanobacterial blooms in a hyper-eutrophic lake (lake taihu, China): The need for a dual nutrient (n & p) management strategy. Water Research 45, 1973–1983. https://doi.org/10.1016/j.watres.2010.09.018

Plaas, H.E., Paerl, H.W., 2020. Toxic cyanobacteria: A growing threat to water and air quality. Environmental Science &amp; Technology 55, 44–64. https://doi.org/10.1021/acs.est.0c06653

Qiu, P., Zhang, Y., Mi, W., Song, G., Bi, Y., 2023. Producers and drivers of odor compounds in a large drinking-water source. Frontiers in Ecology and Evolution 11. https://doi.org/10.3389/fevo.2023.1216567

Reinl, K.L., Harris, T.D., Elfferich, I., Coker, A., Zhan, Q., Domis, L.N.D.S., Morales-Williams, A.M., Bhattacharya, R., Grossart, H.-P., North, R.L., Sweetman, J.N., 2022. The role of organic nutrients in structuring freshwater phytoplankton communities in a rapidly changing world. Water Research 219, 118573. https://doi.org/10.1016/j.watres.2022.118573

Rousso, B.Z., Bertone, E., Stewart, R., Hamilton, D.P., 2020. A systematic literature review of forecasting and predictive models for cyanobacteria blooms in freshwater lakes. Water Research 182, 115959. https://doi.org/10.1016/j.watres.2020.115959

Singh, S., Kate, B.N., Banerjee, U.C., 2005. Bioactive compounds from cyanobacteria and microalgae: An overview. Critical Reviews in Biotechnology 25, 73–95. https://doi.org/10.1080/07388550500248498

Srinivasan, R., Sorial, G.A., 2011. Treatment of taste and odor causing compounds 2-methyl isoborneol and geosmin in drinking water: A critical review. Journal of Environmental Sciences 23, 1–13. https://doi.org/10.1016/s1001-0742(10)60367-1

Su, M., Yu, J., Zhang, J., Chen, H., An, W., Vogt, R.D., Andersen, T., Jia, D., Wang, J., Yang, M., 2015. MIB-producing cyanobacteria (Planktothrix sp.) in a drinking water reservoir: Distribution and odor producing potential. Water Research 68, 444–453. https://doi.org/10.1016/j.watres.2014.09.038

Su, M., Zhu, Y., Jia, Z., Liu, T., Yu, J., Burch, M., Yang, M., 2021. Identification of MIB producers and odor risk assessment using routine data: A case study of an estuary drinking water reservoir. Water Research 192, 116848. https://doi.org/10.1016/j.watres.2021.116848

Sun, D., Yu, J., Yang, M., An, W., Zhao, Y., Lu, N., Yuan, S., Zhang, D., 2014. Occurrence of odor problems in drinking water of major cities across China. Frontiers of Environmental Science & Engineering 8, 411–416. https://doi.org/10.1007/s11783-013-0577-1

Thornton, K.W., 1984. REGIONAL COMPARISONS OF LAKES AND RESERVOIRS: GEOLOGY, CLIMATOLOGY, AND MORPHOLOGY. Lake and Reservoir Management 1, 261–265. https://doi.org/10.1080/07438148409354521

Uwins, H.K., Teasdale, P., Stratton, H., 2007. A case study investigating the occurrence of geosmin and 2-methylisoborneol (MIB) in the surface waters of the hinze dam, gold coast, australia. Water Science and Technology 55, 231–238. https://doi.org/10.2166/wst.2007.184

Wang, R., Li, D., Jin, C.X., Yang, B.W., 2015. Seasonal occurrence and species specificity of fishy and musty odor in huajiang reservoir in winter, china. Water Resources and Industry 11, 13–26. https://doi.org/10.1016/j.wri.2015.04.002

Watson, S.B., 2003. Cyanobacterial and eukaryotic algal odour compounds: Signals or by-products? A review of their biological activity. Phycologia 42, 332–350. https://doi.org/10.2216/i0031-8884-42-4-332.1

Woolway, R.I., Kraemer, B.M., Lenters, J.D., Merchant, C.J., O’Reilly, C.M., Sharma, S., 2020. Global lake responses to climate change. Nature Reviews Earth &amp; Environment 1, 388–403. https://doi.org/10.1038/s43017-020-0067-5

Wu, T., Zhu, G., Wang, Z., Zhu, M., Xu, H., 2022. Seasonal dynamics of odor compounds concentration driven by phytoplankton succession in a subtropical drinking water reservoir, southeast China. Journal of Hazardous Materials 425, 128056. https://doi.org/10.1016/j.jhazmat.2021.128056

Xu, H., Paerl, H.W., Qin, B., Zhu, G., Gaoa, G., 2009. Nitrogen and phosphorus inputs control phytoplankton growth in eutrophic Lake Taihu, China. Limnology and Oceanography 55, 420–432. https://doi.org/10.4319/lo.2010.55.1.0420

Yang, M., Yu, J., Li, Z., Guo, Z., Burch, M., Lin, T., 2008. Taihu Lake not to blame for Wuxi’s Woes. Science 319, 158. https://doi.org/10.1126/science.319.5860.158a

Zamyadi, A., Henderson, R., Stuetz, R., Hofmann, R., Ho, L., Newcombe, G., 2015. Fate of geosmin and 2-methylisoborneol in full-scale water treatment plants. Water Research 83, 171–183. https://doi.org/10.1016/j.watres.2015.06.038

Zhang, Y., Whalen, J.K., Cai, C., Shan, K., Zhou, H., 2023. Harmful cyanobacteria-diatom/dinoflagellate blooms and their cyanotoxins in freshwaters: A nonnegligible chronic health and ecological hazard. Water Research 233, 119807. https://doi.org/10.1016/j.watres.2023.119807

Zohary, T., Gasith, A., 2014. The littoral zone, in: Lake Kinneret. Springer Netherlands, pp. 517–532. https://doi.org/10.1007/978-94-017-8944-8_29