# Integrating littoral zone odor dynamics into early warning and intervention systems for drinking water reservoirs

Yuying Guib,a,#, Tengxin Caob,c,#, Ming Su ID,b,c,*, Yufan Aib,c, Yingjie Lib,c, and Min Yangb,c

# These authors contributed equally to this work.

a College of Environmental Science and Engineering, Ocean University of China, Qingdao 266100, China.
b Key Laboratory of Environmental Aquatic Chemistry, State Key Laboratory of Regional Environment and Sustainability, Research Center for Eco-Environmental Sciences, Chinese Academy of Sciences, Beijing 100085, China.
c University of Chinese Academy of Sciences, Beijing 100049, China.

* Corresponding to: Ming Su (mingsu@rcees.ac.cn)

# Abstract

This is the abstract.

# Introduction

The increasing frequency and intensity of harmful cyanobacterial blooms (CyanoHABs) in surface waters, driven by climate change and intensified anthropogenic pressures, pose a critical and escalating global threat to water security (Feng et al., 2021; Heisler et al., 2008; Huisman et al., 2018; O’neil et al., 2012; Paerl et al., 2001; Paerl and Paul, 2012; Woolway et al., 2020). While substantial research has focused on eutrophication-driven bloom mechanisms and risk assessment in shallow lakes (Kosten et al., 2011; Martinez i Quer et al., 2024; Paerl et al., 2011; Reinl et al., 2022; Rousso et al., 2020; Xu et al., 2009; Zhang et al., 2023), studies on reservoir systems—which constitute a primary source of global drinking water—remain disproportionately scarce. This significant knowledge gap carries critical implications, as CyanoHABs directly threaten drinking water safety through the production of toxins (Carmichael et al., 2001; LEFLAIVE and TEN‐HAGE, 2007; Merel et al., 2013; Plaas and Paerl, 2020) and taste-and-odor (T&O) compounds (Burlingame et al., 2017; Lin et al., 2018; Su et al., 2015; Yang et al., 2008), impacting both human health and water utility. For example, nearly half of China’s source-water reservoirs experience recurrent algal-derived T&O episodes (Sun et al., 2014). Unlike chemical contaminants, cyanobacterial populations can undergo exponential growth under suitable conditions (Asato, 2003; Mehnert et al., 2010), rapidly releasing metabolites (Ai et al., 2025; LEFLAIVE and TEN‐HAGE, 2007; Singh et al., 2005) that often exceed the response capacity of conventional water treatment facilities (Cerón-Vivas et al., 2022; Mustapha et al., 2021; Srinivasan and Sorial, 2011), leading to widespread drinking water quality incidents (Li et al., 2010; Yang et al., 2008).

Effectively managing CyanoHABs in reservoirs presents distinct and formidable challenges. Compared to natural lakes, reservoirs display greater spatial and hydraulic complexity, resulting in less stable and predictable ecological regimes (Dordoni et al., 2023; Thornton, 1984). Conventional reservoir monitoring strategies, typically confined to sampling near dam intakes, fail to capture the spatially heterogeneous distribution of algal biomass and metabolites. Furthermore, the short flow time from intake to drinking water treatment plant—often on the scale of minutes to hours—provides an inadequate operational window for timely intervention. In contrast, littoral zones in reservoirs are characterized by markedly different environmental conditions, such as enhanced nutrient availability and light penetration (Cefalì et al., 2016; Zohary and Gasith, 2014), which may promote early cyanobacterial proliferation. Despite their potential significance as critical sentinel sites, these areas remain critically under-monitored. Consequently, the mechanistic linkages between littoral zone CyanoHAB dynamics and downstream water quality at intakes are poorly understood and quantified.

Geosmin (GSM) and 2-methylisoborneol (MIB) are among the most concerning terpenoid T&O compounds produced by cyanobacteria (Jüttner and Watson, 2007; Watson, 2003). Their low molecular weight and chemical nature allow them to penetrate conventional water treatment barriers and persist into tap water (Lin et al., 2018; Zamyadi et al., 2015), directly impairing drinking water quality and consumer acceptance. In China, MIB-related episodes have been widely reported and studied, with corresponding mitigation strategies developed (Su et al., 2021, 2015). In contrast, GSM incidents have historically been more prevalent in the Southern Hemisphere (Bowmer et al., 1992; Uwins et al., 2007) but are increasingly reported across Northern Hemisphere water bodies (Qiu et al., 2023; Wu et al., 2022). Notably, GSM-producing cyanobacterial taxa are phylogenetically more diverse than those associated with MIB (AWWA, 2010), complicating source-tracking and management efforts. To address these emerging challenges, we conducted a multi-year, spatially resolved monitoring study in Yuqiao Reservoir—a major drinking water source for Tianjin, China, with a documented history of GSM episodes. Integrating water quality analysis, molecular sequencing of algal communities, and advanced statistical modeling, we demonstrate significant spatial decoupling of cyanobacterial assemblages and T&O compounds between littoral zones and the dam area. We further identify key odor-producing taxa and establish that littoral zone cyanobacterial dynamics exert a decisive influence on T&O levels at the intake. These findings position the littoral zone as a critical sentinel for early warning and a strategic target for proactive intervention. This study advances a scientifically informed monitoring framework, shifting the paradigm from reactive response to preventive management of CyanoHAB-derived T&O risks in reservoir-based water supplies.

# Materials and methods

## Study area and sampling sites

Yuqiao Reservoir is located in the east of the city of Jizhou district in Tianjin, with a total capacity of 1.559 billion cubic metres, making it one of the key large-scale reservoirs in the country and an important urban water source for Tianjin. The reservoir has been experiencing persistent odour problems in recent years. Depending on the geographical location, a total of eight sampling points were selected in this reservoir to investigate the spatial distribution of geosmin concentrations, namely, east of the reservoir, southeast of the reservoir, south of the reservoir, centre of the reservoir, west of the reservoir, water discharge hole, north of the reservoir, and Fengshan. Among them, the water depth in the discharge hole could reach 12 metres, where stratified sampling was conducted. The rest of the sites were sampled at depths of 2 to 4 metres and surface water samples were taken. Sampling is carried out during the annual odour period, two to three times per week, with nutrient measurements and algal counts. All samples were stored at 4°C for 24 hours prior to use. Geomin concentrations were determined using solid phase microextraction (SPME) coupled with gas chromatography-mass spectrometry (GC-MS). Physicochemical properties such as water temperature, ph, dissolved oxygen (DO) and turbidity were measured in situ using a multiprobe instrument (EXO2, Xylem, USA). Ammonia (NH 4-N), total nitrogen (TN), and total phosphorus (TP) were measured according to national water quality standards. Samples for algal counts were preserved in 5% luger iodine, concentrated to 10 mL after 48 h of sedimentation, and cell counts were carried out under a microscope (OLYMPUS BX51, Olympus Optical, Tokyo,Japan) using cell counting plates (S52, 1 mL, Sedgewick-Rafter).

## Quantification of geoA gene

Quantitative PCR was employed to determine the absolute abundance of geoA genes,which encode the geosmin syntheses.The amplification of mic genes was carried out using the 173AF(5’-TGTGAGTACCCAAGAGG-3’) and 173AR(5’-CTGCCAATCCTGAAGTCCTTT-3’) primers.The quantitative PCR assays were performed on a LightCycler 96 instrument(Roche, USA) with a 25μL reaction mixture comprising 12.5μLTBGreen®Premix ExTaq™(Takara, Japan), 0.8μL of each primer (173AF and 173AR), 5.9μL deionized water, and 5μL template DNA.

The reaction conditions were as follows: initial pre-incubation at 95°C for 10 min, followed by 35 cycles at 95°C for 20 s, 50°C for 20 s, and 72°C for 20 s. A subsequent DNA melting step ranging from 65°C to 97°C was included.

Absolute abundance was determined based on a standard curve using the formula: Cq = -3.6261lg(cgeoA)+37.70(R2=0.999).All samples were quantified in triplicate, and a negative control was implemented to mitigate the risk of nonspecific amplification.

## epicPCR of geoA-16S rRNA Gene Fusion Fragments

epicPCR combines established cell isolation, encapsulation and paired amplification methods. The method is outlined as follows: an initial aqueous oil-in-oil sample emulsion produces approximately 500 million droplets, each with a volume of approximately 1 nanolitre, containing single cells. These cells are loaded and dispersed such that, on average, less than one out of every 100 droplets contains cells. Each of these droplets also contains acrylamide monomers, which encapsulate the cells when a catalyst is added, forming polyacrylamide beads. The polyacrylamide beads provide protection for the bacterial chromosomes and plasmids, preventing them from spreading when used for fusion PCR. Polyacrylamide beads are typically 5 - 30μm in diameter, with most beads being approximately 10μm in diameter, and fusion PCR is performed on hydrogel-captured genomes in a secondary emulsion to ensure that each epicPCR is compartmentalised and performed as a single reaction. （在2023年水库geosmin爆发期间采集水样进行GC-MS检测，结果表明11月的样品中采集自YQ05的水样、12月的样品中采集自YQ03的水样有着高于同月份其他点位的tgsm，浓度分别达到了28.43ng/L和29.84ng/L.由于产嗅蓝藻通常在水体中的相对丰度较低，所以用1.2um滤膜富集这两个点位的水样以提高产嗅蓝藻的比例。用Fast DNA试剂盒提取富集在滤膜上的藻类DNA进行了Quantitative PCR扩增，结果表明这两个点位的geoA gene浓度均达到了10^5copies/L.对这两个点位的水样制备细胞悬浮液，进行epicPCR扩增，对巢氏PCR扩增产物进行1%琼脂糖凝胶电泳，结果如图x-e所示。）

## Preparation of Polyacrylamide Gel Beads Containing Exposed Genomes:

Sample Collection and Filtration: Water samples were filtered using 1.2μm Isopore™Membrane Filters, and microbial cells within the filters were washed using normal saline(0.85%). 制备环境水样的细胞凝珠：取上述水样、丙烯酰胺溶液和过硫酸铵溶液混匀后加入STT乳化油；接着再加入四甲基乙二胺，混匀，静置状态下，进行丙烯酰胺聚合反应，制成包裹水体中藻类细胞的聚丙烯酰胺-细胞凝珠混合液；对聚丙烯酰胺-细胞凝珠混合液依次进行水饱和乙醚清洗、超纯水洗涤后向聚丙烯酰胺-细胞凝珠加入1×TK缓冲液，形成聚丙烯酰胺-细胞凝珠TK悬液，然后采用细胞过滤器进行过滤，制得纯化聚丙烯酰胺-细胞凝珠TK悬液；向纯化聚丙烯酰胺-细胞凝珠TK悬液中添加溶菌酶，溶解藻类细胞细胞壁后加入蛋白酶K和TritonX-100，酶解去除与核酸结合的组蛋白，使藻类细胞DNA游离，以及破坏藻类细胞细胞膜，使得藻类细胞基因组暴露在聚丙烯酰胺凝珠壳内，获得含有暴露基因组的聚丙烯酰胺-细胞凝珠。 Preparation of environmental water samples of cell beads: take the above water samples, acrylamide solution and ammonium persulfate solution mixed well, then add STT emulsified oil; then add tetramethylethylenediamine, mixing, under the state of static, acrylamide polymerisation reaction, made of polyacrylamide-cell beads wrapped in the algae cells in the water body mixture; polyacrylamide-cell beads mixture in order to carry out water-saturated ethyl ether wash, ultrapure water After washing, 1×TK buffer was added to the polyacrylamide-cell beads to form a polyacrylamide-cell beads TK suspension, which was then filtered using a cell filter to produce a purified polyacrylamide-cell beads TK suspension; lysozyme was added to the purified polyacrylamide-cell beads TK suspension to lyse algal cell cell walls and then proteinase K and TritonX-100 were added to enzymatically digest it. Removing histones bound to nucleic acids, freeing algal cell DNA, as well as destroying the cell membrane of the algal cell, so that the genome of the algal cell is exposed in the shell of the polyacrylamide beads, and obtaining polyacrylamide-cellular gel beads containing the exposed genome.

# Results

## National Survey of Geosmin Occurrence in Drinking Water Sources

A nationwide survey was conducted to assess geosmin (geosmin) concentrations across China’s major drinking water sources, encompassing a total of 214 sampling sites from reservoirs and key surface water bodies in 26 provinces. geosmin was detected in 189 locations. Measured concentrations spanned over four orders of magnitude, ranging from below detection limits to a maximum of 7100 ng L-1, recorded at a site in Zhejiang Province. Overall, 14.5% of sampling sites exceeded the common odor threshold of 10 ng L-1. Geospatial analysis revealed pronounced heterogeneity. Elevated geosmin levels were broadly clustered in the eastern and southern regions, including the Yangtze River Delta, Pearl River Delta, and parts of North China. In contrast, concentrations in northwestern China (e.g., Tibet, Xinjiang, Qinghai) and certain central provinces were consistently low or below detection limits, corroborating a general east-high, west-low spatial pattern. This comprehensive dataset demonstrates that geosmin is a widespread odorant affecting drinking water sources across diverse climatic and geographical zones in China.

Fig. 1: A multi-scale visualization of geosmin occurrence: linking individual sampling sites to provincial-level aggregates (Provincial/regional aggregate bars (log-scale binned) connected to individual sampling sites. Beijing-Tianjin-Hebei and Jiangsu-Shanghai-Zhejiang are treated as single units to avoid spatial overlap).

## Seasonal and Interannual Dynamics in Yuqiao Reservoir

To elucidate the temporal dynamics of geosmin occurrence, Yuqiao Reservoir (YQ, a key drinking water source in the Beijing-Tianjin-Hebei region) was selected for sustained, high-frequency monitoring at its primary water intake (site YQ01) from 2019 to 2024. The long-term record revealed a recurrent and pronounced seasonal cycle (Fig. 2A). Concentrations were consistently low (<5 ng L-1) during spring (April–May), followed by a rapid increase in early summer (June) culminating in a primary peak during the warmest months (July–August). A second, broader peak persisted throughout autumn and early winter (October–December), after which levels gradually declined. The odor threshold of 10 ng L-1 was frequently exceeded. Analysis of the monthly median concentrations across water years (May to April) showed that the threshold was surpassed during at least 40%* of the summer months (July–September) and 100% of the autumn/early winter months (October–December) during the monitoring period, highlighting a persistent odor risk in the warmer half of the year.

At the interannual scale (Fig. 2B, annual boxplots), geosmin concentrations exhibited a notable decreasing trend following an exceptionally high initial year. The median concentration in the 2019–2020 water year was 2.5 ng L-1, substantially exceeding the threshold. Subsequently, from 2020 to 2024, annual median concentrations stabilized within a lower range of 5.2–10 ng L-1. A Kruskal-Wallis test confirmed statistically significant differences among the annual datasets (χ2(4) = 64.3, p < 0.001). This trend suggests a potential shift in baseline conditions or source inputs after the initial monitoring year, though seasonal peaks remained a consistent feature of the reservoir’s annual cycle.

Fig. 2: Long-term dynamics of geosmin and phytoplankton community in Yuqiao Reservoir (2019–2024). (A) Seasonal and interannual variations in geosmin concentrations at the main water intake (site YQ01). (B) Heatmap showing monthly log10-transformed biomass of total phytoplankton, total cyanobacteria, and key geosmin-producing cyanobacterial genera. (C) Temporal patterns of Planktothrix agardhii biomass across sampling sites. (D) Agarose gel electrophoresis confirming the molecular identification of P. agardhii. (E) Spatial mapping of seasonal P. agardhii biomass variations at representative sampling sites overlaid on the reservoir map. (F) Microscopic image of P. agardhii filaments.

## Phytoplankton Community Dynamics and Key Cyanobacterial Genera

To investigate the relationship between geosmin occurrence and phytoplankton dynamics, high-resolution phytoplankton composition data from Yuqiao Reservoir (site YQ01, 2019–2024) were analyzed (Fig. 2B). The total phytoplankton biomass (expressed as log10-transformed cell density) exhibited a clear seasonal pattern, with the lowest levels observed in winter (January–March, ~107 cells L-1) and a rapid increase beginning in spring (April). Biomass peaked during the summer months (July–September, >108 cells L-1), remained elevated through autumn (October–December), and then declined sharply in late winter, mirroring the general temporal trend of geosmin concentrations.

Cyanobacteria dominated the phytoplankton community throughout most of the year, contributing substantially to total biomass. Their seasonal trajectory closely followed that of total phytoplankton, with the highest cyanobacterial biomass recorded from June to October. Within the cyanobacterial assemblage, several filamentous genera known for potential geosmin production exhibited distinct seasonal successions. Planktothrix agardhii and Aphanizomenon gracile were perennial but showed clear seasonality, with biomass maxima in summer and autumn. Pseudanabaena catenata was primarily a spring (April–May) taxon. Several Dolichospermum species (including D. circinale, D. circinalis, D. flos-aquae, D. planctonicum, and D. spiroides) and Aphanizomenon flos-aquae typically emerged in late spring or early summer, proliferated during summer, and declined by late autumn. Other taxa, such as Trichormus variabilis, Dolichospermum circularis, Microcoleus amoenus, Geitlerinema splendidum, and Pseudanabaena limnetica, displayed more sporadic occurrences, often with pronounced peaks limited to specific months within the warm season (e.g., June–August).

The co-occurrence of elevated geosmin concentrations with periods of high cyanobacterial biomass, particularly during summer and autumn blooms dominated by known or putative geosmin-producing genera (e.g., Planktothrix, Aphanizomenon, Dolichospermum), suggests a strong biotic source for the observed odor episodes. The temporal succession of different cyanobacterial taxa may contribute to the characteristic bimodal (summer and autumn) geosmin peak observed in the reservoir.

## Identification and Spatiotemporal Distribution of the Primary Geosmin Producer

Planktothrix agardhii was identified as the dominant geosmin-producing cyanobacterium based on gene sequencing in Yuqiao Reservoir (Fig. 2D, 2F). Spatial monitoring during 2023 revealed a pronounced biomass gradient, with littoral sites (YQ03, YQ04, YQ06, YQ07) sustaining significantly higher cell densities than the central dam intake (YQ01). The maximum observed biomass reached 5.1×107 cells L-1 at a littoral site, compared to 2.9×107 cells L-1 at the dam. Elevated biomass thresholds (>2×107 cells L-1) were exceeded in 7% of littoral samples, respectively, but in 4% of dam samples. Temporally, P. agardhii exhibited a perennial presence with sustained warm-season biomass, contrasting with the ephemeral peaks of other cyanobacteria (Fig. 2C). Major bloom events (≥2×107 cells L-1) propagated from littoral zones to the dam (Fig. 2E), with sites such as YQ05 exceeding the threshold approximately 8 days before YQ01. This spatiotemporal pattern indicates littoral zones as initiation hotspots for P. agardhii proliferation, with subsequent advection likely driving geosmin peaks at the drinking water intake.

The temporal progression of major bloom events (biomass≥2×107 cells L−1) followed a clear spatial sequence from the littoral zone towards the dam. For instance, site YQ05 first exceeded this threshold in mid-October, approximately -8 days before the dam site (YQ01) reached comparable levels in late October. This spatiotemporal decoupling highlights the littoral zones as probable initiation hotspots for P. agardhii proliferation, with subsequent advection contributing to the geosmin peaks observed at the central water intake.

## Distinct Spatiotemporal Patterns of Two Geosmin Episodes

To further elucidate the spatiotemporal dynamics of geosmin (geosmin), we identified two distinct odor episodes in Yuqiao Reservoir (April–August and August–December 2023). Spatial mapping of monthly averaged geosmin concentrations at seven representative sites revealed contrasting patterns between the littoral and dam regions (Fig. 3).

During the first episode (April–August), littoral sites (YQ02, YQ03, YQ04, YQ06, YQ07) exhibited elevated concentrations in spring (April–June), with values ranging from 0 to 21.2 ng L-1. These concentrations then declined by August to between 0.6 and 5.8 ng L-1. In contrast, the central dam site (YQ01) and the nearby YQ05 (located close to littoral zones) showed a delayed response. Their concentrations started low in April (0 ng L−1) and increased steadily, peaking in August at 8.5 ng L-1.

The second episode (August–December) exhibited a reversed spatiotemporal pattern. Littoral sites experienced a sharp peak in October, with concentrations reaching 0 to 36.4 ng L-1, followed by a decline to 0–29.8 ng L-1 by December. Conversely, the dam site (YQ01) showed a sustained, monotonic increase from August (0 ng L-1) to December (24.2 ng L-1). Site YQ05, located between littoral and dam zones, exhibited an intermediate transitional pattern, with its decline beginning in November rather than October, further illustrating the spatiotemporal gradient from littoral production zones to the central reservoir.

These divergent spatiotemporal trajectories (Fig. 3) highlight a systematic decoupling: littoral zones consistently experience an earlier onset and subsequent decline during both odor episodes, while the dam region exhibits a delayed and prolonged response. This pattern likely reflects the integrated effects of initial production hotspots in the littoral zone and subsequent advective transport toward the central reservoir.

Fig. 3: Spatial mapping of monthly mean geosmin concentrations during Episode 1 (April–August, A) and Episode 2 (August–December, B) in Yuqiao Reservoir.

## Predictive Relationship between Planktothrix agardhii Dynamics and Geosmin Episodes

To quantify the causal linkage between the primary producer and odor events, we performed time-lagged correlation analyses between Planktothrix agardhii biomass and geosmin (geosmin) concentrations at the near-littoral site YQ05 and the dam intake YQ01 (Fig. 4A). These sites were selected for their continuous long-term monitoring data. The analysis revealed fundamentally distinct temporal coupling patterns.

At the near-littoral site YQ05, the strongest correlation (R2 = 0.5, p < 0.001) occurred at zero lag, indicating near-instantaneous geosmin production associated with local P. agardhii proliferation. The correlation coefficient decreased progressively as the lag period increased. Conversely, at the downstream dam site YQ01, the correlation was weakest at zero lag but strengthened with increasing lag time, reaching a maximum at a 5-week lag (R2 = 0.41, p < 0.001). This opposite pattern demonstrates that P. agardhii biomass serves as a concurrent indicator at its production zone (YQ05) but as a leading indicator for geosmin risk at the distal water intake (YQ01).

Regression models were established at the respective optimal lag periods (Fig. 4B). For YQ05 (zero lag), the relationship was log10(geosmin) = 0.24 + 0.29×log10(P. agardhii biomass). For YQ01 (5-week lag), the model was log10(geosmin) = 0.56 + 0.2×log10(P. agardhii biomass). The steeper slope for YQ01 suggests a potential amplification effect during the transport and transformation processes from the littoral production zone to the central reservoir.

The distinct temporal phasing is visually evident in the bi-weekly time series (Fig. 4C). At YQ05, peaks in P. agardhii biomass and geosmin concentrations were closely synchronized. In contrast, at YQ01, biomass peaks consistently preceded major geosmin episodes by several weeks. For instance, the autumn biomass peak preceded the highest geosmin concentration by approximately 6.7 weeks. This consistent lag provides a critical early-warning window. When P. agardhii biomass at YQ05 exceeded 5.0×107 cells L-1, geosmin at YQ01 subsequently exceeded the 10 ng L-1 odor threshold within 4-6 weeks in 61.9% of cases. These quantified relationships underscore the utility of near-littoral cyanobacterial monitoring for forecasting odor risks at the drinking water intake.

Fig. 4: Time-lagged relationships between Planktothrix agardhii biomass and geosmin concentrations at near-littoral and dam sites. (A) Correlation coefficients (R²) between P. agardhii biomass and geosmin at different lag periods (0–5 weeks) for site YQ05 (near-littoral) and YQ01 (dam intake). (B) Linear regression models at optimal lag periods: zero lag for YQ05 and 5-week lag for YQ01. (C) Scatter plots with fitted regression lines showing the quantitative relationships between P. agardhii biomass and geosmin at each site.

## Ammonia as a Key Environmental Driver and Proposed Transport Pathway

Principal component analysis (PCA) of key environmental variables identified ammonia (NH₄⁺-N) as the most strongly correlated factor with geosmin (geosmin) variation in the PC1–PC2 plane (Fig. 5A). The first two principal components explained 60.6% of the total variance. PC1 (explaining 43.5%) represented a thermal‑nutrient productivity axis, with high loadings from temperature (0.342), total phosphorus (TP, 0.39), and chlorophyll‑a (Chl‑a, 0.428). PC2 (explaining 17.1%) represented a nitrogen availability axis, dominated by ammonia (-0.449) and total nitrogen (TN, 0.432). geosmin exhibited moderate positive loadings on both PC1 (0.122) and PC2 (0.229), indicating that its variability was jointly influenced by thermal‑nutrient conditions and nitrogen availability.

To examine this relationship spatially, we performed log‑log regressions between ammonia and geosmin across three reservoir zones: littoral sites (YQ02–YQ04, YQ06–YQ07), the transitional near‑littoral site (YQ05), and the central dam intake (YQ01) (Fig. 5B). The correlation strength followed a distinct gradient, with the highest R2 at littoral sites (0.365), intermediate at YQ05 (0.203), and lowest at YQ01 (0.035). This pattern indicates that the linkage between ammonia and geosmin is most direct within littoral production zones and becomes progressively decoupled along the transport pathway toward the dam.

Integrating these findings with the spatiotemporal dynamics of Planktothrix agardhii and geosmin, we propose a conceptual transport pathway for odor episodes (Fig. 5C). High‑resolution bathymetric mapping shows that shallow littoral areas (depths < 4–5 m) correspond to zones of elevated ammonia and serve as initiation hotspots for P. agardhii proliferation and geosmin production. Subsequent advective transport, guided by the reservoir’s hydrodynamic circulation, gradually propagates cells and dissolved geosmin from littoral zones through the transitional region (YQ05) to the central dam area (YQ01). This conceptual model synthesizes three key observations: (1) the spatial gradient in ammonia‑geosmin coupling, (2) the consistent temporal lags in cyanobacterial biomass and geosmin peaks between littoral and dam sites, and (3) the influence of bathymetry on water residence times. Together, these elements provide a mechanistic framework for understanding the origin, transport, and delayed manifestation of geosmin episodes at the drinking‑water intake of Yuqiao Reservoir.

Fig. 5: Environmental drivers and conceptual transport pathway of geosmin in Yuqiao Reservoir. (A) Principal component analysis (PCA) biplot showing relationships between geosmin and key environmental variables. (B) Log-log regressions between ammonia concentration and geosmin across littoral, transitional (YQ05), and dam (YQ01) zones. (C) Bathymetric map of Yuqiao Reservoir depicting the proposed transport pathway from littoral production zones to the central water intake.

# Discussion

## Geosmin as a Prevalent Challenge for Drinking Water Security in China

Our nationwide survey reveals that geosmin contamination is widespread across China’s major drinking water sources, with 14.5% of the surveyed sites exceeding the common odor threshold of 10 ng L-1. This finding underscores geosmin as a significant and pervasive water quality issue, corroborating similar reports from diverse climatic regions globally (Devi et al., 2021; Lin et al., 2018). The pronounced spatial heterogeneity—marked by elevated concentrations in eastern and southern regions such as the Yangtze River Delta and Pearl River Delta—suggests that regional factors, including climate, watershed land use, and nutrient loading patterns, are critical drivers of geosmin risk. The compound’s low odor threshold (often <10 ng L-1, (watson2010aquatic?)) and resistance to conventional treatment pose substantial challenges for water utilities, directly impacting consumer acceptance and compliance with water safety objectives linked to Sustainable Development Goal 6 [@]. The economic ramifications are considerable, driving investments in advanced treatment technologies and proactive management strategies worldwide [cite].

asquith2018distribution

## Littoral Zones as Production Hotspots: Ecological Mechanisms and Spatiotemporal Decoupling

The sustained monitoring in Yuqiao Reservoir elucidates littoral zones as critical ecological niches for the proliferation of geosmin-producing cyanobacteria, particularly Planktothrix agardhii. The distinct spatiotemporal patterns—earlier onset, higher biomass, and stronger correlation with ammonia in littoral areas—highlight a production-transport decoupling central to reservoir-wide odor episodes.

The affinity of filamentous cyanobacteria like P. agardhii for littoral environments can be attributed to several adaptive traits. Their filamentous morphology confers advantages in shallow, intermittently mixed waters through regulated buoyancy and reduced sedimentation losses, enabling optimal positioning for light harvesting [cite]. This is complemented by low-light adaptations via phycobiliprotein pigments, allowing persistence under the variable light regimes typical of littoral zones [cite]. Furthermore, the strong correlation observed between ammonia and geosmin specifically in littoral sites (R²= 0.365) suggests that nitrogen availability, likely influenced by shoreline inputs, is a key regulatory factor for production in these zones [cite]. In contrast, the weakened correlation at the dam intake (R²= 0.035) reflects the integration and modification of signals during transport.

The systematic spatial decoupling is evidenced by the consistent 8-day lag in major bloom events between littoral sites and the dam. This lag represents not merely physical advection but may also involve continued metabolic activity and potential accumulation in transitional zones. These findings challenge conventional reservoir management focused predominantly on pelagic or intake monitoring and align with growing recognition of nearshore areas as initiation sites for cyanobacterial metabolites [cite].

## Towards a Proactive Monitoring Paradigm: Quantitative Predictive Tools

Current monitoring strategies centered on final water intakes are inherently reactive, offering limited time for intervention. Our study establishes a quantitative framework for a proactive, risk-based approach. The time-lagged correlation analysis reveals a fundamental difference in signal utility: P. agardhii biomass acts as a contemporaneous indicator at its production source (YQ05, maximum R²at zero lag) but serves as a leading indicator at the distal intake (YQ01, maximum R²= 0.41 at a 5-week lag).

The derived predictive relationships offer actionable thresholds for early warning. For instance, when P. agardhii biomass in the near-littoral zone (YQ05) exceeded 5.0×106 cells L-1, geosmin at the intake subsequently exceeded the 10 ng L-1 threshold within 4–6 weeks in 61.9% of cases. This lead time is operationally significant, enabling utilities to implement preventive measures such as optimized reservoir operation, selective withdrawal, or pre-emptive treatment adjustments [cite].

We propose a tiered monitoring framework integrating (Fig. 6): (1) sentinel stations in representative littoral zones for early detection of producer biomass, (2) gradient stations along the main transport pathway to track progression, and (3) final verification at the intake. Shifting from a reactive to a predictive paradigm could enhance operational resilience, potentially reducing treatment costs and mitigating service disruptions.

Fig. 6: sketch of geosmin producer

## Implications, Limitations, and Future Perspectives

This study provides mechanistic insights into geosmin dynamics in a drinking water reservoir and translates them into practical monitoring tools. The identification of littoral zones as production hotspots and the quantification of time-lagged predictive relationships offer a blueprint for evidence-based management.

Several limitations should be acknowledged. First, while the 4–6 year dataset captures clear seasonal and interannual patterns, longer-term monitoring would strengthen the understanding of trends, particularly under climate change scenarios. Second, the dominant role of P. agardhii is specific to Yuqiao Reservoir; other systems may harbor different producer taxa (e.g., Dolichospermum, Aphanizomenon) with distinct ecological drivers [cite]. Third, the predictive models require validation across diverse reservoir types and geographical settings to assess their broader applicability.

Future research should focus on: (1) integrating molecular tools to characterize the diversity and expression of geosmin synthesis genes within cyanobacterial communities, (2) coupling ecological monitoring with high-resolution hydrodynamic modeling to precisely quantify transport pathways and residence times, and (3) conducting cost-benefit analyses to evaluate the economic viability of implementing expanded littoral monitoring programs. Furthermore, investigating the interactions between climate variables (e.g., warming, extreme precipitation) and the identified production-transport mechanisms will be crucial for forecasting future risk scenarios. By unraveling the spatiotemporal mechanisms of geosmin production and transport, and by establishing quantifiable early-warning indicators, this work provides a scientific foundation for moving beyond reactive response. It equips water managers with a proactive strategy to safeguard drinking water quality against this persistent olfactory challenge.

# Conclusions

# Acknowledgements

This work was financially supported by the National Natural Science Foundation of China (52030002, 51878649), and Youth Innovation Promotion Association CAS.

# References

Ai, Y., Wu, Y., Su, M., Gui, Y., Du, Y., Fang, J., Cao, T., Yang, M., 2025. Cyanobacterial crowding-out effects on metabolite partitioning: Modeling 2-methylisoborneol (MIB) release dynamics and implications. Journal of Hazardous Materials 140118. https://doi.org/https://doi.org/10.1016/j.jhazmat.2025.140118

Asato, Y., 2003. Toward an understanding of cell growth and the cell division cycle of unicellular photoautotrophic cyanobacteria. Cellular and Molecular Life Sciences (CMLS) 60, 663–687. https://doi.org/10.1007/s00018-003-2079-y

AWWA, A.W.W.A., 2010. Algae: Source to treatment. American Water Works Association.

Bowmer, K.H., Padovan, A., Oliver, R.L., Korth, W., Ganf, G.G., 1992. Physiology of geosmin production by Anabaena Circinalis isolated from the murrumbidgee river, australia. Water Science and Technology 25, 259–267. https://doi.org/10.2166/wst.1992.0060

Burlingame, G.A., Doty, R.L., Dietrich, A.M., 2017. Humans as sensors to evaluate drinking water taste and odor: A review. Journal AWWA 109, 13–24. https://doi.org/10.5942/jawwa.2017.109.0118

Carmichael, W.W., Azevedo, S.M., An, J.S., Molica, R.J., Jochimsen, E.M., Lau, S., Rinehart, K.L., Shaw, G.R., Eaglesham, G.K., 2001. Human fatalities from cyanobacteria: Chemical and biological evidence for cyanotoxins. Environmental Health Perspectives 109, 663–668. https://doi.org/10.1289/ehp.01109663

Cefalì, M.E., Cebrian, E., Chappuis, E., Pinedo, S., Terradas, M., Mariani, S., Ballesteros, E., 2016. Life on the boundary: Environmental factors as drivers of habitat distribution in the littoral zone. Estuarine, Coastal and Shelf Science 172, 81–92. https://doi.org/10.1016/j.ecss.2016.01.043

Cerón-Vivas, A., Villamizar León, M.P., Cajigas, Á.A., 2022. Geosmin and 2-methylisoborneol removal in drinking water treatment. Water Practice and Technology 18, 159–167. https://doi.org/10.2166/wpt.2022.167

Devi, A., Chiu, Y.-T., Hsueh, H.-T., Lin, T.-F., 2021. Quantitative PCR based detection system for cyanobacterial geosmin/2-methylisoborneol (2-MIB) events in drinking water sources: Current status and challenges. Water Research 188, 116478. https://doi.org/10.1016/j.watres.2020.116478

Dordoni, M., Zappalà, P., Barth, J.A.C., 2023. A preliminary global hydrochemical comparison of lakes and reservoirs. Frontiers in Water 5. https://doi.org/10.3389/frwa.2023.1084050

Feng, L., Dai, Y., Hou, X., Xu, Y., Liu, J., Zheng, C., 2021. Concerns about phytoplankton bloom trends in global lakes. Nature 590, E35–E47. https://doi.org/10.1038/s41586-021-03254-3

Heisler, J., Glibert, P.M., Burkholder, J.M., Anderson, D.M., Cochlan, W., Dennison, W.C., Dortch, Q., Gobler, C.J., Heil, C.A., Humphries, E., Lewitus, A., Magnien, R., Marshall, H.G., Sellner, K., Stockwell, D.A., Stoecker, D.K., Suddleson, M., 2008. Eutrophication and harmful algal blooms: A scientific consensus. Harmful Algae 8, 3–13. https://doi.org/10.1016/j.hal.2008.08.006

Huisman, J., Codd, G.A., Paerl, H.W., Ibelings, B.W., Verspagen, J.M.H., Visser, P.M., 2018. Cyanobacterial blooms. Nature Reviews Microbiology 16, 471–483. https://doi.org/10.1038/s41579-018-0040-1

Jüttner, F., Watson, S.B., 2007. Biochemical and ecological control of geosmin and 2-methylisoborneol in source waters. Applied and Environmental Microbiology 73, 4395–4406. https://doi.org/10.1128/aem.02250-06

Kosten, S., Huszar, V.L.M., Bécares, E., Costa, L.S., Donk, E. van, Hansson, L., Jeppesen, E., Kruk, C., Lacerot, G., Mazzeo, N., De Meester, L., Moss, B., Lürling, M., Nõges, T., Romo, S., Scheffer, M., 2011. Warmer climates boost cyanobacterial dominance in shallow lakes. Global Change Biology 18, 118–126. https://doi.org/10.1111/j.1365-2486.2011.02488.x

LEFLAIVE, J., TEN‐HAGE, L., 2007. Algal and cyanobacterial secondary metabolites in freshwaters: A comparison of allelopathic compounds and toxins. Freshwater Biology 52, 199–214. https://doi.org/10.1111/j.1365-2427.2006.01689.x

Li, Z., Yu, J., Yang, M., Zhang, J., Burch, M.D., Han, W., 2010. Cyanobacterial population and harmful metabolites dynamics during a bloom in yanghe reservoir, north China. Harmful Algae 9, 481–488. https://doi.org/10.1016/j.hal.2010.03.003

Lin, T.-F., Watson, S., Suffet, I.H.M., 2018. Taste and odour in source and drinking water: Causes, controls, and consequences. IWA Publishing.

Martinez i Quer, A., Larsson, Y., Johansen, A., Arias, C.A., Carvalho, P.N., 2024. Cyanobacterial blooms in surface waters – nature-based solutions, cyanotoxins and their biotransformation products. Water Research 251, 121122. https://doi.org/10.1016/j.watres.2024.121122

Mehnert, G., Leunert, F., Cires, S., Johnk, K.D., Rucker, J., Nixdorf, B., Wiedner, C., 2010. Competitiveness of invasive and native cyanobacteria from temperate freshwaters under various light and temperature conditions. Journal of Plankton Research 32, 1009–1021. https://doi.org/10.1093/plankt/fbq033

Merel, S., Walker, D., Chicana, R., Snyder, S., Baurès, E., Thomas, O., 2013. State of knowledge and concerns on cyanobacterial blooms and cyanotoxins. Environment International 59, 303–327. https://doi.org/10.1016/j.envint.2013.06.013

Mustapha, S., Tijani, J.O., Ndamitso, M.M., Abdulkareem, A.S., Shuaib, D.T., Mohammed, A.K., 2021. A critical review on geosmin and 2-methylisoborneol in water: Sources, effects, detection, and removal techniques. Environmental Monitoring and Assessment 193, 204. https://doi.org/10.1007/s10661-021-08980-9

O’neil, J.m., Davis, T.w., Burford, M.a., Gobler, C.j., 2012. The rise of harmful cyanobacteria blooms: The potential roles of eutrophication and climate change. Harmful Algae 14, 313–334. https://doi.org/10.1016/j.hal.2011.10.027

Paerl, H.W., Fulton, R.S., Moisander, P.H., Dyble, J., 2001. Harmful freshwater algal blooms, with an emphasis on cyanobacteria. The Scientific World Journal 1, 76–113. https://doi.org/10.1100/tsw.2001.16

Paerl, H.W., Paul, V.J., 2012. Climate change: Links to global expansion of harmful cyanobacteria. Water Research 46, 1349–1363. https://doi.org/10.1016/j.watres.2011.08.002

Paerl, H.W., Xu, H., Mccarthy, M.J., Zhu, G., Qin, B., Li, Y., Gardner, W.S., 2011. Controlling harmful cyanobacterial blooms in a hyper-eutrophic lake (lake taihu, China): The need for a dual nutrient (n & p) management strategy. Water Research 45, 1973–1983. https://doi.org/10.1016/j.watres.2010.09.018

Plaas, H.E., Paerl, H.W., 2020. Toxic cyanobacteria: A growing threat to water and air quality. Environmental Science &amp; Technology 55, 44–64. https://doi.org/10.1021/acs.est.0c06653

Qiu, P., Zhang, Y., Mi, W., Song, G., Bi, Y., 2023. Producers and drivers of odor compounds in a large drinking-water source. Frontiers in Ecology and Evolution 11. https://doi.org/10.3389/fevo.2023.1216567

Reinl, K.L., Harris, T.D., Elfferich, I., Coker, A., Zhan, Q., Domis, L.N.D.S., Morales-Williams, A.M., Bhattacharya, R., Grossart, H.-P., North, R.L., Sweetman, J.N., 2022. The role of organic nutrients in structuring freshwater phytoplankton communities in a rapidly changing world. Water Research 219, 118573. https://doi.org/10.1016/j.watres.2022.118573

Rousso, B.Z., Bertone, E., Stewart, R., Hamilton, D.P., 2020. A systematic literature review of forecasting and predictive models for cyanobacteria blooms in freshwater lakes. Water Research 182, 115959. https://doi.org/10.1016/j.watres.2020.115959

Singh, S., Kate, B.N., Banerjee, U.C., 2005. Bioactive compounds from cyanobacteria and microalgae: An overview. Critical Reviews in Biotechnology 25, 73–95. https://doi.org/10.1080/07388550500248498

Srinivasan, R., Sorial, G.A., 2011. Treatment of taste and odor causing compounds 2-methyl isoborneol and geosmin in drinking water: A critical review. Journal of Environmental Sciences 23, 1–13. https://doi.org/10.1016/s1001-0742(10)60367-1

Su, M., Yu, J., Zhang, J., Chen, H., An, W., Vogt, R.D., Andersen, T., Jia, D., Wang, J., Yang, M., 2015. MIB-producing cyanobacteria (Planktothrix sp.) in a drinking water reservoir: Distribution and odor producing potential. Water Research 68, 444–453. https://doi.org/10.1016/j.watres.2014.09.038

Su, M., Zhu, Y., Jia, Z., Liu, T., Yu, J., Burch, M., Yang, M., 2021. Identification of MIB producers and odor risk assessment using routine data: A case study of an estuary drinking water reservoir. Water Research 192, 116848. https://doi.org/10.1016/j.watres.2021.116848

Sun, D., Yu, J., Yang, M., An, W., Zhao, Y., Lu, N., Yuan, S., Zhang, D., 2014. Occurrence of odor problems in drinking water of major cities across China. Frontiers of Environmental Science & Engineering 8, 411–416. https://doi.org/10.1007/s11783-013-0577-1

Thornton, K.W., 1984. REGIONAL COMPARISONS OF LAKES AND RESERVOIRS: GEOLOGY, CLIMATOLOGY, AND MORPHOLOGY. Lake and Reservoir Management 1, 261–265. https://doi.org/10.1080/07438148409354521

Uwins, H.K., Teasdale, P., Stratton, H., 2007. A case study investigating the occurrence of geosmin and 2-methylisoborneol (MIB) in the surface waters of the hinze dam, gold coast, australia. Water Science and Technology 55, 231–238. https://doi.org/10.2166/wst.2007.184

Watson, S.B., 2003. Cyanobacterial and eukaryotic algal odour compounds: Signals or by-products? A review of their biological activity. Phycologia 42, 332–350. https://doi.org/10.2216/i0031-8884-42-4-332.1

Woolway, R.I., Kraemer, B.M., Lenters, J.D., Merchant, C.J., O’Reilly, C.M., Sharma, S., 2020. Global lake responses to climate change. Nature Reviews Earth &amp; Environment 1, 388–403. https://doi.org/10.1038/s43017-020-0067-5

Wu, T., Zhu, G., Wang, Z., Zhu, M., Xu, H., 2022. Seasonal dynamics of odor compounds concentration driven by phytoplankton succession in a subtropical drinking water reservoir, southeast China. Journal of Hazardous Materials 425, 128056. https://doi.org/10.1016/j.jhazmat.2021.128056

Xu, H., Paerl, H.W., Qin, B., Zhu, G., Gaoa, G., 2009. Nitrogen and phosphorus inputs control phytoplankton growth in eutrophic Lake Taihu, China. Limnology and Oceanography 55, 420–432. https://doi.org/10.4319/lo.2010.55.1.0420

Yang, M., Yu, J., Li, Z., Guo, Z., Burch, M., Lin, T., 2008. Taihu Lake not to blame for Wuxi’s Woes. Science 319, 158. https://doi.org/10.1126/science.319.5860.158a

Zamyadi, A., Henderson, R., Stuetz, R., Hofmann, R., Ho, L., Newcombe, G., 2015. Fate of geosmin and 2-methylisoborneol in full-scale water treatment plants. Water Research 83, 171–183. https://doi.org/10.1016/j.watres.2015.06.038

Zhang, Y., Whalen, J.K., Cai, C., Shan, K., Zhou, H., 2023. Harmful cyanobacteria-diatom/dinoflagellate blooms and their cyanotoxins in freshwaters: A nonnegligible chronic health and ecological hazard. Water Research 233, 119807. https://doi.org/10.1016/j.watres.2023.119807

Zohary, T., Gasith, A., 2014. The littoral zone, in: Lake Kinneret. Springer Netherlands, pp. 517–532. https://doi.org/10.1007/978-94-017-8944-8_29