# Light-dominated selection shaping filamentous cyanobacterial assemblages drives odor problem in a drinking water reservoir

Supplementary Material

Ming Su1,5, Yiping Zhu2, Tom Andersen3, Xianyun Wang4, Zhiyong Yu1,5, Jinping Lu1,5, Yichao Song2, Tengxin Cao1,5, Jianwei Yu1,5, Yu Zhang1,5, and Min Yang1,5,✉

2025-01-09

1 Key Laboratory of Drinking Water Science and Technology, Research Center for Eco-Environmental Sciences, Chinese Academy of Sciences
2 Shanghai Chengtou Raw Water Co. Ltd
3 Department of Biosciences, University of Oslo
4 National Engineering Research Center of China (South) for Urban Water
5 University of Chinese Academy of Sciences

✉ Correspondence: Min Yang <yangmin@rcees.ac.cn>

# Supplementary Material

All data analysis and illustration were performed using R 4.01. Data pretreatment and summary were performed using the dplyr2 and base packages in R, regression analysis including linear and generalized linear models were performed using the stats package1, generalized additive modelling was performed using the mgcv package3,4 quantile regression analysis was performed using the quantreg package5; contour figures were created by the graphics package1, other figures were prepared using the ggplot2 package6.

Fig. S1 Seasonal dynamics of MIB of Yangtze River water (inlet) from 2011 to 2015”

Fig. S2 Composition of filamentous cyanobacteria and Microcystis in QCS Reservoir

Fig. S3 Seasonal dynamics and long-term trends of Planktothrix, Pseudanabaena, Lyngbya, Phormidium and Microcystis in QCS Reservoir

Fig. S4 Seasonal dynamics and long-term trends of nutrients (total nitrogen, nitrate, ammonia and total phosphorus) in QCS Reservoir

Fig. S5 Correlation between factors and filamentous abundances

Year; Season; Min; Max; Median; IQR; Mean; SD; SE; CI. 2013; Spring; 0.037; 0.078; 0.055; 0.013; 0.055; 0.010; 0.002; 0.004. 2013; Summer; 0.033; 0.139; 0.058; 0.018; 0.063; 0.023; 0.003; 0.006. 2013; Autumn; 0.035; 0.127; 0.067; 0.027; 0.065; 0.021; 0.004; 0.007. 2013; Winter; 0.028; 0.201; 0.053; 0.089; 0.088; 0.064; 0.021; 0.050. 2014; Spring; 0.045; 0.107; 0.071; 0.019; 0.074; 0.016; 0.002; 0.005. 2014; Summer; 0.032; 0.093; 0.059; 0.016; 0.061; 0.013; 0.001; 0.003. 2014; Autumn; 0.024; 0.105; 0.044; 0.027; 0.051; 0.019; 0.003; 0.007. 2014; Winter; 0.034; 0.183; 0.067; 0.024; 0.078; 0.034; 0.007; 0.015. 2015; Spring; 0.034; 0.104; 0.069; 0.019; 0.068; 0.016; 0.003; 0.006. 2015; Summer; 0.033; 0.109; 0.058; 0.022; 0.062; 0.016; 0.002; 0.005. 2015; Autumn; 0.039; 0.077; 0.054; 0.008; 0.056; 0.011; 0.003; 0.007. 2015; Winter; 0.022; 0.081; 0.053; 0.028; 0.052; 0.022; 0.009; 0.023. Table S1 Descriptive statistics of ammonia nitrogen in QCS Reservoir (unit: mg L-1)

Year; Season; Min; Max; Median; IQR; Mean; SD; SE; CI. 2013; Spring; 1.213; 1.702; 1.415; 0.179; 1.431; 0.118; 0.024; 0.05. 2013; Summer; 0.759; 1.665; 1.110; 0.269; 1.137; 0.194; 0.025; 0.05. 2013; Autumn; 0.843; 1.71; 1.029; 0.445; 1.140; 0.281; 0.047; 0.095. 2013; Winter; 1.390; 1.828; 1.557; 0.167; 1.600; 0.136; 0.045; 0.105. 2014; Spring; 1.578; 2.162; 1.796; 0.121; 1.800; 0.130; 0.019; 0.038. 2014; Summer; 0.944; 1.737; 1.293; 0.176; 1.306; 0.163; 0.019; 0.038. 2014; Autumn; 1.159; 1.707; 1.309; 0.119; 1.352; 0.129; 0.023; 0.047. 2014; Winter; 1.561; 2.362; 2.008; 0.196; 2.008; 0.189; 0.039; 0.082. 2015; Spring; 1.157; 1.793; 1.485; 0.271; 1.474; 0.186; 0.033; 0.068. 2015; Summer; 0.895; 1.867; 1.411; 0.414; 1.333; 0.264; 0.037; 0.074. 2015; Autumn; 0.729; 1.365; 1.145; 0.234; 1.092; 0.189; 0.052; 0.114. 2015; Winter; 1.595; 1.774; 1.708; 0.131; 1.694; 0.078; 0.032; 0.082. Table S2 Descriptive statistics of nitrate nitrogen in QCS Reservoir (unit: mg L-1)

Year; Season; Min; Max; Median; IQR; Mean; SD; SE; CI. 2013; Spring; 1.008; 1.469; 1.326; 0.156; 1.313; 0.113; 0.023; 0.048. 2013; Summer; 0.683; 1.411; 1.126; 0.150; 1.094; 0.153; 0.020; 0.040. 2013; Autumn; 0.319; 1.592; 1.006; 0.583; 1.052; 0.351; 0.059; 0.119. 2013; Winter; 1.367; 1.735; 1.543; 0.161; 1.542; 0.132; 0.044; 0.101. 2014; Spring; 1.478; 2.265; 1.808; 0.217; 1.795; 0.164; 0.024; 0.048. 2014; Summer; 0.808; 1.705; 1.237; 0.192; 1.228; 0.195; 0.023; 0.045. 2014; Autumn; 0.959; 1.601; 1.236; 0.125; 1.250; 0.142; 0.025; 0.052. 2014; Winter; 1.361; 2.429; 2.073; 0.337; 1.974; 0.259; 0.054; 0.112. 2015; Spring; 1.234; 1.875; 1.419; 0.191; 1.450; 0.154; 0.028; 0.056. 2015; Summer; 0.718; 1.654; 1.317; 0.355; 1.292; 0.223; 0.031; 0.063. 2015; Autumn; 0.726; 1.348; 1.013; 0.135; 1.017; 0.156; 0.043; 0.094. 2015; Winter; 1.348; 1.75; 1.59; 0.279; 1.581; 0.177; 0.072; 0.186. Table S3 Descriptive statistics of total nitrogen (TN) in QCS Reservoir (unit: mg L-1)

Year; Season; Min; Max; Median; IQR; Mean; SD; SE; CI. 2013; Spring; 0.053; 0.095; 0.077; 0.016; 0.078; 0.012; 0.003; 0.005. 2013; Summer; 0.033; 0.25; 0.064; 0.023; 0.07; 0.036; 0.005; 0.009. 2013; Autumn; 0.046; 0.121; 0.070; 0.017; 0.072; 0.015; 0.003; 0.005. 2013; Winter; 0.055; 0.095; 0.060; 0.018; 0.067; 0.013; 0.004; 0.010. 2014; Spring; 0.050; 0.133; 0.089; 0.022; 0.089; 0.019; 0.003; 0.005. 2014; Summer; 0.047; 0.118; 0.081; 0.024; 0.079; 0.017; 0.002; 0.004. 2014; Autumn; 0.056; 0.126; 0.084; 0.011; 0.088; 0.013; 0.002; 0.005. 2014; Winter; 0.058; 0.133; 0.092; 0.037; 0.091; 0.021; 0.004; 0.009. 2015; Spring; 0.042; 0.115; 0.073; 0.023; 0.073; 0.019; 0.003; 0.007. 2015; Summer; 0.046; 0.222; 0.077; 0.022; 0.083; 0.027; 0.004; 0.007. 2015; Autumn; 0.065; 0.092; 0.072; 0.012; 0.075; 0.008; 0.002; 0.005. 2015; Winter; 0.069; 0.096; 0.075; 0.006; 0.078; 0.010; 0.004; 0.010. Table S4 Descriptive statistics of total phosphrus (TP) in QCS Reservoir (unit: mg L-1)

Year; Season; Min; Max; Median; IQR; Mean; SD; SE; CI. 2013; Spring; 11.208; 24.642; 19.468; 5.594; 19.475; 4.088; 0.834; 1.726. 2013; Summer; 24.563; 30.343; 28.701; 1.977; 28.348; 1.609; 0.208; 0.416. 2013; Autumn; 11.564; 27.028; 21.502; 7.032; 20.788; 5.019; 0.836; 1.698. 2013; Winter; 3.927; 9.514; 5.846; 4.501; 6.853; 2.403; 0.801; 1.847. 2014; Spring; 12.100; 25.821; 20.99; 6.767; 20.322; 4.091; 0.591; 1.188. 2014; Summer; 25.688; 28.549; 27.386; 0.832; 27.367; 0.691; 0.08; 0.159. 2014; Autumn; 12.874; 26.943; 21.812; 7.086; 21.447; 4.271; 0.767; 1.567. 2014; Winter; 7.19; 11.001; 8.403; 1.759; 8.637; 1.168; 0.244; 0.505. 2015; Spring; 11.17; 25.049; 22.13; 3.92; 20.856; 4.131; 0.742; 1.515. 2015; Summer; 24.69; 31.491; 28.798; 2.92; 28.475; 1.953; 0.273; 0.549. 2015; Autumn; 16.102; 28.22; 26.413; 5.503; 23.534; 4.587; 1.272; 2.772. 2015; Winter; 7.861; 8.981; 8.092; 0.554; 8.278; 0.447; 0.182; 0.469. Table S5 Descriptive statistics of water temperature in QCS Reservoir (unit: °C)

Year; Season; Min; Max; Median; IQR; Mean; SD; SE; CI. 2013; Spring; 0.815; 1.879; 1.273; 0.410; 1.322; 0.314; 0.064; 0.133. 2013; Summer; 0.901; 2.261; 1.638; 0.810; 1.621; 0.416; 0.054; 0.107. 2013; Autumn; 0.701; 1.468; 1.152; 0.409; 1.110; 0.247; 0.041; 0.084. 2013; Winter; 0.483; 1.311; 0.798; 0.139; 0.873; 0.298; 0.099; 0.229. 2014; Spring; 0.854; 1.977; 1.552; 0.395; 1.528; 0.284; 0.041; 0.083. 2014; Summer; 0.666; 1.840; 1.226; 0.453; 1.235; 0.316; 0.036; 0.073. 2014; Autumn; 0.643; 1.487; 0.957; 0.368; 0.999; 0.254; 0.046; 0.093. 2014; Winter; 0.461; 1.254; 0.938; 0.241; 0.857; 0.248; 0.052; 0.107. 2015; Spring; 0.870; 2.033; 1.462; 0.572; 1.418; 0.355; 0.064; 0.130. 2015; Summer; 0.734; 2.138; 1.235; 0.482; 1.293; 0.352; 0.049; 0.099. 2015; Autumn; 0.638; 1.460; 1.253; 0.185; 1.195; 0.329; 0.091; 0.199. 2015; Winter; 0.415; 0.994; 0.598; 0.297; 0.700; 0.239; 0.097; 0.251. Table S6 Descriptive statistics of pre-week PAR in QCS Reservoir (unit: mol m-2 d-1)

Term; EDF; Ref.DF; Statistic; P.Value. s(nweek); 5.293509; 8.000000; 21.395662; 0.000000. s(ndate); 1.757591; 1.757591; 3.755324; 0.091522. Table S7 Summary of time series analysis for Planktothrix; s(nweek) denotes the seasonal pattern, and s(ndate) denotes the long-term pattern

Fig. S6 Time series analysis of Planktothrix based on seasonal and long-term trend smooth functions

Seasonal and long-term trends of Planktothrix and Pseudanabaena were evaluated using time series analysis, as illustrated in Table S7, Fig. S6, Table S8 and Fig. S7.

Term; EDF; Ref.DF; Statistic; P.Value. s(nweek); 6.246786; 8.000000; 11.302972; 0.000000. s(ndate); 1.943873; 1.943873; 82.456653; 0.000000. Table S8 Summary of time series analysis for Pseudanabaena; s(nweek) denotes the seasonal pattern, and s(ndate) denotes the long-term pattern

Fig. S7 Time series analysis for Pseudanabaena based on seasonal and long-term trend smooth functions

We select water temperature (temp), pre-week photosynthetically active radiation (weekPAR), total nitrogen (TN), nitrate (NO3), total phosphate (TP), ammonia (NH4), wind speed (wind) and maximum daily air temperature (maxtemp) as the potential predictors for abundances of Planktothrix (Table S9) and Pseudanabaena (Table S10). Linear model (Y=∑βiXi+β0+ϵ, ϵ∈N0,1,i=1,2,...) between these predictors (Xi, i = 1, 2, …) and logarithm transformed abundances (Y=log101+N).

Term; Estimate; Std.Error; Statistic; P.Value. (Intercept); 0.055381; 0.204114; 0.271325; 0.786326. temp; 0.020686; 0.006190; 3.342053; 0.000936. weekPAR; -0.043151; 0.062931; -0.685693; 0.493431. TN; -0.125703; 0.134448; -0.934960; 0.350554. NO3; -0.110292; 0.143055; -0.770975; 0.441323. TP; 0.915797; 1.105912; 0.828092; 0.408270. NH4; -0.721135; 0.911296; -0.791330; 0.429371. wind; 0.085435; 0.042653; 2.003032; 0.046064. maxtemp; -0.003568; 0.005304; -0.672707; 0.501646. Table S9 Correlation analysis between Planktothrix and environmental factors (named as LM1)

Term; Estimate; Std.Error; Statistic; P.Value. (Intercept); -0.249268; 0.208706; -1.194353; 0.233274. temp; 0.041253; 0.006329; 6.518053; 2.97e-10. weekPAR; -0.119826; 0.064347; -1.862196; 0.063543. TN; 0.189667; 0.137473; 1.379667; 0.168707. NO3; -0.068600; 0.146274; -0.468984; 0.639418. TP; 0.202906; 1.130792; 0.179437; 0.857715. NH4; -0.737178; 0.931798; -0.791136; 0.429484. wind; -0.034087; 0.043612; -0.781579; 0.435072. maxtemp; -0.012824; 0.005423; -2.364817; 0.018669. Table S10 Correlation analysis between Pseudanabaena and environmental factors (named as LM2)

We performed Backward Stepwise Regressions to identify the significant variables responsible for the abundances of Planktothrix and Pseudanabaena. The regression started with a model that contains all variables, and then removing the least significant variables one by one, until a pre-specified stopping rule (here we use AIC rule) is reached.

Below is the Backward Stepwise Regressions of linear model for Planktothrix, the water temperature (temp), pre-week PAR (weekPAR), total nitrogen (TN) and wind speed (wind) were considered as effect predictors for Planktothrix.

Term; Estimate; Std.Error; Statistic; P.Value. (Intercept); 0.029206; 0.183201; 0.159421; 0.873442. temp; 0.018787; 0.003960; 4.744185; 3.21e-06. weekPAR; -0.066606; 0.057760; -1.153142; 0.249749. TN; -0.194775; 0.073604; -2.646272; 0.008558. wind; 0.075903; 0.040487; 1.874726; 0.061780. Table S11 Backward Stepwise Regression of linear models for Planktothrix

Below is the Backward Stepwise Regressions of linear model for Pseudanabaena, the water temperature (temp), pre-week PAR (weekPAR), total nitrogen (TN), ammonia (NH4) and maximum daily air temperature (maxtemp) were considered as effect predictors for Pseudanabaena.

Term; Estimate; Std.Error; Statistic; P.Value. (Intercept); -0.351; 0.162542; -2.159404; 0.031596. temp; 0.043007; 0.005947; 7.231744; 3.84e-12. weekPAR; -0.118911; 0.063542; -1.871372; 0.062248. TN; 0.142841; 0.078959; 1.809052; 0.071424. NH4; -0.888384; 0.900174; -0.986903; 0.324470. maxtemp; -0.013610; 0.005303; -2.566502; 0.010749. Table S12 Backward Stepwise Regression of linear models for Pseudanabaena

Backward Stepwise Regression approaches were performed for the two linear models to find out the optimum selections of predictors, as summarize in Table S11 (Planktothrix) and Table S12 (Pseudanabaena), respectively. Water temperature (temp), pre-week PAR (weekPAR), total nitrogen (TN), wind speed (wind) and maximum daily air temperature (maxtemp) were sorted out.

Parameter; VIF. temp; 2.025278. weekPAR; 1.449414. TN; 1.734340. TP; 1.148326. NH4; 1.179282. wind; 1.119961. Table S13 VIF analysis of selected predictors of Planktothrix and Pseudanabaena models

According to the Backward Stepwise Regressions, water temperature (temp), pre-week PAR (weekPAR), total nitrogen (TN), total phosphate (TP), ammonia (NH4) and wind speed (wind) were selected as the valid predictors and used for following general additive models (GAMs). Variance inflation factors (VIF) were calculated and validated the rationality (<5) for these predictors , as summarized in Table S13.

Fig. S8 Correlation between environmental factors

Correlation coefficients among these predictors were computed by Pearson Method, as illustrated in Fig. S8. The results were used for the optimization of non-parameter smooth functions (smoother) in following GAMs for Planktothrix and Pseudanabaena. Specifically, high correlation coefficient between water temperature (temp) and pre-week PAR (PAR) suggests they are interacting predictors; similarly, total nitrogen (TN) and ammonia (NH4) are interacting predictors.

Term; edf; ref.df; Statistic; p-value. t2(temp, weekPAR); 2.999974; 2.999974; 8.441657; 2.12287e-05. t2(TN, NH4); 2.999898; 2.999898; 2.800880; 0.04014. s(TP); 1.000008; 1.000008; 0.759490; 0.38418. s(wind); 1.000000; 1.000000; 3.944025; 0.04794. Table S14 Summary of GAM model with 6 predictors for Planktothrix (named as GAM1)

Iterations between water temperature and pre-week PAR (t2(temp, weekPAR)), and between total nitrogen and ammonia (t2(TN, NH4)) were both evaluated by the tensor products (f1x1⊗f2x2).

The results of “GAM1” suggest water temperature (temp), pre-week PAR (weekPAR), total nitrogen (TN) and ammonia (NH4) are most important predictors for Planktothrix abundance.

Term; edf; ref.df; Statistic; p-value. t2(temp, weekPAR); 5.733091; 5.733091; 9.604014; 0. t2(TN, NH4); 2.999999; 2.999999; 0.964346; 0.41001. s(TP); 2.236544; 2.236544; 2.650483; 0.1095. s(wind); 4.787511; 4.787511; 3.886710; 0.00265. Table S15 Summary of GAM model with 6 predictors for Pseudanabaena (named as GAM2)

The results of “GAM2” suggest water temperature (temp), pre-week PAR (weekPAR), total phosphrus (TP) and wind speed (wind) are most important predictors for Pseudanabaena abundance.

Term; edf; ref.df; Statistic; p-value. t2(temp, weekPAR); 2.999993; 2.999993; 8.439392; 1.912541e-05. t2(TN, NH4); 2.999870; 2.999870; 4.088651; 0.00689. Table S16 Summary of GAM model with 4 predictors for Planktothrix (named as GAM3)

According to “GAM1”, the predictors of Planktothrix abundances were further optimized to 4 factors, which are water temperature (temp), pre-week PAR (weekPAR), total nitrogen (TN) and ammonia (NH4). The summary of optimized Planktothrix GAM (GAM3) results is shown in Table S16.

Term; edf; ref.df; Statistic; p-value. t2(temp, weekPAR); 5.870002; 5.870002; 9.200651; 0. s(TP); 2.222784; 2.222784; 3.073369; 0.06404. s(wind); 4.665106; 4.665106; 3.782337; 0.00378. Table S17 Summary of GAM model with 4 predictors for Pseudanabaena (named as GAM4)

According to “GAM2”, the predictors of Pseudanabaena abundances were further optimized to 4 factors, which are water temperature (temp), pre-week PAR (weekPAR), total phosphorus (TP) and wind speed (wind). The summary of optimized Pseudanabaena GAM (GAM4) results is shown in Table S17.

## R Language demonstration code

# demo data frame
require(lubridate)
require(mgcv)
modeldf <- data.frame(date = ymd("2020-01-01") + 0:365,
                      y = rnorm(366))
# seasonal pattern: week number
modeldf$x1 <- week(modeldf$date)
# long-term pattern: decimal number
modeldf$x2 <- year(modeldf$date) + yday(modeldf$date) / 366
# demo gam model
m <- gamm(y ~ s(x1, bs = "cc") + s(x2), data = modeldf)

1. 	R Core Team. R: A language and environment for statistical computing. (R Foundation for Statistical Computing, 2020).

2. 	Wickham, H., François, R., Henry, L. & Müller, K. dplyr: A grammar of data manipulation. (2018).

3. 	Wood, S. N. Stable and efficient multiple smoothing parameter estimation for generalized additive models. Journal of the American Statistical Association 99, 673–686 (2004).

4. 	Wood, S. N. Fast stable restricted maximum likelihood and marginal likelihood estimation of semiparametric generalized linear models. Journal of the Royal Statistical Society (b) 73, 3–36 (2011).

5. 	Koenker, R. Quantreg: Quantile regression. (2017).

6. 	Wickham, H. ggplot2: Elegant graphics for data analysis. (Springer-verlag New York, 2016).